INTERNATIONAL ELECTROTECHNICAL COMMISSION

TECHNICAL COMMITTEE No. 65:
INDUSTRIAL-PROCESS MEASUREMENT AND CONTROL
SUB-COMMITTEE 65B: DEVICES

WORKING GROUP 7: PROGRAMMABLE CONTROLLERS

FINAL DRAFT - IEC 61131-3, 2nd Ed.
PROGRAMMABLE CONTROLLERS - PROGRAMMING LANGUAGES

FOREWORD

This document is the result of the review by IEC TC65/WG6, at its meeting in Melbourne, Florida USA on 15-16 January 2001, of known National Committee comments received on the document IEC 65B/373/CDV, Voting Draft - IEC 61131-3, 2nd Edition. Additional comments received from the German Advisory Group to IEC SC65B were also considered.

Annexes A, B, C, D, and E of this document are normative.

TABLE OF CONTENTS

1.  General	9

1.1  Scope	9

1.2  Normative references	9

1.3  Definitions	9

1.4  Overview and general requirements	14

1.4.1  Software model	14

1.4.2  Communication model	16

1.4.3  Programming model	18

1.5  Compliance	21

1.5.1  System compliance	21

1.5.2  Program compliance	23

2.  Common elements	24

2.1  Use of printed characters	24

2.1.1  Character set	24

2.1.2  Identifiers	24

2.1.3  Keywords	25

2.1.4 Use of white space	25

2.1.5  Comments	25

2.1.6  Pragmas	26

2.2  External representation of data	26

2.2.1  Numeric literals	26

2.2.2  Character string literals	27

2.2.3  Time literals	29

2.2.3.1  Duration	30

2.2.3.2  Time of day and date	30

2.3  Data types	31

2.3.1  Elementary data types	31

2.3.2  Generic data types	33

2.3.3  Derived data types	34

2.3.3.1  Declaration	34

2.3.3.2  Initialization	35

2.3.3.3 Usage	37

2.4  Variables	38

2.4.1  Representation	38

2.4.1.1  Single-element variables	38

2.4.1.2  Multi-element variables	40

2.4.2  Initialization	41

2.4.3  Declaration	41

2.4.3.1  Type assignment	43

2.4.3.2  Initial value assignment	45

2.5  Program organization units	47

2.5.1  Functions	48

2.5.1.1  Representation	49

2.5.1.2  Execution control	53

2.5.1.3  Declaration	54

2.5.1.4  Typing, overloading, and type conversion	57

2.5.1.5  Standard functions	59

2.5.1.5.1  Type conversion functions	60

2.5.1.5.2  Numerical functions	61

2.5.1.5.3  Bit string functions	64

2.5.1.5.4  Selection and comparison functions	64

2.5.1.5.5  Character string functions	67

2.5.1.5.6  Functions of time data types	69

2.5.1.5.7  Functions of enumerated data types	71

2.5.2  Function blocks	72

2.5.2.1  Representation	72

2.5.2.1a  Use of EN and ENO in function blocks	74

2.5.2.2  Declaration	75

2.5.2.3  Standard function blocks	82

2.5.2.3.1  Bistable elements	82

2.5.2.3.2  Edge detection	83

2.5.2.3.3  Counters	84

2.5.2.3.4  Timers	87

2.5.2.3.5  Communication function blocks	88

2.5.3  Programs	89

2.6  Sequential Function Chart (SFC) elements	90

2.6.1  General	90

2.6.2  Steps	90

2.6.3  Transitions	93

2.6.4  Actions	96

2.6.4.1  Declaration	96

2.6.4.2  Association with steps	98

2.6.4.3  Action blocks	99

2.6.4.4  Action qualifiers	100

2.6.4.5  Action control	101

2.6.5  Rules of evolution	106

2.6.6  Compatibility of SFC elements	117

2.6.7  SFC Compliance requirements	118

2.7  Configuration elements	118

2.7.1  Configurations, resources, and access paths	120

2.7.2  Tasks	124

3.  Textual languages	134

3.1  Common elements	134

3.2  Instruction list (IL)	134

3.2.1  Instructions	134

3.2.2  Operators, modifiers and operands	135

3.2.3  Functions and Function Blocks	137

3.3  Structured Text (ST)	141

3.3.1  Expressions	141

3.3.2  Statements	143

3.3.2.1  Assignment statements	144

3.3.2.2  Function and function block control statements	144

3.3.2.3  Selection statements	145

3.3.2.4  Iteration statements	145

4.  Graphic languages	147

4.1  Common elements	147

4.1.1 Representation of lines and blocks	147

4.1.2  Direction of flow in networks	147

4.1.3  Evaluation of networks	149

4.1.4  Execution control elements	150

4.2  Ladder Diagram (LD)	152

4.2.1  Power rails	152

4.2.2  Link elements and states	152

4.2.3  Contacts	153

4.2.4  Coils	153

4.2.5  Functions and function blocks	153

4.2.6  Order of network evaluation	154

4.3  Function Block Diagram (FBD)	156

4.3.1  General	156

4.3.2  Combination of elements	156

4.3.3  Order of network evaluation	156

ANNEX A - Specification method for textual languages (normative)	157

A.1  Syntax	157

A.1.1    Terminal symbols	157

A.1.2    Non-terminal symbols	157

A.1.3   Production rules	158

A.2  Semantics	158

ANNEX B - Formal specifications of language elements (normative)	159

B.0  Programming model	159

B.1  Common elements	159

B.1.1 Letters, digits and identifiers	159

B.1.2  Constants	160

B.1.2.1   Numeric literals	160

B.1.2.2  Character strings	160

B.1.2.3 Time literals	161

B.1.2.3.1  Duration	161

B.1.2.3.2  Time of day and date	161

B.1.3   Data types	162

B.1.3.1  Elementary data types	162

B.1.3.2  Generic data types	162

B.1.3.3  Derived data types	163

B.1.4  Variables	164

B.1.4.1  Directly represented variables	164

B.1.4.2  Multi-element variables	165

B.1.4.3  Declaration and initialization	165

B.1.5  Program organization units	167

B.1.5.1  Functions	167

B.1.5.2  Function blocks	168

B.1.5.3  Programs	169

B.1.6  Sequential function chart elements	169

B.1.7  Configuration elements	170

B.2  Language IL (Instruction List)	172

B.2.1  Instructions and operands	172

B.2.2  Operators	173

B.3  Language ST (Structured Text)	173

B.3.1  Expressions	173

B.3.2  Statements	174

B.3.2.1  Assignment statements	174

B.3.2.2  Subprogram control statements	174

B.3.2.3  Selection statements	174

B.3.2.4  Iteration statements	175

ANNEX C - Delimiters and Keywords (normative)	176

ANNEX D - Implementation-dependent parameters (normative)	180

ANNEX E - Error Conditions (normative)	182

ANNEX F -  Examples (informative)	184

F.1  Function WEIGH	184

F.2  Function block CMD_MONITOR	185

F.3  Function block FWD_REV_MON	188

F.4  Function block STACK_INT	194

F.5  Function block MIX_2_BRIX	199

F.6  Analog signal processing	203

F.6.1  Function block LAG1	204

F.6.2  Function block DELAY	205

F.6.3  Function block AVERAGE	206

F.6.4  Function block INTEGRAL	207

F.6.5  Function block DERIVATIVE	208

F.6.6  Function block HYSTERESIS	208

F.6.7  Function block LIMITS_ALARM	209

F.6.8  Structure ANALOG_LIMITS	209

F.6.9  Function block ANALOG_MONITOR	210

F.6.10 Function block PID	211

F.6.11 Function block DIFFEQ	212

F.6.12  Function block RAMP	213

F.6.13  Function block TRANSFER	214

F.7  Program GRAVEL	214

F.8  Program AGV	223

F.9  Use of enumerated data types	227

F.10  Function block RTC (Real Time Clock)	227

F.11  Function block ALRM_INT	228

ANNEX G - Index (informative)	229

ANNEX H - Reference character set (informative)	240

LIST OF TABLES

Table 1 - Character set features	24

Table 2 - Identifier features	25

Table 3 - Comment feature	26

Table 3a - Pragma feature	26

Table 4 - Numeric literals	27

Table 5 - Character string literal features	28

Table 6 - Two-character combinations in character strings	29

Table 7 - Duration literal features	30

Table 8 - Date and time of day literals	30

Table 9 - Examples of date and time of day literals	31

Table 10 - Elementary data types	32

Table 11 - Hierarchy of generic data types	34

Table 12 - Data type declaration features	35

Table 13 - Default initial values of elementary data types	36

Table 14 - Data type initial value declaration features	37

Table 15 - Location and size prefix features for directly represented variables	40

Table 16a - Variable declaration keywords	42

Table 16b - Usages of VAR_GLOBAL, VAR_EXTERNAL and CONSTANT declarations	43

Table 17 - Variable type assignment features	44

Table 18 - Variable initial value assignment features	46

Table 19 - Graphical negation of Boolean signals	51

Table 19a - Textual invocation of functions for formal and non-formal argument list	53

Table 20 - Use of EN input and ENO output	54

Table 20a - Function  features	55

Table 21 - Typed and overloaded functions	57

Table 22 - Type conversion function features	60

Table 23 - Standard functions of one numeric variable	62

Table 24 - Standard arithmetic functions	63

Table 25 - Standard bit shift functions	64

Table 26 - Standard bitwise Boolean functions	65

Table 27 - Standard selection functionsd	66

Table 28 - Standard comparison functions	67

Table 29 - Standard character string functions	68

Table 30 - Functions of time data types	69

Table 31 - Functions of enumerated data types	71

Table 32 - Examples of function block I/O variable usage	74

Table 33 - Function block declaration and usage features	77

Table 34 - Standard bistable function blocks a	83

Table 35 - Standard edge detection function blocks	84

Table 36 - Standard counter function blocks	84

Table 37 - Standard timer function blocks	87

Table 38 - Standard timer function blocks - timing diagrams	87

Table 39 - Program declaration features	90

Table 40 - Step features	92

Table 41 - Transitions and transition conditions	94

Table 42 - Declaration of actions a,b	97

Table 43 - Step/action association	99

Table 44 - Action block features	100

Table 45 - Action qualifiers	101

Table 45a - Action control features	105

Table 46 - Sequence evolution	108

Table 47 - Compatible SFC features	118

Table 48 - SFC minimal compliance requirements	118

Table 49 - Configuration and resource declaration features	122

Table 50 - Task features	126

Table 51a - Examples of instruction fields	135

Table 51b - Parenthesized expression features for IL language	136

Table 52 - Instruction List operators	136

Table 53 - Function Block invocation and Function invocation features for IL language	138

Table 54 - Standard Function Block input operators for IL language	140

Table 55 - Operators of the ST language	142

Table 56 - ST language statements	143

Table 57 - Representation of lines and blocks	148

Table 58 - Graphic execution control elements	151

Table 59 - Power rails	152

Table 60 - Link elements	153

Table 61 - Contacts a	154

Table 62 - Coils	155

Table C.1 - Delimiters	176

Table C.2 - Keywords	177

Table D.1 - Implementation-dependent parameters	180

Table E.1 - Error conditions	182

Table H.1 - Character representations	240

Table H.2 - Character encodings	241

LIST OF FIGURES

Figure 1 - Software model	15

Figure 2a - Data flow connection within a program	16

Figure 2b - Communication via GLOBAL variables	17

Figure 2c - Communication function blocks	17

Figure 2d - Communication via access paths	18

Figure 3 - Combination of programmable controller language elements	20

Figure 4 - Examples of function usage	48

Figure 5 - Use of formal argument names	52

Figure 6 - Examples of function declarations and usage	56

Figure 7 - Examples of explicit type conversion with overloaded functions	58

Figure 8 - Examples of explicit type conversion with typed functions	59

Figure 9 - Function block instantiation examples	73

Figure 10 - Examples of function block declarations	77

Figure 11a - Graphical use of a function block name as an input variable	79

Figure 11b - Graphical use of a function block name as an in-out variable	80

Figure 11c - Graphical use of a function block name as an external variable	81

Figure 12 - Declaration and usage of in-out variables in function blocks	82

Figure 14 - ACTION_CONTROL function block - External interface(Not visible to the user)	102

Figure 15a - ACTION_CONTROL function block body with "final scan" logic	103

Figure 15b - ACTION_CONTROL function block body without "final scan" logic	104

Figure 16a - Action control example - SFC representation	105

Figure 16b - Action control example - functional equivalent	106

Figure 17 - Examples of SFC evolution rules	114

Figure 18a - Examples of SFC errors: an "unsafe" SFC	116

Figure 18b - Examples of SFC errors: an "unreachable" SFC	117

Figure 19a - Graphical example of a configuration	119

Figure 19b - Skeleton function block and program declarations for configuration example	120

Figure 20 - Examples of CONFIGURATION and RESOURCE declaration features	123

Figure 21a - Synchronization of function blocks with explicit task associations	131

Figure 21b - Synchronization of function blocks with implicit task associations	132

Figure 21c - Explicit task associations equivalent to figure 21b	133

Figure 22 - EXIT statement example	146

Figure 23 - Feedback path example	150

Figure 24 - Boolean OR Examples	156

# 1.  General

## 1.1  Scope

This Part specifies syntax and semantics of programming languages for programmable controllers as defined in Part 1 of this Standard.

The functions of program entry, testing, monitoring, operating system, etc., are specified in Part 1 of this Standard.

## 1.2  Normative references

The following normative documents contain provisions which, through reference in this text, constitute provisions of this part of IEC 61131. At the time of publication, the editions indicated were valid. All normative documents are subject to revision, and parties to agreements based on this part of IEC 61131 are encouraged to investigate the possibility of applying the most recent editions of the normative documents indicated below. Members of IEC and ISO maintain registers of currently valid International Standards.

IEC 50: International Electrotechnical Vocabulary (IEV)

IEC 559: 1989, Binary floating-point arithmetic for microprocessors systems

IEC 617-12: 1991, Graphical symbols for diagrams, Part 12: Binary logic elements

IEC 617-13: 1978, Graphical symbols for diagrams, Part 13: Analogue elements

IEC 848: 1988, Preparation of function charts for control systems

ISO/AFNOR: 1989, Dictionary of computer science, ISBN 2-12-4869111-6

ISO 7185: 1990, Information technology - Programming languages - Pascal

ISO 7498: 1984, Information processing systems - Open systems interconnection - Basic reference model

ISO/IEC 10646-1:1993, Information technology - Universal multiple-octet coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane

## 1.3  Definitions

For the purposes of this part of IEC 61131, the following definitions apply. Definitions applying to all parts of IEC 61131 are given in part 1.

NOTE 1	Terms defined in this subclause are italicized where they appear in the bodies of definitions.

NOTE 2	The notation "(ISO)" following a definition indicates that the definition is taken from the ISO/AFNOR Dictionary of computer science.

NOTE 3	The ISO/AFNOR Dictionary of computer science and the International Electrotechnical Vocabulary should be consulted for terms not defined in this standard.

1.3.1. absolute time: The combination of time of day and date information.

1.3.2. access path: The association of a symbolic name with a variable for the purpose of open communication.

1.3.3. action: A Boolean variable, or a collection of operations to be performed, together with an associated control structure, as specified in 2.6.4.

1.3.4. action block: A graphical language element which utilizes a Boolean input variable to determine the value of a Boolean output variable or the enabling condition for an action, according to a predetermined control structure as defined in 2.6.4.5.

1.3.5. aggregate: A structured collection of data objects forming a data type. (ISO)

1.3.6. argument: Synonymous with input variable, output variable or in-out variable.

1.3.7. array: An aggregate that consists of data objects, with identical attributes, each of which may be uniquely referenced by subscripting. (ISO)

1.3.8. assignment: A mechanism to give a value to a variable or to an aggregate. (ISO)

1.3.9. based number: A number represented in a specified base other than ten.

1.3.10. bistable function block: A function block with two stable states controlled by one or more inputs.

1.3.11. bit string: A data element consisting of one or more bits.

1.3.12. body: That portion of a program organization unit which specifies the operations to be performed on the declared operands of the program organization unit when its execution is invoked.

1.3.13. call: A language construct for invoking the execution of a function or function block.

1.3.14. character string: An aggregate that consists of an ordered sequence of characters.

1.3.15. comment: A language construct for the inclusion of text in a program and having no impact on the execution of the program. (ISO)

1.3.16. compile: To translate a program organization unit or data type specification into its machine language equivalent or an intermediate form.

1.3.17. configuration: A language element corresponding to a programmable controller system as defined in IEC 61131-1.

1.3.18. counter function block: A function block which accumulates a value for the number of changes sensed at one or more specified inputs.

1.3.19. data type: A set of values together with a set of permitted operations. (ISO)

1.3.20. date and time: The date within the year and the time of day represented as a single language element.

1.3.21. declaration: The mechanism for establishing the definition of a language element. A declaration normally involves attaching an identifier to the language element, and allocating attributes such as data types and algorithms to it.

1.3.22. delimiter: A character or combination of characters used to separate program language elements.

1.3.23. direct representation: A means of representing a variable in a programmable controller program from which a manufacturer-specified correspondence to a physical or logical location may be determined directly.

1.3.24. double word: A data element containing 32 bits.

1.3.25. evaluation: The process of establishing a value for an expression or a function, or for the outputs of a network or function block, during program execution.

1.3.26. execution control element: A language element which controls the flow of program execution.

1.3.27. falling edge: The change from 1 to 0 of a Boolean variable.

1.3.28. function (procedure): A program organization unit which, when executed, yields exactly one data element and possibly additional output variables (which may be multi-valued, e.g., an array or structure), and whose invocation can be used in textual languages as an operand in an expression.

1.3.29. function block instance(function block):  An instance of a function block type.

1.3.30. function block type: A programmable controller programming language element consisting of: (i) the definition of a data structure partitioned into input, output, and internal variables; and (ii) a set of operations to be performed upon the elements of the data structure when an instance of the function block type is invoked.

1.3.31. function block diagram: A network in which the nodes are function block instances, graphically represented functions (procedures), variables, literals, and labels.

1.3.32. generic data type: A data type which represents more than one type of data, as specified in 2.3.2.

1.3.33. global scope: Scope of a declaration applying to all program organization units within a resource or configuration.

1.3.34. global variable: A variable whose scope is global.

1.3.35. hierarchical addressing: The direct representation of a data element as a member of a physical or logical hierarchy, e.g., a point within a module which is contained in a rack, which in turn is contained in a cubicle, etc.

1.3.36. identifier: A combination of letters, numbers, and underline characters, as specified in 2.1.2, which begins with a letter or underline and which names a language element.

1.3.37. in-out variable: A variable that is declared in a VAR_IN_OUT...END_VAR block.

1.3.38. initial value: The value assigned to a variable at system start-up.

1.3.39. input variable (input): A variable which is used to supply an argument to a program organization unit.

1.3.40. instance: An individual, named copy of the data structure associated with a function block type or program type, which persists from one invocation of the associated operations to the next.

1.3.41. instance name: An identifier associated with a specific instance.

1.3.42. instantiation: The creation of an instance.

1.3.43. 	integer literal: A literal which directly represents a value of type SINT, INT, DINT, LINT, BOOL, BYTE, WORD, DWORD, or LWORD, as defined in 2.3.1.

1.3.44. invocation: The process of initiating the execution of the operations specified in a program organization unit.

1.3.45. keyword: A lexical unit that characterizes a language element, e.g., "IF".

1.3.46. label: A language construction naming an instruction, network, or group of networks, and including an identifier.

1.3.47. language element: Any item identified by a symbol on the left-hand side of a production rule in the formal specification given in annex B of this part of IEC 61131.

1.3.48. literal: A lexical unit that directly represents a value. (ISO)

1.3.49. local scope: The scope of a declaration or label applying only to the program organization unit in which the declaration or label appears.

1.3.50. logical location: The location of a hierarchically addressed variable in a schema which may or may not bear any relation to the physical structure of the programmable controller's inputs, outputs, and memory.

1.3.51. long real: A real number represented in a long word.

1.3.52. long word: A 64-bit data element.

1.3.53. memory (user data storage): A functional unit to which the user program can store data and from which it can retrieve the stored data.

1.3.54. named element: An element of a structure which is named by its associated identifier.

1.3.55. network: An arrangement of nodes and interconnecting branches.

1.3.56. off-delay (on-delay) timer function block: A function block which delays the falling (rising) edge of a Boolean input by a specified duration.

1.3.57. operand: A language element on which an operation is performed.

1.3.58. operator: A symbol that represents the action to be performed in an operation.

1.3.59. output variable (output): A variable which is used to return the result(s) of the evaluation of a program organization unit.

1.3.60. overloaded: With respect to an operation or function, capable of operating on data of different types, as specified in 2.5.1.4.

1.3.61. power flow: The symbolic flow of electrical power in a ladder diagram, used to denote the progression of a logic solving algorithm.

1.3.62. pragma: A language construct for the inclusion of text in a program organization unit which may affect the preparation of the program for execution.

1.3.63. program (verb): To design, write, and test user programs.

1.3.64. program organization unit: A function, function block, or program.

NOTE	This term may refer to either a type or an instance.

1.3.65. real literal: A literal representing data of type REAL or LREAL.

1.3.66. resource: A language element corresponding to a "signal processing function" and its "man-machine interface" and "sensor and actuator interface functions", if any, as defined in IEC 61131-1.

1.3.67. retentive data: Data stored in such a way that its value remains unchanged after a power down / power up sequence.

1.3.68. return: A language construction within a program organization unit designating an end to the execution sequences in the unit.

1.3.69. rising edge: The change from 0 to 1 of a Boolean variable.

1.3.70. scope: That portion of a language element within which a declaration or label applies.

1.3.71. semantics: The relationships between the symbolic elements of a programming language and their meanings, interpretation and use.

1.3.72. semigraphic representation: Representation of graphic information by the use of a limited set of characters.

1.3.73. single data element: A data element consisting of a single value.

1.3.74. single-element variable: A variable which represents a single data element.

1.3.75. step: A situation in which the behavior of a program organization unit with respect to its inputs and outputs follows a set of rules defined by the associated actions of the step.

1.3.76. structured data type: An aggregate data type which has been declared using a STRUCT or FUNCTION_BLOCK declaration.

1.3.77. subscripting: A mechanism for referencing an array element by means of an array reference and one or more expressions that, when evaluated, denote the position of the element.

1.3.78. symbolic representation: The use of identifiers to name variables.

1.3.79. task: An execution control element providing for periodic or triggered execution of a group of associated program organization units.

1.3.80. time literal: A literal representing data of type TIME, DATE, TIME_OF_DAY, or DATE_AND_TIME.

1.3.81. transition: The condition whereby control passes from one or more predecessor steps to one or more successor steps along a directed link.

1.3.82. unsigned integer: An integer literal not containing a leading plus (+) or minus (-) sign.

1.3.83. wired OR: A construction for achieving the Boolean OR function in the LD language by connecting together the right ends of horizontal connectives with vertical connectives

## 1.4  Overview and general requirements

This part of IEC 61131 specifies the syntax and semantics of a unified suite of programming languages for programmable controllers (PCs). These consist of two textual languages, IL (Instruction List) and ST (Structured Text), and two graphical languages, LD (Ladder Diagram) and FBD (Function Block Diagram).

Sequential Function Chart (SFC) elements are defined for structuring the internal organization of programmable controller programs and function blocks. Also, configuration elements are defined which support the installation of programmable controller programs into programmable controller systems.

In addition, features are defined which facilitate communication among programmable controllers and other components of automated systems.

The programming language elements defined in this part may be used in an interactive programming environment. The specification of such environments is beyond the scope of this part; however, such an environment shall be capable of producing textual or graphic program documentation in the formats specified in this part.

The material in this part is arranged in "bottom-up" fashion, that is, simpler language elements are presented first, in order to minimize forward references in the text. The remainder of this subclause provides an overview of the material presented in this part and incorporates some general requirements.

### 1.4.1  Software model

The basic high-level language elements and their interrelationships are illustrated in figure 1. These consist of elements which are programmed using the languages defined in this Part, that is, programs and function blocks; and configuration elements, namely, configurations, resources, tasks, global variables, access paths, and instance-specific initializations, which support the installation of programmable controller programs into programmable controller systems.

NOTE 1	This figure is illustrative only. The graphical representation is not normative.

NOTE 2	In a configuration with a single resource, the resource need not be explicitly represented.

Figure 1 - Software model

A configuration is the language element which corresponds to a programmable controller system as defined in IEC 61131-1. A resource corresponds to a "signal processing function" and its "man-machine interface" and "sensor and actuator interface" functions (if any) as defined in IEC 61131-1. A configuration contains one or more resources, each of which contains one or more programs executed under the control of zero or more tasks. A program may contain zero or more function blocks or other language elements as defined in this part.

Configurations and resources can be started and stopped via the "operator interface", "programming, testing, and monitoring", or "operating system" functions defined in IEC 61131-1. The starting of a configuration shall cause the initialization of its global variables according to the rules given in 2.4.2, followed by the starting of all the resources in the configuration. The starting of a resource shall cause the initialization of all the variables in the resource, followed by the enabling of all the tasks in the resource. The stopping of a resource shall cause the disabling of all its tasks, while the stopping of a configuration shall cause the stopping of all its resources. Mechanisms for the control of tasks are defined in 2.7.2, while mechanisms for the starting and stopping of configurations and resources via communication functions are defined in IEC 61131-5.

Programs, resources, global variables, access paths (and their corresponding access privileges), and configurations can be loaded or deleted by the "communication function" defined in IEC 61131-1. The loading or deletion of a configuration or resource shall be equivalent to the loading or deletion of all the elements it contains.

Access paths and their corresponding access privileges are defined in 2.7.1.

The mapping of the language elements defined in this subclause on to communication objects is defined in IEC 61131-5.

### 1.4.2  Communication model

Figure 2 illustrates the ways that values of variables can be communicated among software elements.

As shown in figure 2a, variable values within a program can be communicated directly by connection of the output of one program element to the input of another. This connection is shown explicitly in graphical languages and implicitly in textual languages.

Variable values can be communicated between programs in the same configuration via global variables such as the variable x illustrated in figure 2b. These variables shall be declared as GLOBAL in the configuration, and as EXTERNAL in the programs, as specified in 2.4.3.

As illustrated in figure 2c, the values of variables can be communicated between different parts of a program, between programs in the same or different configurations, or between a programmable controller program and a non-programmable controller system, using the communication function blocks defined in IEC 61131-5 and described in 2.5.2.3.5. In addition, programmable controllers or non-programmable controller systems can transfer data which is made available by access paths, as illustrated in figure 2d, using the mechanisms defined in IEC 61131-5.

Figure 2a - Data flow connection within a program

Figure 2b - Communication via GLOBAL variables

Figure 2c - Communication function blocks

Figure 2d - Communication via access paths

NOTE 1	This figure is illustrative only. The graphical representation is not normative.

NOTE 2	In these examples, configurations C and D are each considered to have a single resource.

NOTE 3	The details of the communication function blocks are not shown in this figure. See 2.5.2.3.5 and IEC 61131-5.

NOTE 4	As specified in 2.7, access paths can be declared on directly represented variables, global variables, or input, output, or internal variables of programs or function block instances.

NOTE 5	IEC 61131-5 specifies the means by which both PC and non-PC systems can use access paths for reading and writing of variables.

### 1.4.3  Programming model

The elements of programmable controller programming languages, and the subclauses in which they appear in this part, are classified as follows:

Data types (2.3)

Variables (2.4)

Program organization units (2.5)

Functions (2.5.1)

Function blocks (2.5.2)

Programs (2.5.3)

Sequential Function Chart (SFC) elements (2.6)

Configuration elements (2.7)

Global variables (2.7.1)

Resources (2.7.1)

Access paths (2.7.1)

Tasks (2.7.2)

As shown in figure 3, the combination of these elements shall obey the following rules:

1)	Derived data types shall be declared as specified in 2.3.3, using the standard data types specified in 2.3.1 and 2.3.2 and any previously derived data types.

2)	Derived functions can be declared as specified in 2.5.1.3, using standard or derived data types, the standard functions defined in 2.5.1.5, and any previously derived functions. This declaration shall use the mechanisms defined for the IL, ST, LD or FBD language.

3)	Derived function blocks can be declared as specified in 2.5.2.2, using standard or derived data types and functions, the standard function blocks defined in 2.5.2.3, and any previously derived function blocks. This declaration shall use the mechanisms defined for the IL, ST, LD, or FBD language, and can include Sequential Function Chart (SFC) elements as defined in 2.6.

4)	A program shall be declared as specified in 2.5.3, using standard or derived data types, functions, and function blocks. This declaration shall use the mechanisms defined for the IL, ST, LD, or FBD language, and can include Sequential Function Chart (SFC) elements as defined in 2.6.

5)	Programs can be combined into configurations using the elements defined in 2.7, that is, global variables, resources, tasks, and access paths.

Reference to "previously derived" data types, functions, and function blocks in the above rules is intended to imply that once such a derived element has been declared, its definition is available, e.g., in a "library" of derived elements, for use in further derivations. Therefore, the declaration of a derived element type shall not be contained within the declaration of another derived element type.

A programming language other than one of those defined in this standard may be used in the declaration of a function or function block. The means by which a user program written in one of the languages defined in this standard invokes the execution of, and accesses the data associated with, such a derived function or function block shall be as defined in this standard.

NOTE 1	The parenthesized numbers (1) to (5) refer to corresponding paragraphs in subclause 1.4.3.

NOTE 2	Data types are used in all productions. For clarity, the corresponding linkages are omitted in this figure.

Figure 3 - Combination of programmable controller language elements

LD - Ladder Diagram (4.2)

FBD - Function Block Diagram (4.3)

IL - Instruction List (3.2)

ST - Structured Text (3.3)

OTHERS - Other programming languages (1.4.3)

## 1.5  Compliance

This subclause defines the requirements which shall be met by programmable controller systems and programs which claim compliance with this part of IEC 61131.

### 1.5.1  System compliance

A programmable controller system, as defined in IEC 61131-1, which claims to comply, wholly or partially, with the requirements of this part of IEC 61131 shall do so only as described below.

A compliance statement shall be included in the documentation accompanying the system, or shall be produced by the system itself. The form of the compliance statement shall be:

"This system complies with the requirements of IEC 61131-3, for the following language features:",

followed by a set of compliance tables in the following format:

Table title

| Table No. | Feature No. | Features description |
| --- | --- | --- |
| ... | ... | ... |

Table and feature numbers and descriptions are to be taken from the tables given in the relevant subclauses of this part of IEC 61131. Table titles are to be taken from the following table.

| Table title | For features in: |
| --- | --- |
| Common elements | Clause 2 |
| Common textual elements | Subclause 3.1 |
| IL language elements | Subclauses 3.2.1 to 3.2.3 |
| ST language elements | Subclauses 3.3.1 to 3.3.2.4 |
| Common graphical elements | Subclauses 4.1 to 4.1.4 |
| LD language elements | Subclauses 4.2 to 4.2.6 |
| FBD language elements | Subclauses 4.3 to 4.3.3 |

For the purposes of determining compliance, tables 9, 11, 13, 16a, 16b, 32, 38, 47, 48 and 51 shall not be considered tables of features.

A programmable controller system complying with the requirements of this part with respect to a language defined in this part:

a)	shall not require the inclusion of substitute or additional language elements in order to accomplish any of the features specified in this part,unless such elements are identified and treated as noted in rules (e) and (f) below;

b)	shall be accompanied by a document that specifies the values of all implementation-dependent parameters as listed in annex D;

c)	shall be able to determine whether or not a user's language element violates any requirement of this part, where such a violation is not designated an error in annex E, and report the result of this determination to the user. In the case where the system does not examine the whole program organization unit, the user shall be notified that the determination is incomplete whenever no violations have been detected in the portion of the program organization unit examined;

d)	shall treat each user violation that is designated an error in annex E in at least one of the following ways:

1)	there shall be a statement in an accompanying document that the error is not reported;

2)	the system shall report during preparation of the program for execution that an occurrence of that error is possible;

3)	the system shall report the error during preparation of the program for execution;

4)	the system shall report the error during execution of the program and initiate appropriate system- or user-defined error handling procedures;

and if any violations that are designated as errors are treated in the manner described in d)1) above, then a note referencing each such treatment shall appear in a separate section of the accompanying document;

e)	shall be accompanied by a document that separately describes any features accepted by the system that are prohibited or not specified in this part. Such features shall be described as being "extensions to the <language> language as defined in IEC 61131-3";

f)	shall be able to process in a manner similar to that specified for errors any use of any such extension;

g)	shall be able to process in a manner similar to that specified for errors any use of one of the implementation-dependent features specified in annex D;

h)	shall not use any of the standard data type, function or function block names defined in this part for manufacturer-defined features whose functionality differs from that described in this part,unless such features are identified and treated as noted in rules (e) and (f) above;

i)	shall be accompanied by a document defining, in the form specified in annex A, the formal syntax of all textual language elements supported by the system.

j)	Shall be capable of reading and writing files containing any of the language elements defined as alternatives in the production library_element_declaration in B.0, in the syntax defined in requirement (i) above, encoded according to the "ISO-646 IRV" given as Table 1 - Row 00 of ISO/IEC 10646.

The phrase "be able to" is used in this subclause to permit the implementation of a software switch with which the user may control the reporting of errors.

In cases where compilation or program entry is aborted due to some limitation of tables, etc., an incomplete determination of the kind "no violations were detected, but the examination is incomplete" will satisfy the requirements of this subclause.

### 1.5.2  Program compliance

A programmable controller program complying with the requirements of IEC 61131-3:

a)	shall use only those features specified in this part for the particular language used;

b)	shall not use any features identified as extensions to the language;

c)	shall not rely on any particular interpretation of implementation-dependent features.

The results produced by a complying program shall be the same when processed by any complying system which supports the features used by the program, except as these results are influenced by program execution timing, the use of implementation-dependent features (as listed in annex D) in the program, and the execution of error handling procedures.

# 2.  Common elements

This clause defines textual and graphic elements which are common to all the programmable controller programming languages specified in this Part of IEC 61131.

## 2.1  Use of printed characters

### 2.1.1  Character set

Textual languages and textual elements of graphic languages shall be represented in terms of the "ISO-646 IRV" given as Table 1 - Row 00 of ISO/IEC 10646.

The use of characters from additional character sets , e.g., the "Latin-1 Supplement" given as Table 2 - Row 00 of ISO/IEC 10646, is a typical extension of this standard.  The encoding of such characters shall be consistent with ISO/IEC 10646.

The required character set consists of all the characters in columns 002 through 007 of the "ISO-646 IRV" as defined above, except for lower-case letters.

Table 1 - Character set features

| No. | Description |
| --- | --- |
| 2 | Lower case charactersa |
| 3a 3b | Number sign (#) OR Pound sign (£) |
| 4a 4b | Dollar sign ($) OR Currency sign (¤) |
| 5a 5b | Vertical bar (¦) OR Exclamation mark (!) |
| NOTE The feature numbering in this table is such as to maintain consistency with IEC 61131-3, First Edition. |  |
| a When lower-case letters (feature 2) are supported, the case of letters shall not be significant in language elements except within comments as defined in 2.1.5, string literals as defined in 2.2.2, and variables of type STRING and WSTRING as defined in 2.3.1. |  |

### 2.1.2  Identifiers

An identifier is a string of letters, digits, and underline characters which shall begin with a letter or underline character.

The case of letters shall not be significant in identifiers, e.g., the identifiers abcd, ABCD, and aBCd shall be interpreted identically.

Underlines shall be significant in identifiers, e.g., A_BCD and AB_CD shall be interpreted as different identifiers. Multiple leading or multiple embedded underlines are not allowed; for example, the character sequences __LIM_SW5 and LIM__SW5 are not  valid identifiers. Trailing underlines are not allowed; for example, the character sequence LIM_SW5_ is not a valid identifier.

At least six characters of uniqueness shall be supported in all systems which support the use of identifiers, e.g., ABCDE1 shall be interpreted as different from ABCDE2 in all such systems. The maximum number of characters allowed in an identifier is an implementation-dependent parameter.

Identifier features and examples are shown in table 2.

Table 2 - Identifier features

| No. | Feature description | Examples |
| --- | --- | --- |
| 1 | Upper case and numbers | IW215 IW215Z QX75 IDENT |
| 2 | Upper and lower case, numbers, embedded underlines | All the above plus: LIM_SW_5 LimSw5 abcd ab_Cd |
| 3 | Upper and lower case, numbers, leading or embedded underlines | All the above plus: _MAIN _12V7 |

### 2.1.3  Keywords

Keywords are unique combinations of characters utilized as individual syntactic elements as defined in annex B.  All keywords used in this part are listed in annex C.  Keywords shall not contain imbedded spaces. The case of characters shall not be significant in keywords; for instance, the keywords "FOR" and "for" are syntactically equivalent.  The keywords listed in annex C shall not be used for any other purpose, e.g., variable names or extensions as defined in 1.5.1.

NOTE	National standards organizations can publish tables of translations of the keywords given in annex C.

### 2.1.4 Use of white space

The user shall be allowed to insert one or more characters of "white space" anywhere in the text of programmable controller programs except within keywords, literals, enumerated values, identifiers, directly represented variables as described in subclause 2.4.1.1, or delimiter combinations (e.g., for comments as defined in 2.1.5). "White space" is defined as the SPACE character with encoded value 32 decimal, as well as non-printing characters such as tab, newline, etc. for which no encoding is given in IEC/ISO 10646-1.

### 2.1.5  Comments

User comments shall be delimited at the beginning and end by the special character combinations "(*" and "*)", respectively, as shown in table 3.  Comments shall be permitted anywhere in the program where spaces are allowed, except within character string literals as defined in 2.2.2.  Comments shall have no syntactic or semantic significance in any of the languages defined in this part.

The use of nested comments, e.g., (* (* NESTED *) *), shall be treated as an error according to the provisions of 1.5.1(d).

The maximum number of characters allowed in a comment is an implementation-dependent parameter.

Table 3 - Comment feature

| No. | Feature description | Example |
| --- | --- | --- |
| 1 | Comments | (*****************************) (* A framed comment *) (*****************************) |
| NOTE The example given above represents three separate comments. |  |  |

### 2.1.6  Pragmas

As illustrated in Table 3a, pragmas shall be delimited at the beginning and end by curly brackets "{" and "}", respectively. The syntax and semantics of particular pragma constructions are implementation dependent. Pragmas shall be permitted anywhere in the program where spaces are allowed, except within character string literals as defined in 2.2.2.

NOTE	Curly brackets inside a comment have no semantic meaning; comments inside curly brackets may or may not have semantic meaning depending on the implementation.

Table 3a - Pragma feature

| No. | Feature description | Examples |
| --- | --- | --- |
| 1 | Pragmas | {VERSION 3.1} {AUTHOR JHC} {x := 256, y := 384} |

## 2.2  External representation of data

External representations of data in the various programmable controller programming languages shall consist of numeric literals, character strings, and time literals.

### 2.2.1  Numeric literals

There are two classes of numeric literals: integer literals and real literals.  A numeric literal is defined as a decimal number or a based number.  The maximum number of digits for each kind of numeric literal shall be sufficient to express the entire range and precision of values of all the data types which are represented by the literal in a given implementation.

Single underline characters ( _ ) inserted between the digits of a numeric literal shall not be significant.  No other use of underline characters in numeric literals is allowed.

Decimal literals shall be represented in conventional decimal notation.  Real literals shall be distinguished by the presence of a decimal point.  An exponent indicates the integer power of ten by which the preceding number is to be multiplied to obtain the value represented.  Decimal literals and their exponents can contain a preceding sign (+ or -).

Integer literals can also be represented in base 2, 8, or 16.  The base shall be in decimal notation.  For base 16, an extended set of digits consisting of the letters A through F shall be used, with the conventional significance of decimal 10 through 15, respectively.  Based numbers shall not contain a leading sign (+ or -).

Boolean data shall be represented by integer literals with the value zero (0) or one (1), or the keywords FALSE or TRUE, respectively.

Numeric literal features and examples are shown in table 4.

The data type of a boolean or numeric literal can be specified by adding a type prefix to the literal, consisting of the name of an elementary data type and the '#' sign.  For examples see feature 9 in table 4.

Table 4 - Numeric literals

| No. | Feature description |  | Examples |
| --- | --- | --- | --- |
| 1 | Integer literals |  | -12 0 123_456 +986 |
| 2 | Real literals |  | -12.0 0.0 0.4560 3.14159_26 |
| 3 | Real literals with exponents |  | -1.34E-12 or -1.34e-12 1.0E+6 or 1.0e+6 1.234E6 or 1.234e6 |
| 4 | Base 2 literals |  | 2#1111_1111 (255 decimal) 2#1110_0000 (240 decimal) |
| 5 | Base 8 literals |  | 8#377 (255 decimal) 8#340 (240 decimal) |
| 6 | Base 16 literals |  | 16#FF or 16#ff (255 decimal) 16#E0 or 16#e0 (240 decimal) |
| 7 | Boolean zero and one |  | 0 1 |
| 8 | Boolean FALSE and TRUE |  | FALSE TRUE |
| 9 | Typed literals | DINT#5 (DINT representation of 5) UINT#16#9AF (UINT representation of the hexadecimal value 9AF ) BOOL#0 BOOL#1 BOOL#TRUE BOOL#FALSE |  |
| NOTE The keywords FALSE and TRUE correspond to Boolean values of 0 and 1, respectively. |  |  |  |

### 2.2.2  Character string literals

Character string literals include single-byte or double-byte encoded characters.

A single-byte character string literal is a sequence of zero or more characters from Row 00 of the ISO/IEC 10646 character set prefixed and terminated by the single quote character (').  In single-byte character strings, the three-character combination of the dollar sign ($) followed by two hexadecimal digits shall be interpreted as the hexadecimal representation of the eight-bit character code, as shown in feature 1 of table 5.

A double-byte character string literal is a sequence of zero or more characters from the ISO/IEC 10646 character set prefixed and terminated by the double quote character (").  In double-byte character strings, the five-character combination of the dollar sign ($) followed by four hexadecimal digits shall be interpreted as the hexadecimal representation of the sixteen-bit character code, as shown in feature 2 of table 5.

Two-character combinations beginning with the dollar sign shall be interpreted as shown in table 6 when they occur in character strings.

| Table 5 - Character string literal features |  |  |
| --- | --- | --- |
| No. | Example | Explanation |
| 1 | Single-byte character strings |  |
|  | '' | Empty string (length zero) |
|  | 'A' | String of length one containing the single character A |
|  | ' ' | String of length one containing the "space" character |
|  | '$'' | String of length one containing the "single quote" character |
|  | '"' | String of length one containing the "double quote" character |
|  | '$R$L' | String of length two containing CR and LF characters |
|  | '$0A' | String of length one containing the LF character |
|  | '$$1.00' | String of length five which would print as "$1.00" |
|  | 'ÄË' '$C4$CB' | Equivalent strings of length two |
| 2 | Double-byte character strings |  |
|  | "" | Empty string (length zero) |
|  | "A" | String of length one containing the single character A |
|  | " " | String of length one containing the "space" character |
|  | "'" | String of length one containing the "single quote" character |
|  | "$"" | String of length one containing the "double quote" character |
|  | "$R$L" | String of length two containing CR and LF characters |
|  | "$$1.00" | String of length five which would print as "$1.00" |
|  | "ÄË" "$00C4$00CB" | Equivalent strings of length two |
| 3 | Single-byte typed string literals |  |
|  | STRING#'OK' | String of length two containing two single-byte characters |
| 4 | Double-byte typed string literals |  |
|  | WSTRING#'OK' | String of length two containing two double-byte characters |
| NOTE If a particular implementation supports Feature #4 but not Feature #2, the implementor may specify implementation-dependent syntax and semantics for the use of the double-quote character. |  |  |

| Table 6 - Two-character combinations in character strings |  |  |
| --- | --- | --- |
| No. | Combination | Interpretation when printed |
| 2 | $$ | Dollar sign |
| 3 | $' | Single quote |
| 4 | $L or $l | Line feed |
| 5 | $N or $n | Newline |
| 6 | $P or $p | Form feed (page) |
| 7 | $R or $r | Carriage return |
| 8 | $T or $t | Tab |
| 9 | $" | Double quote |
| NOTE 1 The "newline" character provides an implementation-independent means of defining the end of a line of data for both physical and file I/O; for printing, the effect is that of ending a line of data and resuming printing at the beginning of the next line. NOTE 2 The $' combination is only valid inside single quoted string literals. NOTE 3 The $" combination is only valid inside double quoted string literals. |  |  |

### 2.2.3  Time literals

The need to provide external representations for two distinct types of time-related data is recognized: duration data for measuring or controlling the elapsed time of a control event, and time of day data (which may also include date information) for synchronizing the beginning or end of a control event to an absolute time reference.

Duration and time of day literals shall be delimited on the left by the keywords defined in 2.2.3.1 and 2.2.3.2.

#### 2.2.3.1  Duration

Duration data shall be delimited on the left by the keyword T# or TIME#.  The representation of duration data in terms of days, hours, minutes, seconds, and milliseconds, or any combination thereof, shall be supported as shown in table 7.  The least significant time unit can be written in real notation without exponent.

The units of duration literals can be separated by underline characters.

"Overflow" of the most significant unit of a duration literal is permitted, e.g., the notation T#25h_15m is permitted.

Time units, e.g., seconds, milliseconds, etc., can be represented in upper- or lower- case letters.

As illustrated in Table 7, both positive and negative values are allowed for durations.

Table 7 - Duration literal features

| No. | Feature description | Examples |
| --- | --- | --- |
| 1a | Duration literals without underlines: short prefix | T#14ms T#-14ms T#14.7s T#14.7m T#14.7h t#14.7d t#25h15m t#5d14h12m18s3.5ms |
| 1b | long prefix | TIME#14ms TIME#-14ms time#14.7s |
| 2a | Duration literals with underlines: short prefix | t#25h_15m t#5d_14h_12m_18s_3.5ms |
| 2b | long prefix | TIME#25h_15m time#5d_14h_12m_18s_3.5ms |

#### 2.2.3.2  Time of day and date

Prefix keywords for time of day and date literals shall be as shown in table 8.  As illustrated in table 9, representation of time-of-day and date information shall be as specified by the syntax given in Annex B.1.2.3.2.

Table 8 - Date and time of day literals

| No. | Feature description | Prefix Keyword |
| --- | --- | --- |
| 1 | Date literals (long prefix) | DATE# |
| 2 | Date literals (short prefix) | D# |
| 3 | Time of day literals (long prefix) | TIME_OF_DAY# |
| 4 | Time of day literals (short prefix) | TOD# |
| 5 | Date and time literals (long prefix) | DATE_AND_TIME# |
| 6 | Date and time literals (short prefix) | DT# |

Table 9 - Examples of date and time of day literals

| Long prefix notation | Short prefix notation |
| --- | --- |
| DATE#1984-06-25 date#1984-06-25 | D#1984-06-25 d#1984-06-25 |
| TIME_OF_DAY#15:36:55.36 time_of_day#15:36:55.36 | TOD#15:36:55.36 tod#15:36:55.36 |
| DATE_AND_TIME#1984-06-25-15:36:55.36 date_and_time#1984-06-25-15:36:55.36 | DT#1984-06-25-15:36:55.36 dt#1984-06-25-15:36:55.36 |

## 2.3  Data types

A number of elementary (pre-defined) data types are recognized by this standard.  Additionally, generic data types are defined for use in the definition of overloaded functions (see 2.5.1.4).  A mechanism for the user or manufacturer to specify additional data types is also defined.

### 2.3.1  Elementary data types

The elementary data types, keyword for each data type, number of  bits per data element, and range of values for each elementary data type shall be as shown in table 10.

| Table 10 - Elementary data types |  |  |  |
| --- | --- | --- | --- |
| No. | Keyword | Data type | N a |
| 1 | BOOL | Boolean | 1 h |
| 2 | SINT | Short integer | 8 c |
| 3 | INT | Integer | 16 c |
| 4 | DINT | Double integer | 32 c |
| 5 | LINT | Long integer | 64 c |
| 6 | USINT | Unsigned short integer | 8 d |
| 7 | UINT | Unsigned integer | 16 d |
| 8 | UDINT | Unsigned double integer | 32 d |
| 9 | ULINT | Unsigned long integer | 64 d |
| 10 | REAL | Real numbers | 32 e |
| 11 | LREAL | Long reals | 64 f |
| 12 | TIME | Duration | -- b |
| 13 | DATE | Date (only) | -- b |
| 14 | TIME_OF_DAY or TOD | Time of day (only) | -- b |
| 15 | DATE_AND_TIME or DT | Date and time of Day | -- b |
| 16 | STRING | Variable-length single-byte character string | 8 i,g |
| 17 | BYTE | Bit string of length 8 | 8 j,g |
| 18 | WORD | Bit string of length 16 | 16 j,g |
| 19 | DWORD | Bit string of length 32 | 32 j,g |
| 20 | LWORD | Bit string of length 64 | 64 j,g |
| 21 | WSTRING | Variable-length double-byte character string | 16 i,g |
| a Entries in this column shall be interpreted as specified in the footnotes. b The range of values and precision of representation in these data types is implementation-dependent. c The range of values for variables of this data type is from -(2N-1) to (2N-1)-1. d The range of values for variables of this data type is from 0 to (2N)-1. e The range of values for variables of this data type shall be as defined in IEC 559 for the basic single width floating-point format. f The range of values for variables of this data type shall be as defined in IEC 559 for the basic double width floating-point format. g A numeric range of values does not apply to this data type. h The possible values of variables of this data type shall be 0 and 1, corresponding to the keywords FALSE and TRUE, respectively. i The value of N indicates the number of bits/character for this data type. j The value of N indicates the number of bits in the bit string for this data type. |  |  |  |

### 2.3.2  Generic data types

In addition to the data types shown in table 10, the hierarchy of generic data types shown in table 11 can be used in the specification of inputs and outputs of standard functions and function blocks (see subclause 2.5.1.4).  Generic data types are identified by the prefix "ANY".  The use of generic data types is subject to the following rules:

- Generic data types shall not be used in user-declared program organization units as defined in 2.5.

- The generic type of a subrange derived type (feature 3 of table 12) shall be ANY_INT.

- The generic type of a directly derived type (feature 1 of table 12) shall be the same as the generic type of the elementary type from which it is derived.

- The generic type of all other derived types defined in table 12 shall be ANY_DERIVED.

Table 11 - Hierarchy of generic data types

ANY ANY_DERIVED (Derived data types - see preceding text) ANY_ELEMENTARY ANY_MAGNITUDE ANY_NUM ANY_REAL LREAL REAL ANY_INT LINT, DINT, INT, SINT ULINT, UDINT, UINT, USINT TIME ANY_BIT LWORD, DWORD, WORD, BYTE, BOOL ANY_STRING STRING WSTRING ANY_DATE DATE_AND_TIME DATE, TIME_OF_DAY

### 2.3.3  Derived data types

#### 2.3.3.1  Declaration

Derived (i.e., user- or manufacturer-specified) data types can be declared using the TYPE...END_TYPE textual construction shown in table 12. These derived data types can then be used, in addition to the elementary data types defined in 2.3.1, in variable declarations as defined in 2.4.3.

An enumerated data type declaration specifies that the value of any data element of that type can only take on one of the values given in the associated list of identifiers, as illustrated in table 12.  The enumeration list defines an ordered set of enumerated values, starting with the first identifier of the list, and ending with the last. Different enumerated data types may use the same identifiers for enumerated values. The maximum allowed number of enumerated values is an implementation-dependent parameter.

To enable unique identification when used in a particular context, enumerated literals may be qualified by a prefix consisting of their associated data type name and the '#' sign, similar to typed literals defined in 2.2.1. Such a prefix shall not be used inside an enumeration list. It is an error if sufficient information is not provided in an enumerated literal to determine its value unambiguously.

A subrange declaration specifies that the value of any data element of that type can only take on values between and including the specified upper and lower limits, as illustrated in table 12. It is an error if the value of a value of a subrange type falls outside the specified range of values.

A STRUCT declaration specifies that data elements of that type shall contain sub-elements of specified types which can be accessed by the specified names.  For instance, an element of data type ANALOG_CHANNEL_CONFIGURATION as declared in table 12 will contain a RANGE sub-element of type ANALOG_SIGNAL_RANGE, a MIN_SCALE sub-element of type ANALOG_DATA, and a MAX_SCALE element of type ANALOG_DATA. The maximum number of structure elements, the maximum amount of data that can be contained in a structure, and the maximum number of nested levels of structure element addressing are implementation-dependent parameters.

An ARRAY declaration specifies that a sufficient amount of data storage shall be allocated for each element of that type to store all the data which can be indexed by the specified index subrange(s).  Thus, any element of type ANALOG_16_INPUT_CONFIGURATION as shown in table 12 contains (among other elements) sufficient storage for 16 CHANNEL elements of type ANALOG_CHANNEL_CONFIGURATION.  Mechanisms for access to array elements are defined in 2.4.1.2.  The maximum number of array subscripts, maximum array size and maximum range of subscript values are implementation-dependent parameters.

#### 2.3.3.2  Initialization

The default initial value of an enumerated data type shall be the first identifier in the associated enumeration list, or a value specified by the assignment operator ":=".  For instance, as shown in table 12, No.2, and table 14, No.2, the default initial values of elements of data types ANALOG_SIGNAL_TYPE and ANALOG_SIGNAL_RANGE are SINGLE_ENDED and UNIPOLAR_1_5V, respectively.

For data types with subranges, the default initial values shall be the first (lower) limit of the subrange, unless otherwise specified by an assignment operator.  For instance, as declared in table 12, the default initial value of elements of type ANALOG_DATA is -4095, while the default initial value for the FILTER_PARAMETER sub-element of elements of type ANALOG_16_INPUT_CONFIGURATION is zero.  In contrast, the default initial value of elements of type ANALOG_DATAZ as declared in table 14 is zero.

For other derived data types, the default initial values, unless specified otherwise by the use of the assignment operator ":=" in the TYPE declaration, shall be the default initial values of the underlying elementary data types as defined in table 13.  Further examples of the use of the assignment operator for initialization are given in 2.4.2.

The default maximum length of elements of type STRING and WSTRING shall be an implementation-dependent value unless specified otherwise by a parenthesized maximum length (which shall not exceed the implementation-dependent default value) in the associated declaration.  For example, if type STR10 is declared by

TYPE STR10 : STRING[10] := 'ABCDEF'; END_TYPE

the maximum length, default initial value, and default initial length of data elements of type STR10 are 10 characters, 'ABCDEF', and 6 characters, respectively.  The maximum allowed length of STRING and WSTRING variables is an implementation-dependent parameter.

| Table 12 - Data type declaration features |  |
| --- | --- |
| No. | Feature/textual example |
| 1 | Direct derivation from elementary types, e.g.: TYPE RU_REAL : REAL ; END_TYPE |
| 2 | Enumerated data types, e.g.: TYPE ANALOG_SIGNAL_TYPE : (SINGLE_ENDED, DIFFERENTIAL) ; END_TYPE |
| 3 | Subrange data types, e.g.: TYPE ANALOG_DATA : INT (-4095..4095) ; END_TYPE |
| 4 | Array data types, e.g.: TYPE ANALOG_16_INPUT_DATA : ARRAY [1..16] OF ANALOG_DATA ; END_TYPE |
| 5 | Structured data types, e.g.: TYPE ANALOG_CHANNEL_CONFIGURATION : STRUCT RANGE : ANALOG_SIGNAL_RANGE ; MIN_SCALE : ANALOG_DATA ; MAX_SCALE : ANALOG_DATA ; END_STRUCT ; ANALOG_16_INPUT_CONFIGURATION : STRUCT SIGNAL_TYPE : ANALOG_SIGNAL_TYPE ; FILTER_PARAMETER : SINT (0..99) ; CHANNEL : ARRAY [1..16] OF ANALOG_CHANNEL_CONFIGURATION ; END_STRUCT ; END_TYPE |
| NOTE For examples of the use of these types in variable declarations, see 2.3.3.3, 2.4.1.2, and table 17. |  |

Table 13 - Default initial values of elementary data types

| Data type(s) | Initial value |
| --- | --- |
| BOOL, SINT, INT, DINT, LINT | 0 |
| USINT, UINT, UDINT, ULINT | 0 |
| BYTE, WORD, DWORD, LWORD | 0 |
| REAL, LREAL | 0.0 |
| TIME | T#0S |
| DATE | D#0001-01-01 |
| TIME_OF_DAY | TOD#00:00:00 |
| DATE_AND_TIME | DT#0001-01-01-00:00:00 |
| STRING | '' (the empty string) |
| WSTRING | "" (the empty string) |

| Table 14 - Data type initial value declaration features |  |
| --- | --- |
| No. | Feature/textual example |
| 1 | Initialization of directly derived types, e.g.: TYPE FREQ : REAL := 50.0 ; END_TYPE |
| 2 | Initialization of enumerated data types, e.g.: TYPE ANALOG_SIGNAL_RANGE : (BIPOLAR_10V, (* -10 to +10 VDC *) UNIPOLAR_10V, (* 0 to +10 VDC *) UNIPOLAR_1_5V, (* + 1 to + 5 VDC *) UNIPOLAR_0_5V, (* 0 to + 5 VDC *) UNIPOLAR_4_20_MA, (* + 4 to +20 mADC *) UNIPOLAR_0_20_MA (* 0 to +20 mADC *) ) := UNIPOLAR_1_5V ; END_TYPE |
| 3 | Initialization of subrange data types, e.g.: TYPE ANALOG_DATAZ : INT (-4095..4095) := 0 ; END_TYPE |
| 4 | Initialization of array data types, e.g.: TYPE ANALOG_16_INPUT_DATAI : ARRAY [1..16] OF ANALOG_DATA := [8(-4095), 8(4095)] ; END_TYPE |
| 5 | Initialization of structured data type elements, e.g.: TYPE ANALOG_CHANNEL_CONFIGURATIONI : STRUCT RANGE : ANALOG_SIGNAL_RANGE ; MIN_SCALE : ANALOG_DATA := -4095 ; MAX_SCALE : ANALOG_DATA := 4095 ; END_STRUCT ; END_TYPE |
| 6 | Initialization of derived structured data types, e.g.: TYPE ANALOG_CHANNEL_CONFIGZ : ANALOG_CHANNEL_CONFIGURATIONI := (MIN_SCALE := 0, MAX_SCALE := 4000); END_TYPE |

#### 2.3.3.3 Usage

The usage of variables which are declared (as defined in 2.4.3.1) to be of derived data types shall conform to the following rules:

(1) A single-element variable, as defined in 2.4.1.1, of a derived type, can be used anywhere that a variable of its "parent's" type can be used, e.g. variables of the types RU_REAL and FREQ as shown in tables 12 and 14 can be used anywhere that a variable of type REAL could be used, and variables of type ANALOG_DATA can be used anywhere that a variable of type INT could be used.

This rule can be applied recursively. For example, given the declarations below, the variable R3 of type R2 can be used anywhere a variable of type REAL can be used:

TYPE R1 : REAL := 1.0 ; END_TYPE
TYPE R2 : R1 ; END_TYPE
VAR R3: R2; END_VAR

(2) An element of a multi-element variable, as defined in 2.4.1.2, can be used anywhere the "parent" type can be used, e.g., given the declaration of ANALOG_16_INPUT_DATA in table 12 and the declaration

VAR INS : ANALOG_16_INPUT_DATA ; END_VAR

the variables INS[1] through INS[16] can be used anywhere that a variable of type INT could be used.

This rule can also be applied recursively, e.g., given the declarations of ANALOG_16_INPUT_CONFIGURATION, ANALOG_CHANNEL_CONFIGURATION, and ANALOG_DATA in table 12 and the declaration

VAR CONF : ANALOG_16_INPUT_CONFIGURATION ; END_VAR

the variable CONF.CHANNEL[2].MIN_SCALE can be used anywhere that a variable of type INT could be used.

## 2.4  Variables

In contrast to the external representations of data described in 2.2, variables provide a means of identifying data objects whose contents may change, e.g., data associated with the inputs, outputs, or memory of the programmable controller. A variable can be declared to be one of the elementary types defined in 2.3.1, or one of the derived types which are declared as defined in 2.3.3.1.

### 2.4.1  Representation

#### 2.4.1.1  Single-element variables

A single-element variable is defined as a variable which represents a single data element of one of the elementary types defined in 2.3.1; a derived enumeration or subrange type as defined in 2.3.3.1; or a derived type whose "parentage", as defined recursively in 2.3.3.3, is traceable to an elementary, enumeration or subrange type.  This subclause defines the means of representing such variables symbolically, or alternatively in a manner which directly represents the association of the data element with physical or logical locations in the programmable controller's input, output, or memory structure.

Identifiers, as defined in 2.1.2, shall be used for symbolic  representation of variables.

Direct representation of a single-element variable shall be provided by a special symbol formed by the concatenation of the percent sign "%" (character code 037 decimal in Table 1 - Row 00 of ISO/IEC 10646), a location prefix and a size prefix from table 15, and one or more unsigned integers, separated by periods (.).

In the case that a directly represented variable is used in a location assignment to an internal variable in the declaration part of a program or a function block type as defined in 2.4.3.1, an asterisk "*" shall be used in place of the size prefix and the one or several unsigned integers in the concatenation to indicate that the direct representation is not yet fully specified. The percent sign and the location prefix I, Q or M from table 15 shall always be present in the direct representation.

In both cases the use of this feature requires that the location of the variable so declared shall be fully specified inside the VAR_CONFIG...END_VAR construction of the configuration as defined in 2.7.1 for every instance of the containing type.

It is an error if any of the full specifications in the VAR_CONFIG...END_VAR construction is missing for any incomplete address specification expressed by the asterisk notation in any instance of programs or function block types which contain such incomplete specifications.

EXAMPLES

%QX75 and %Q75	Output bit 75

%IW215	Input word location 215

%QB7	Output byte location 7

%MD48	Double word at memory location 48

%IW2.5.7.1	See explanation below

%Q*	Output at not yet specified location

The manufacturer shall specify the correspondence between the direct representation of a variable and the physical or logical location of the addressed item in memory, input or output.  When a direct representation is extended with additional integer fields separated by periods, it shall be interpreted as a hierarchical physical or logical address with the leftmost field representing the highest level of the hierarchy, with successively lower levels appearing to the right.  For instance, the variable %IW2.5.7.1 may represent the first "channel" (word) of the seventh "module" in the fifth "rack" of the second "I/O bus" of a programmable controller system.

The use of hierarchical addressing to permit a program in one programmable controller system to access data in another programmable controller shall be considered a language extension.

The use of directly represented variables is permitted in function blocks as defined in 2.5.2, programs as defined in 2.5.3, and in configurations and resources as defined in 2.7.1.  The maximum number of levels of hierarchical addressing is an implementation-dependent parameter.

| Table 15 - Location and size prefix features for directly represented variables |  |  |  |
| --- | --- | --- | --- |
| No. | Prefix | Meaning | Default data type |
| 1 | I | Input location |  |
| 2 | Q | Output location |  |
| 3 | M | Memory location |  |
| 4 | X | Single bit size | BOOL |
| 5 | None | Single bit size | BOOL |
| 6 | B | Byte (8 bits) size | BYTE |
| 7 | W | Word (16 bits) size | WORD |
| 8 | D | Double word (32 bits) size | DWORD |
| 9 | L | Long (quad) word (64 bits) size | LWORD |
| 10 | Use of an asterisk (*) to indicate not yet specified location (NOTE 2) |  |  |
| NOTE 1 National standards organizations can publish tables of translations of these prefixes. NOTE 2 Use of Feature No. 10 in this table requires Feature No. 11 of table 49 and vice versa. |  |  |  |

#### 2.4.1.2  Multi-element variables

The multi-element variable types defined in this standard are arrays and structures.

An array is a collection of data elements of the same data type referenced by one or more subscripts enclosed in brackets and separated by commas.  In the ST language defined in subclause 3.3, a subscript shall be an expression yielding a value corresponding to one of the sub-types of generic type ANY_INT as defined in table 11. The form of subscripts in the IL language defined in subclause 3.2, and the graphic languages defined in clause 4, is restricted to single-element variables or integer literals.

An example of the use of array variables in the ST language as defined in 3.3 is:

OUTARY[%MB6,SYM] := INARY[0] + INARY[7] - INARY[%MB6] * %IW62 ;

A structured variable is a variable which is declared to be of a type which has previously been specified to be a data structure, i.e., a data type consisting of a collection of named elements.

An element of a structured variable shall be represented by two or more identifiers or array accesses separated by single periods (.). The first identifier represents the name of the structured element, and subsequent identifiers represent the sequence of component names to access the particular data element within the data structure.

For instance, if the variable MODULE_5_CONFIG has been declared to be of type ANALOG_16_INPUT_CONFIGURATION as shown in table 12, the following statements in the ST language defined in 3.3 would cause the value SINGLE_ENDED to be assigned to the element SIGNAL_TYPE of the variable MODULE_5_CONFIG, while the value BIPOLAR_10V would be assigned to the RANGE sub-element of the fifth CHANNEL element of MODULE_5_CONFIG:

MODULE_5_CONFIG.SIGNAL_TYPE := SINGLE_ENDED;
MODULE_5_CONFIG.CHANNEL[5].RANGE := BIPOLAR_10V;

### 2.4.2  Initialization

When a configuration element (resource or configuration) is "started" as defined in 1.4.1, each of the variables associated with the configuration element and its programs can take on one of the following initial values:

- the value the variable had when the configuration element was "stopped" (a retained value);

- a user-specified initial value;

- the default initial value for the variable's associated data type.

The user can declare that a variable is to be retentive by using the RETAIN qualifier specified in table 16a, when this feature is supported by the implementation.

The initial value of a variable upon starting of its associated configuration element shall be determined according to the following rules:

1) If the starting operation is a "warm restart" as defined in IEC 61131-1, the initial values of retentive variables shall be their retained values as defined above.

2) If the operation is a "cold restart" as defined in IEC 61131-1, the initial values of retentive variables shall be the user-specified initial values, or the default value, as defined in 2.3.3.2, for the associated data type of any variable for which no initial value is specified by the user.

3) Non-retained variables shall be initialized to the user-specified initial values, or to the default value, as defined in 2.3.3.2, for the associated data type of any variable for which no initial value is specified by the user.

4) Variables which represent inputs of the programmable controller system as defined in IEC 61131-1 shall be initialized in an implementation-dependent manner.

### 2.4.3  Declaration

Each declaration of a program organization unit type (i.e., each declaration of a program, function, or function block, as defined in 2.5) shall contain at its beginning at least one declaration part which specifies the types (and, if necessary, the physical or logical location) of the variables used in the organization unit.  This declaration part shall have the textual form of one of the keywords VAR, VAR_INPUT, or VAR_OUTPUT as defined in table 16a, followed in the case of VAR by zero or one occurrence of the qualifiers RETAIN,NON_RETAIN or the qualifier CONSTANT, and in the case of VAR_INPUT or VAR_OUTPUT by zero or one occurrence of the qualifier RETAIN or NON_RETAIN, followed by one or more declarations separated by semicolons and terminated by the keyword END_VAR.  When a programmable controller supports the declaration by the user of initial values for variables, this declaration shall be accomplished in the declaration part(s) as defined in this subclause.

| Table 16a - Variable declaration keywords |  |
| --- | --- |
| Keyword | Variable usage |
| VAR | Internal to organization unit |
| VAR_INPUT | Externally supplied, not modifiable within organization unit |
| VAR_OUTPUT | Supplied by organization unit to external entities |
| VAR_IN_OUT | Supplied by external entities - Can be modified within organization unit |
| VAR_EXTERNAL | Supplied by configuration via VAR_GLOBAL (2.7.1) Can be modified within organization unit |
| VAR_GLOBAL | Global variable declaration (2.7.1) |
| VAR_ACCESS | Access path declaration (2.7.1) |
| VAR_TEMP | Temporary storage for variables in function blocks and programs (2.4.3) |
| VAR_CONFIG | Instance-specific initialization and location assignment. |
| RETAINb,c,d,e | Retentive variables (see preceding text) |
| NON_RETAINb,c,d,e | Non-retentive variables (see preceding text) |
| CONSTANTa | Constant (variable cannot be modified) |
| AT | Location assignment (2.4.3.1) |
| a The CONSTANT qualifier shall not be used in the declaration of function block instances as described in 2.5.2.1. b The RETAIN and NON_RETAIN qualifiers may be used for variables declared in VAR, VAR_INPUT, VAR_OUTPUT, and VAR_GLOBAL blocks but not in VAR_IN_OUT blocks and not for individual elements of structures. c Usage of RETAIN and NON_RETAIN for function block and program instances is allowed. The effect is that all members of the instance are treated as RETAIN or NON_RETAIN, except if: - the member is explicitly declared as RETAIN or NON_RETAIN in the function block or program type definition; - the member itself is a function block. d Usage of RETAIN and NON_RETAIN for instances of structured data types is allowed. The effect is that all structure members, also those of nested structures, are treated as RETAIN or NON_RETAIN. e Both RETAIN and NON_RETAIN are features. If a variable is neither explicitly declared as RETAIN nor as NON_RETAIN the "warm start" behaviour of the variable is implementation dependent.. |  |
| NOTE 1 The usage of these keywords is a feature of the program organization unit or configuration element in which they are used. Normative requirements for the use of these keywords are given in 2.4.3.1, 2.4.3.2, 2.5 and 2.7. NOTE 2 Examples of the use of VAR_IN_OUT variables are given in figures 11b and 12. |  |

Within function blocks and programs, variables can be declared in a VAR_TEMP...END_VAR construction. These variables are allocated and initialized at each invocation of an instance of the program organization unit, and do not persist between invocations.

The scope (range of validity) of the declarations contained in the declaration part shall be local to the program organization unit in which the declaration part is contained.  That is, the declared variables shall not be accessible to other program organization units except by explicit argument passing via variables which have been declared as inputs or outputs of those units.  The one exception to this rule is the case of variables which have been declared to be global, as defined in 2.7.1.  Such variables are only accessible to a program organization unit via a VAR_EXTERNAL declaration.  The type of a variable declared in a VAR_EXTERNAL block shall agree with the type declared in the VAR_GLOBAL block of the associated program, configuration or resource.

It shall be an error if:

- any program organization unit attempts to modify the value of a variable that has been declared with the CONSTANT qualifier;

- a variable declared as VAR_GLOBAL CONSTANT in a configuration element or program organization unit (the "containing element") is used in a VAR_EXTERNAL declaration (without the CONSTANT qualifier) of any element contained within the containing element as illustrated below.

The maximum number of variables allowed in a variable declaration block is an implementation-dependent parameter.

Table 16b - Usages of VAR_GLOBAL, VAR_EXTERNAL and CONSTANT declarations

| Declaration in containing element | Declaration in contained element | Allowed? |
| --- | --- | --- |
| VAR_GLOBAL X ... | VAR_EXTERNAL CONSTANT X... | Yes |
| VAR_GLOBAL X ... | VAR_EXTERNAL X... | Yes |
| VAR_GLOBAL CONSTANT X ... | VAR_EXTERNAL CONSTANT X ... | Yes |
| VAR_GLOBAL CONSTANT X ... | VAR_EXTERNAL X ... | NO |

#### 2.4.3.1  Type assignment

As shown in table 17, the VAR...END_VAR construction shall be used to specify data types and retentivity for directly represented variables.  This construction shall also be used to specify data types, retentivity, and (where necessary, in programs and VAR_GLOBAL declarations only) the physical or logical location of symbolically represented single- or multi-element variables.  The usage of the VAR_INPUT, VAR_OUTPUT, and VAR_IN_OUT constructions is defined in 2.5.

The assignment of a physical or logical address to a symbolically represented variable shall be accomplished by the use of the AT keyword.  Where no such assignment is made, automatic allocation of the variable to an appropriate location in the programmable controller memory shall be provided.

The asterisk notation (feature No. 10 in table 15) can be used in address assignments inside programs and function block types to denote not yet fully specified locations for directly represented variables.

| Table 17 - Variable type assignment features |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Feature/examples |  |  |  |  |  |
| 1a | Declaration of directly represented variables |  |  |  |  |  |
|  | VAR AT %IW6.2 : WORD; AT %MW6 : INT ; END_VAR | 16-bit string (note 2) 16-bit integer, initial value = 0 |  |  |  |  |
| 2a | Declaration of directly represented retentive variables |  |  |  |  |  |
|  | VAR RETAIN AT %QW5 : WORD ; END_VAR | At cold restart, will be initialized to a 16-bit string with value 16#0000 |  |  |  |  |
| 3a | Declaration of locations of symbolic variables |  |  |  |  |  |
|  | VAR_GLOBAL LIM_SW_S5 AT %IX27 : BOOL; |  |  | Assigns input bit 27 to the Boolean variable LIM_SW_5 (note 2) |  |  |
|  | CONV_START AT %QX25 : BOOL; |  |  | Assigns output bit 25 to the Boolean variable CONV_START |  |  |
|  | TEMPERATURE AT %IW28: INT; |  |  | Assigns input word 28 to the integer variable TEMPERATURE (note 2) |  |  |
|  | VAR C2 AT %Q* : BYTE ; END_VAR |  |  | Assigns not yet located output byte to bitstring variable C2 of length 8 bits |  |  |
| 4a | Array location assignment |  |  |  |  |  |
|  | VAR INARY AT %IW6 : ARRAY [0..9] OF INT; END_VAR | Declares an array of 10 integers to be allocated to contiguous input locations starting at %IW6 (note 2) |  |  |  |  |
| 5 | Automatic memory allocation of symbolic variables |  |  |  |  |  |
|  | VAR CONDITION_RED : BOOL; IBOUNCE : WORD ; MYDUB : DWORD ; AWORD, BWORD, CWORD : INT; MYSTR: STRING[10] ; END_VAR |  | Allocates a memory bit to the Boolean variable CONDITION_RED. Allocates a memory word to the 16-bit string variable IBOUNCE. Allocates a double memory word to the 32-bit-string variable MYDUB. Allocates 3 separate memory words for the integer variables AWORD, BWORD, and CWORD. Allocates memory to contain a string with a maximum length of 10 characters. After initialization, the string has length 0 and contains the empty string ''. |  |  |  |
| 6 | Array declaration |  |  |  |  |  |
|  | VAR THREE : ARRAY[1..5,1..10,1..8] OF INT; END_VAR |  |  | Allocates 400 memory words for a three-dimensional array of integers |  |  |
| 7 | Retentive array declaration |  |  |  |  |  |
|  | VAR RETAIN RTBT: ARRAY[1..2,1..3] OF INT; END_VAR |  |  |  | Declares retentive array RTBT with "cold restart" initial values of 0 for all elements |  |
| 8 | Declaration of structured variables |  |  |  |  |  |
|  | VAR MODULE_8_CONFIG : ANALOG_16_INPUT_CONFIGURATION; END_VAR |  |  |  |  | Declaration of a variable of derived data type (see table 12) |
| a If directly represented variables are explicitly located, features 1 to 4 can only be used in PROGRAM and VAR_GLOBAL declarations, as defined in 2.5.3 and 2.7.1, respectively. If the asterisk notation of feature 10 in table 15 is used to indicate instance specific location assignment of a partly specified directly represented variable, features 1 and 2 can not be used, and features 3 and 4 can only be used in declarations of internal variables of function blocks and programs, as defined in 2.5.2 and 2.5.3, respectively. |  |  |  |  |  |  |
| NOTE 1 Initialization of system inputs is implementation-dependent; see 2.4.2. NOTE 2 The NOTES to Table 16a also apply to this table. |  |  |  |  |  |  |

#### 2.4.3.2  Initial value assignment

The VAR...END_VAR construction can be used as shown in table 18 to specify initial values of directly represented variables or symbolically represented single- or multi-element variables.

Initial values can also be specified by using the instance-specific initialization feature provided by the VAR_CONFIG...END_VAR construct described in 2.7.1 (table 49, feature 11). Instance-specific initial values always override type-specific initial values.

NOTE	The usage of the VAR_INPUT, VAR_OUTPUT, and VAR_IN_OUT constructions is defined in subclause 2.5.

Initial values cannot be given in VAR_EXTERNAL declarations.

During initialization of arrays, the rightmost subscript of an array shall vary most rapidly with respect to filling the array from the list of initialization variables.

Parentheses can be used as a repetition factor in array initialization lists, e.g., 2(1,2,3) is equivalent to the initialization sequence 1,2,3,1,2,3.

If the number of initial values given in the initialization list exceeds the number of array entries, the excess (rightmost) initial values shall be ignored.  If the number of initial values is less than the number of array entries, the remaining array entries shall be filled with the default initial values for the corresponding data type.  In either case, the user shall be warned of this condition during preparation of the program for execution.

When a variable is declared to be of a derived, structured data type as defined in 2.3.3.1, initial values for the elements of the variable can be declared in a parenthesized list following the data type identifier, as shown in table 18.  Elements for which initial values are not listed in the initial value list shall have the default initial values declared for those elements in the data type declaration.

When a variable is declared to be a function block instance, as defined in 2.5.2.2, initial values for the inputs and any accessible variables of the function block can be declared in a parenthesized list following the assignment operator that follows the function block type identifier as shown in table 18. Elements for which initial values are not listed shall have the default initial values declared for those elements in the function block declaration.

| Table 18 - Variable initial value assignment features |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Feature/examples |  |  |  |  |  |  |  |  |
| 1 a | Initialization of directly represented variables |  |  |  |  |  |  |  |  |
|  | VAR AT %QX5.1 : BOOL :=1; AT %MW6 : INT := 8 ; END_VAR |  |  |  |  | Boolean type, initial value = 1 Initializes a memory word to integer 8 |  |  |  |
| 2 a | Initialization of directly represented retentive variables |  |  |  |  |  |  |  |  |
|  | VAR RETAIN AT %QW5 : WORD := 16#FF00 ; END_VAR |  |  | At cold restart, the 8 most significant bits of the 16-bit string at output word 5 are to be initialized to 1 and the 8 least significant bits to 0 |  |  |  |  |  |
| 3 a | Location and initial value assignment to symbolic variables |  |  |  |  |  |  |  |  |
|  | VAR VALVE_POS AT %QW28 : INT := 100; END_VAR |  |  |  |  |  |  |  | Assigns output word 28 to the integer variable VALVE_POS, with an initial value of 100 |
| 4 a | Array location assignment and initialization |  |  |  |  |  |  |  |  |
|  | VAR OUTARY AT %QW6 : ARRAY[0..9] OF INT := [10(1)]; END_VAR |  |  |  |  |  | Declares an array of 10 integers to be allocated to contiguous output locations starting at %QW6, each with an initial value of 1 |  |  |
| 5 | Initialization of symbolic variables |  |  |  |  |  |  |  |  |
|  | VAR MYBIT : BOOL := 1 ; OKAY : STRING[10] := 'OK'; END_VAR |  | Allocates a memory bit to the Boolean variable MYBIT with an initial value of 1. Allocates memory to contain a string with a maximum length of 10 characters. After initialization, the string has length 2 and contains the two-byte sequence of characters 'OK' (decimal 79 and 75 respectively), in an order appropriate for printing as a character string. |  |  |  |  |  |  |
| 6 | Array initialization |  |  |  |  |  |  |  |  |
|  | VAR BITS : ARRAY[0..7] OF BOOL := [1,1,0,0,0,1,0,0] ; TBT : ARRAY [1..2,1..3] OF INT := [1,2,3(4),6] ; END_VAR |  | Allocates 8 memory bits to contain initial values BITS[0]:= 1, BITS[1] := 1,..., BITS[6]:= 0, BITS[7] := 0. Allocates a 2-by-3 integer array TBT with initial values TBT[1,1]:=1, TBT[1,2]:=2, TBT[1,3]:=4, TBT[2,1]:=4, TBT[2,2]:=4, TBT[2,3]:=6. |  |  |  |  |  |  |
| 7 | Retentive array declaration and initialization |  |  |  |  |  |  |  |  |
|  | VAR RETAIN RTBT : ARRAY(1..2,1..3) OF INT := [1,2,3(4)]; END_VAR | Declares retentive array RTBT with "cold restart" initial values of: RTBT[1,1] := 1, RTBT[1,2] := 2, RTBT[1,3] := 4, RTBT[2,1] := 4, RTBT[2,2] := 4, RTBT[2,3] := 0. |  |  |  |  |  |  |  |
| 8 | Initialization of structured variables |  |  |  |  |  |  |  |  |
|  | VAR MODULE_8_CONFIG: ANALOG_16_INPUT_CONFIGURATION := (SIGNAL_TYPE := DIFFERENTIAL, CHANNEL := [4((RANGE := UNIPOLAR_1_5V)), (RANGE:= BIPOLAR_10_V, MIN_SCALE := 0, MAX_SCALE := 500)]); END_VAR |  |  |  |  |  |  | Initialization of a variable of derived data type (see table 12) This example illustrates the declaration of a non-default initial value for the fifth element of the CHANNEL array of the variable MODULE_8_CONFIG. |  |
| 9 | Initialization of constants |  |  |  |  |  |  |  |  |
|  | VAR CONSTANT PI : REAL := 3.141592 ; END_VAR |  |  |  |  |  |  |  |  |
| 10 | Initialization of function block instances |  |  |  |  |  |  |  |  |
|  | VAR TempLoop : PID := (PropBand := 2.5, Integral := T#5s); END_VAR |  |  |  | Allocates initial values to inputs and outputs of a function block instance |  |  |  |  |
| a Features 1 to 4 can only be used in PROGRAM and VAR_GLOBAL declarations, as defined in 2.5.3 and 2.7.1 respectively. |  |  |  |  |  |  |  |  |  |

## 2.5  Program organization units

The program organization units defined in this Part of IEC 61131 are the function, function block, and program.  These program organization units can be delivered by the manufacturer, or programmed by the user by the means defined in this part of the standard.

Program organization units shall not be recursive; that is, the invocation of a program organization unit shall not cause the invocation of another program organization unit of the same type.

The information necessary to determine execution times of program organization units may consist of one or more implementation-dependent parameters.

### 2.5.1  Functions

For the purposes of programmable controller programming languages, a function is defined as a program organization unit which, when executed, yields exactly one data element, which is considered to be the function result, and arbitrarily many additional output elements (VAR_OUTPUT and VAR_IN_OUT). As for any data element, the function result can be multi-valued, e.g., an array or structure.The invocation of a function can be used in textual languages as an operand in an expression.  For example, the SIN and COS functions could be used as shown in figure 4.

| a) | VAR X,Y,Z,RES1,RES2 : REAL; EN1,V : BOOL; END_VAR RES1 := DIV(IN1 := COS(X), IN2 := SIN(Y), ENO => EN1); RES2 := MUL (SIN(X), COS(Y)); Z: = ADD(EN := EN1, IN1 := RES1, IN2 := RES2, ENO => V); |
| --- | --- |
| b) | +-----+ +------+ +------+ X ---+-¦ COS ¦--+ -¦EN ENO¦-----¦EN ENO¦--- V ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +-----+ +---¦ DIV ¦-----¦ ADD ¦--- Z ¦ ¦ ¦ ¦ ¦ ¦ +-----+ ¦ ¦ +-¦ ¦ Y -+---¦ SIN ¦------¦ ¦ ¦ +------+ ¦ ¦ ¦ ¦ +------+ ¦ ¦ ¦ +-----+ ¦ ¦ ¦ ¦ ¦ ¦ +-----+ +------+ ¦ ¦ +-¦ SIN ¦--+ -¦EN ENO¦- ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +-----+ +- -¦ MUL ¦---+ ¦ ¦ ¦ ¦ +-----+ ¦ ¦ +---¦ COS ¦------¦ ¦ ¦ ¦ +------+ +-----+ |

NOTE	This figure shows two different representations of the same functionality. It is not required to support any automatic transformation between the two forms of representation.

Figure 4 - Examples of function usage

a) Structured Text (ST) language - see subclause 3.3
b)Function Block Diagram (FBD) language - see subclause 4.3

Functions shall contain no internal state information, i.e., invocation of a function with the same arguments (input variables VAR_INPUT and in-out variables VAR_IN_OUT) shall always yield the same values (output variables VAR_OUTPUT, in-out variables VAR_IN_OUT and function result). It shall be an error if external variables as defined in 2.4.3 cause the violation of this rule.

Any function type which has already been declared can be used in the declaration of another program organization unit, as shown in figure 3.

#### 2.5.1.1  Representation

Functions and their invocation can be represented either graphically or textually.

In the textual languages defined in clause 3 of this Part, the invocation of functions shall be according to the following rules:

- Input argument assignment shall follow the rules given in table 19a.

- Assignments of output variables of the function shall be either empty or to variables.

- Assignments to VAR_IN_OUT arguments shall be variables.

- Assignments to VAR_INPUT arguments may be empty (feature 1 of table 19a), constants, variables or function calls. In the latter case the function result is used as the actual argument.

In the graphic languages defined in clause 4 of this part, functions shall be represented as graphic blocks according to the following rules:

1.	The form of the block shall be rectangular or square.

2.	The size and proportions of the block may vary depending on the number of inputs and other information to be displayed.

3.	The direction of processing through the block shall be from left to right (input variables on the left and output variables on the right).

4.	The function name or symbol, as specified below, shall be located inside the block.

5.	Provision shall be made for input and output variable names appearing at the inside left and right sides of the block respectively when the block represents:

- one of the standard functions defined in subclause 2.5.1.5, when the given graphical form includes the variable names; or

- any additional function declared as specified in subclause 2.5.1.3.

This usage is subject to the following provisions:

- Where no names are given for input variables in standard functions, the default names IN1, IN2,... shall apply in top-to-bottom order.

- When a standard function has a single unnamed input, the default name IN shall apply.

- The default names described above may, but need not appear at the inside left-hand side of the graphic representation.

6.	An additional input EN and/or output ENO as specified in 2.5.1.2 may be used. If present, they shall be shown at the upper most positions at the left and right side of the block, respectively.

7.	The function result shall be shown at the upper most position at the right side of the block, except if there is an ENO output, in which case the function result shall be shown at the next position below the ENO output. Since the name of the function is used for the assignment of its output value as specified in 2.5.1.3, no output variable name shall be shown at the right side of the block.

8.	Argument connections (including function result) shall be shown by signal flow lines.

9.	Negation of Boolean signals shall be shown by placing an open circle just outside of the input or output line intersection with the block.  In the character set defined in 2.1.1, this shall be represented by the upper case alphabetic "O", as shown in table 19.

10.	All inputs and outputs (including function result) of a graphically represented function shall be represented by a single line outside the corresponding side of the block, even though the data element may be a multi-element variable.

11.	Function results and function outputs (VAR_OUTPUT) can be connected to a variable, used as input to other function blocks or functions, or can be left unconnected.

12.	It shall be an error if any VAR_IN_OUT variable of any function block invocation or function invocation within a POU is not"properly mapped".  A VAR_IN_OUT variable is "properly mapped" if it is connected graphically at the left, or assigned using the ":=" operator in a textual invocation, to a variable declared (without the CONSTANT qualifier) in a VAR_IN_OUT, VAR, VAR_OUT, or VAR_EXTERNAL block of the containing program organization unit, or to a "properly mapped" VAR_IN_OUT of another contained function block instance or function invocation.

13.	A "properly mapped" (see rule 12 above) VAR_IN_OUT variable of a function block instance or a function invocation can be connected graphically at the right, or assigned using the ":=" operator in a textual assignment statement, to a variable declared in a VAR, VAR_OUT or VAR_EXTERNAL block of the containing program organization unit.  It shall be an error if such connection would lead to an ambiguous value of the variable so connected.

Table 19 - Graphical negation of Boolean signals

| No. | Featurea, b | Representation |
| --- | --- | --- |
| 1 | Negated input | +---+ ---O¦ ¦--- +---+ |
| 2 | Negated output | +---+ ----¦ ¦O--- +---+ |
| a If either of these features is supported for functions, it shall also be supported for function blocks as defined in 2.5.2, and vice versa. b The use of these constructs is forbidden for in-out variables. |  |  |

Figure 5 illustrates both the graphical and equivalent textual use of functions, including the use of a standard function (ADD) with no defined formal argument names; a standard function (SHL) with defined formal argument names; the same function with additional use of EN input and negated ENO output; and a user-defined function (INC) with defined formal argument names.

| Example | Explanation |
| --- | --- |
| +-----+ ¦ ADD ¦ B---¦ ¦---A C---¦ ¦ D---¦ ¦ +-----+ | Graphical use of ADD function (See 2.5.1.5.2) (FBD language; see 4.3) (No formal variable names) |
| A := ADD(B,C,D); | Textual use of ADD function (ST language; see 3.3) |
| +-----+ ¦ SHL ¦ B---¦IN ¦---A C---¦N ¦ +-----+ | Graphical use of SHL function (See 2.5.1.5.3) (FBD language; see 4.3) (Formal argument names) |
| A := SHL(IN := B,N := C); | Textual use of SHL function (ST language; see 3.3) |
| +---------+ ¦ SHL ¦ ENABLE---¦EN ENO¦O--NO_ERR B---¦IN ¦---A C---¦N ¦ +---------+ | Graphical use of SHL function (See 2.5.1.5.3) (FBD language; see 4.3) (Formal argument names; use of EN input and negated ENO output) |
| A := SHL(EN:=ENABLE, IN:=B, N:=C, NOT ENO => NO_ERR); | Textual use of SHL function (ST language; see 3.3) |
| +-----+ ¦ INC ¦ ¦ ¦---A X---¦V---V¦---X +-----+ | Graphical use of user-defined INC function (FBD language - see 4.3) (Formal argument names for VAR_IN_OUT) |
| A := INC(V := X) ; | Textual use of INC function (ST language - see 3.3) |

Figure 5 - Use of formal argument names

Features for the textual invocation of functions are defined in table 19a. The textual invocation of a function shall consist of the function name followed by a list of arguments. In the ST language defined in subclause 3.3, the arguments shall be separated by commas and this list shall be delimited on the left and right by parentheses.

In feature 1 of table 19a (formal invocation), the argument list has the form of a set of assignments of actual values to the formal argument names (formal argument list), that is: (1) assignments of values to input and in-out variables using the ":=" operator, and (2) assignments of the values of output variables to variables using the "=>" operator. The ordering of arguments in the list shall be insignificant. In feature 1 of table 19a, any variable not assigned a value in the list shall have the default value, if any, assigned in the function specification, or the default value for the associated data type.

In feature 2 of table 19a (non-formal invocation), the argument list shall contain exactly the same number of arguments, in exactly the same order and of the same data types as given in the function definition, except the execution control arguments EN and ENO.

Table 19a - Textual invocation of functions for formal and non-formal argument list

| No. | Feature |  |  |  | Example |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Invocation type | Variable assignment | Variable order | Number of variables |  | in Structured Text (ST) language - see 3.3 |
| 1 | formal | yes | any | any |  | A := LIMIT(EN:=COND, IN:=B, MX:=5, ENO=>TEMPL); |
| 2 a | non-formal | no | fixed | fixed |  | A := LIMIT(1, B, 5); |
| a Feature #2 is required for invocation of any of the standard functions defined in subclause 2.5.1.5 without formal names for one or more input variables, but Feature #1 shall be used if EN/ENO is necessary in function invocations. |  |  |  |  |  |  |
| NOTE 1 In the example given in feature #1, the MN variable will have the default value 0 (zero). NOTE 2 The example given in feature #2 is semantically equivalent to the following invocation with formal variable assignments (feature #1): A := LIMIT(EN := TRUE,MN := 1, IN := B, MX := 5); |  |  |  |  |  |  |

#### 2.5.1.2  Execution control

As shown in table 20, an additional Boolean EN (Enable) input or ENO (Enable Out) output, or both, can be provided by the manufacturer or user according to the declarations

VAR_INPUT   EN: BOOL := 1;  END_VAR
VAR_OUTPUT  ENO: BOOL;  END_VAR

When these variables are used, the execution of the operations defined by the function shall be controlled according to the following rules:

- If the value of EN is FALSE (0) when the function is invoked, the operations defined by the function body shall not be executed and the value of ENO shall be reset to FALSE (0) by the programmable controller system.

- Otherwise, the value of ENO shall be set to TRUE (1) by the programmable controller system, and the operations defined by the function body shall be executed.  These operations can include the assignment of a Boolean value to ENO.

- If any of the errors defined in table E.1 for subclauses of 2.5.1.5 occurs during the execution of one of the standard functions defined in 2.5.1.5, the ENO output of that function shall be reset to FALSE (0) by the programmable controller system, or the manufacturer shall specify other disposition of such an error according to the provisions of 1.5.1.

- If the ENO output is evaluated to FALSE (0), the values of all function outputs (VAR_OUTPUT, VAR_IN_OUT and function result) shall be considered to be implementation-dependent.

NOTE	It is a consequence of these rules that the ENO output of a function must be explicitly examined by the invoking entity if necessary to account for possible error conditions.

Table 20 - Use of EN input and ENO output

| No. | Feature | Examplea |
| --- | --- | --- |
| 1 | Use of EN and ENO Shown in LD (Ladder Diagram) language; see 4.2 | +-------+ ¦ ¦ ADD_EN ¦ + ¦ ADD_OK ¦ +---¦¦---¦EN ENO¦---( )---+ ¦ ¦ ¦ ¦ ¦ A---¦ ¦---C ¦ ¦ B---¦ ¦ ¦ +-------+ ¦ |
| 2 | Usage without EN and ENO Shown in FBD (Function Block Diagram) language; see 4.3 | +-----+ A---¦ + ¦---C B---¦ ¦ +-----+ |
| 3 | Usage with EN and without ENO Shown in FBD (Function Block Diagram) language; see 4.3 | +-----+ ADD_EN---¦EN ¦ A---¦ + ¦---C B---¦ ¦ +-----+ |
| 4 | Usage without EN and with ENO Shown in FBD (Function Block Diagram) language; see 4.3 | +-----+ ¦ ENO¦---ADD_OK A---¦ + ¦---C B---¦ ¦ +-----+ |
| a The graphical languages chosen for demonstrating the features above are only exemplary. Features, if chosen by a vendor, shall be in effect for all languages supported by the vendor (IL, ST, LD, FBD). |  |  |

#### 2.5.1.3  Declaration

Features for the textual and graphical declaration of functions are listed in Table 20a.

As illustrated in figure 6, the textual declaration of a function shall consist of the following elements:

- The keyword FUNCTION, followed by an identifier specifying the name of the function being declared, a colon (:), and the data type of the value to be returned by the function;

- A VAR_INPUT...END_VAR construct as defined in 2.4.3, specifying the names and types of the function's input variables;

- VAR_IN_OUT...END_VAR and VAR_OUTPUT...END_VAR constructs (see Annex F.11 for an example of the use of the latter construct) as defined in 2.4.3, if required, specifying the names and types of the function's in-out and output variables;

- A VAR...END_VAR construct, if required, specifying the names and types of the function's internal variables;

- A function body, written in one of the languages defined in this Part, or another programming language as defined in 1.4.3, which specifies the operations to be performed upon the variable(s) in order to assign values dependent on the function's semantics to a variable with the same name as the function, which represents the function result to be returned by the function (function result), as well as to in-out or output variables;

- The terminating keyword END_FUNCTION.

If the generic data types given in table 11 are used in the declaration of standard function variables, then the rules for inferring the actual types of the arguments of such functions shall be part of the function definition.

The variable initialization constructs defined in 2.4.3.2 can be used for the declaration of default values of function inputs and initial values of their internal and output variables.

The values of variables which are passed to the function via a VAR_IN_OUT construct can be modified from within the function.

As illustrated in figure 6, the graphic declaration of a  function shall consist of the following elements:

1) The bracketing keywords FUNCTION...END_FUNCTION or a graphical equivalent;

2) A graphic specification of the function name and the names, types and possibly initial values of the function's result and variables (input, output and in-out).

3) A specification of the names, types and possibly initial values of the internal variables used in the function, e.g., using the VAR...END_VAR construct;

4) A function body as defined above.

The maximum number of function specifications allowed in a particular resource is an implementation-dependent parameter.

Table 20a - Function  features

| No. | Description | Example |
| --- | --- | --- |
| 1 | In-out variable declaration (textual) | VAR_IN_OUT A: INT; END_VAR |
| 2 | In-out variable declaration (graphical) | See figure 6b |
| 3 | Graphical connection of in-out variable to different variables(graphical) | See figure 6d |

| a) | FUNCTION SIMPLE_FUN : REAL VAR_INPUT A,B : REAL ; (* External interface specification *) C : REAL := 1.0; END_VAR VAR_IN_OUT COUNT : INT ; END_VAR VAR COUNTP1 : INT ; END_VAR COUNTP1 := ADD(COUNT,1); (*Function body specification *) COUNT := COUNTP1 ; SIMPLE_FUN := A*B/C; END_FUNCTION |
| --- | --- |
|  | NOTE In the above example, the input variable is given a default value of 1.0, as specified in 2.4.3.2, to avoid a "division by zero" error if the input is not specified when the function is invoked, for example, if a graphical input to the function is left unconnected. |
| b) | FUNCTION +-------------+ (* External interface specification *) ¦ SIMPLE_FUN ¦ REAL----¦A ¦----REAL REAL----¦B ¦ REAL----¦C ¦ INT-----¦COUNT---COUNT¦----INT +-------------+ (* Function body specification *) +---+ ¦ADD¦--- +----+ COUNT--¦ ¦---COUNTP1--¦ := ¦---COUNT 1--¦ ¦ +----+ +---+ +---+ A---¦ * ¦ +---+ B---¦ ¦---¦ / ¦---SIMPLE_FUN +---+ ¦ ¦ C-----------¦ ¦ +---+ END_FUNCTION |
| c) | .... VAR X,Y,Z,RESULT : REAL: VAR COUNT1,COUNT2 : INT; ... RESULT := SIMPLE_FUN(A:=X,B:=Y,C:=Z,COUNT:=COUNT1); COUNT2 := COUNT1; ... |

Figure 6 - Examples of function declarations and usage

a) Textual declaration in ST language (subclause 3.3)
b) Graphical declaration in FBD language (subclause 4.3)
c) Usage of a function in ST language

| d) | +-------------+ ¦ SIMPLE_FUN ¦ X----¦A ¦----RESULT Y----¦B ¦ Z----¦C ¦ COUNT1---¦COUNT---COUNT¦----COUNT2 +-------------+ |
| --- | --- |
| NOTE The effect of this invocation of this function is identical to that in Figure 6(c). |  |

Figure 6 - Examples of function declarations and usage -cont'd.
d) Usage of a function in FBD language (subclause 4.3)

#### 2.5.1.4  Typing, overloading, and type conversion

A standard function, function block type, operator, or instruction is said to be overloaded when it can operate on input data elements of various types within a generic type designator as defined in 2.3.2.  For instance, an overloaded addition function on generic type ANY_NUM can operate on data of types LREAL, REAL, DINT, INT, and SINT.

When a programmable controller system supports an overloaded standard function, function block type, operator, or instruction, this standard function, function block type, operator, or instruction shall apply to all data types of the given generic type which are supported by that system.  For example, if a programmable controller system supports the overloaded function ADD and the data types SINT, INT, and REAL, then the system shall support the ADD function on inputs of type SINT, INT, and REAL.

When a function which normally represents an overloaded operator is to be typed, i.e., the types of its inputs and outputs restricted to a particular elementary or derived data type as defined in 2.3, this shall be done by appending an "underline" character followed by the required type, as shown in table 21.

| Table 21 - Typed and overloaded functions |  |  |
| --- | --- | --- |
| No. | Feature | Example |
| 1 | Overloaded functions | +-----+ ¦ ADD ¦ ANY_NUM-----¦ ¦----ANY_NUM ANY_NUM-----¦ ¦ . -----¦ ¦ . -----¦ ¦ ANY_NUM-----¦ ¦ +-----+ |
| 2 a | Typed functions | +---------+ ¦ ADD_INT ¦ INT-----¦ ¦----INT INT-----¦ ¦ . -----¦ ¦ . -----¦ ¦ INT-----¦ ¦ +---------+ |
| NOTE The overloading of non-standard functions or function block types is beyond the scope of this Standard. |  |  |
| a If feature 2 is supported, the manufacturer shall provide a table of which functions are overloaded and which are typed in the implementation. |  |  |

When the type of the result of a standard function defined in 2.5.1.5 is generic, then the actual types of all input variables of the same generic type shall be of the same type as the actual type of the function value in a given invocation of the function.  If necessary, the type conversion functions defined in 2.5.1.5.1 can be used to meet this requirement.  Examples of the application of this rule are given in figures 7 and 8.

| Type declaration (ST language - see 3.3) | Usage (FBD language - see 4.3) (ST language - see 3.3) |
| --- | --- |
| VAR A : INT ; B : INT ; C : INT ; END_VAR | +---+ A---¦ + ¦---C B---¦ ¦ +---+ C := A+B; |
| NOTE Type conversion is not required in the example shown above. |  |
| VAR A : INT ; B : REAL ; C : REAL; END_VAR | +-----------+ +---+ A---¦INT_TO_REAL¦---¦ + ¦---C +-----------+ ¦ ¦ B-------------------¦ ¦ +---+ C := INT_TO_REAL(A)+B; |
| VAR A : INT ; B : INT ; C : REAL; END_VAR | +---+ +-----------+ A----¦ + ¦---¦INT_TO_REAL¦---C B----¦ ¦ +-----------+ +---+ C := INT_TO_REAL(A+B); |

Figure 7 - Examples of explicit type conversion with overloaded functions

| Type declaration (ST language - see 3.3) | Usage (FBD language - see 4.3) (ST language - see 3.3) |
| --- | --- |
| VAR A : INT ; B : INT ; C : INT ; END_VAR | +---------+ A---¦ ADD_INT ¦---C B---¦ ¦ +---------+ C := ADD_INT(A,B); |
| NOTE Type conversion is not required in the example shown above. |  |
| VAR A : INT ; B : REAL ; C : REAL; END_VAR | +-----------+ +----------+ A---¦INT_TO_REAL¦---¦ ADD_REAL ¦---C +-----------+ ¦ ¦ B-------------------¦ ¦ +----------+ C := ADD_REAL(INT_TO_REAL(A),B); |
| VAR A : INT ; B : INT ; C : REAL; END_VAR | +---------+ +-----------+ A---¦ ADD_INT ¦---¦INT_TO_REAL¦---C ¦ ¦ +-----------+ B---¦ ¦ +---------+ C := INT_TO_REAL(ADD_INT(A,B)); |

Figure 8 - Examples of explicit type conversion with typed functions

#### 2.5.1.5  Standard functions

Definitions of functions common to all programmable controller programming languages are given in this subclause.  Where graphical representations of standard functions are shown in this subclause, equivalent textual declarations may be written as specified in 2.5.1.3.

A standard function specified in this subclause to be extensible is allowed to have two or more inputs to which the indicated operationis to be applied, e.g., extensible addition shall give as its output the sum of all its inputs.  The maximum number of inputs of an extensible function is an implementation-dependent parameter.   The actual number of inputs effective in a formal call of an extensible function is determined by the formal input name with the highest position in the sequence of parameter names.

EXAMPLE 1 The statement  
X := ADD(Y1,Y2,Y3); 
is equivalent to
 X := ADD(IN1 := Y1, IN2 := Y2, IN3 := Y3);

EXAMPLE 2 The following statements are equivalent:  
I := MUX_INT(K:=3,IN0 := 1, IN2 := 2, IN4 := 3); 
I := 0;

2.5.1.5.1  Type conversion functions

As shown in table 22, type conversion functions shall have the form *_TO_**, where "*" is the type of the input variable IN, and "**" the type of the output variable OUT, e.g., INT_TO_REAL.  The effects of type conversions on accuracy, and the types of errors that may arise during execution of type conversion operations, are implementation-dependent parameters.

| Table 22 - Type conversion function features |  |  |
| --- | --- | --- |
| No. | Graphical form | Usage example |
| 1a,b,e | +---------+ * ---¦ *_TO_** ¦--- ** +---------+ (*) - Input data type, e.g., INT (**) - Output data type, e.g., REAL (*_TO_**) - Function name, e.g., INT_TO_REAL | A := INT_TO_REAL(B) ; |
| 2c | +-------+ ANY_REAL---¦ TRUNC ¦---ANY_INT +-------+ | A := TRUNC(B) ; |
| 3d | +-------------+ *--¦ *_BCD_TO_** ¦---** +-------------+ | A := WORD_BCD_TO_INT(B); |
| 4d | +-------------+ **--¦ **_TO_BCD_* ¦---* +-------------+ | A := INT_TO_BCD_WORD(B); |
| NOTE Usage examples are given in the ST language defined in 3.3. |  |  |
| a A statement of conformance to feature 1 of this table shall include a list of the specific type conversions supported, and a statement of the effects of performing each conversion. b Conversion from type REAL or LREAL to SINT, INT, DINT or LINT shall round according to the convention of IEC 559, according to which, if the two nearest integers are equally near, the result shall be the nearest even integer, e.g.: REAL_TO_INT(1.6) is equivalent to 2 REAL_TO_INT(-1.6) is equivalent to -2 REAL_TO_INT(1.5) is equivalent to 2 REAL_TO_INT(-1.5) is equivalent to -2 REAL_TO_INT(1.4) is equivalent to 1 REAL_TO_INT(-1.4) is equivalent to -1 REAL_TO_INT(2.5) is equivalent to 2 REAL_TO_INT(-2.5) is equivalent to -2 |  |  |
| c The function TRUNC shall be used for truncation toward zero of a REAL or LREAL, yielding one of the integer types, for instance, TRUNC(1.6) is equivalent to 1 TRUNC(-1.6) is equivalent to -1 TRUNC(1.4) is equivalent to 1 TRUNC(-1.4) is equivalent to -1 d The conversion functions *_BCD_TO_** and **_TO_BCD_* shall perform conversions between variables of type BYTE, WORD, DWORD, and LWORD and variables of type USINT, UINT, UDINT and ULINT (represented by "*" and "**" respectively), when the corresponding bit-string variables contain data encoded in BCD format. For example, the value of USINT_TO_BCD_BYTE(25) would be 2#0010_0101, and the value of WORD_BCD_TO_UINT (2#0011_0110_1001) would be 369. e When an input or output of a type conversion function is of type STRING or WSTRING, the character string data shall conform to the external representation of the corresponding data, as specified in 2.2, in the character set defined in 2.1.1. |  |  |

##### 2.5.1.5.2  Numerical functions

The standard graphical representation, function names, input and output variable types, and function descriptions of functions of a single numeric variable shall be as defined in table 23.  These functions shall be overloaded on the defined generic types, and can be typed as defined in 2.5.1.4.  For these functions, the types of the input and output shall be the same.

The standard graphical representation, function names and symbols, and descriptions of arithmetic functions of two or more variables shall be as shown in table 24.  These functions shall be overloaded on all numeric types, and can be typed as defined in 2.5.1.4.

The accuracy of numerical functions shall be expressed in terms of one or more implementation-dependent parameters.

It is an error if the result of evaluation of one of these functions exceeds the range of values specified for the data type of the function output, or if division by zero is attempted.

Table 23 - Standard functions of one numeric variable

| Graphical form |  |  |  | Usage example |
| --- | --- | --- | --- | --- |
| +---------+ * ---¦ ** ¦--- * +---------+ (*) - Input/Output (I/O) type (**) - Function name |  |  |  | A := SIN(B) ; (ST language - see 3.3) |
| No. | Function name | I/O type | Description |  |
|  | General functions |  |  |  |
| 1 | ABS | ANY_NUM | Absolute value |  |
| 2 | SQRT | ANY_REAL | Square root |  |
|  | Logarithmic functions |  |  |  |
| 3 | LN | ANY_REAL | Natural logarithm |  |
| 4 | LOG | ANY_REAL | Logarithm base 10 |  |
| 5 | EXP | ANY_REAL | Natural exponential |  |
|  | Trigonometric functions |  |  |  |
| 6 | SIN | ANY_REAL | Sine of input in radians |  |
| 7 | COS | ANY_REAL | Cosine in radians |  |
| 8 | TAN | ANY_REAL | Tangent in radians |  |
| 9 | ASIN | ANY_REAL | Principal arc sine |  |
| 10 | ACOS | ANY_REAL | Principal arc cosine |  |
| 11 | ATAN | ANY_REAL | Principal arc tangent |  |

| Table 24 - Standard arithmetic functions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Graphical form |  |  |  | Usage example |
| +-----+ ANY_NUM ---¦ *** ¦--- ANY_NUM ANY_NUM ---¦ ¦ . ---¦ ¦ . ---¦ ¦ ANY_NUM ---¦ ¦ +-----+ (***) - Name or Symbol |  |  |  | A := ADD(B,C,D) ; or A := B+C+D ; |
| No. d,e | Name | Symbol | Description |  |
|  | Extensible arithmetic functions |  |  |  |
| 12 g | ADD | + | OUT := IN1 + IN2 + ... + INn |  |
| 13 | MUL | * | OUT := IN1 * IN2 * ... * INn |  |
|  | Non-extensible arithmetic functions |  |  |  |
| 14 g | SUB | - | OUT := IN1 - IN2 |  |
| 15 c | DIV | / | OUT := IN1 / IN2 |  |
| 16 a | MOD |  | OUT := IN1 modulo IN2 |  |
| 17 b | EXPT | ** | Exponentiation: OUT := IN1IN2 |  |
| 18 f | MOVE | := | OUT := IN |  |
| NOTE 1 Non-blank entries in the Symbol column are suitable for use as operators in textual languages, as shown in tables 52 and 55. NOTE 2 The notations IN1, IN2, ..., INn refer to the inputs in top-to-bottom order; OUT refers to the output. NOTE 3 Usage examples and descriptions are given in the ST language defined in Clause 3.3. |  |  |  |  |
| a IN1 and IN2 shall be of generic type ANY_INT for this function. The result of evaluating this function shall be the equivalent of executing the following statements in the ST language as defined in 3.3: IF (IN2 = 0) THEN OUT:=0 ; ELSE OUT:=IN1 - (IN1/IN2)*IN2 ; END_IF b IN1 shall be of type ANY_REAL, and IN2 of type ANY_NUM for this function. The output shall be of the same type as IN1. c The result of division of integers shall be an integer of the same type with truncation toward zero, for instance, 7/3 = 2 and (-7)/3 = -2. d When the named representation of a function is supported, this shall be indicated by the suffix "n" in the compliance statement. For example, "12n" represents the notation "ADD". e When the symbolic representation of a function is supported, this shall be indicated by the suffix "s" in the compliance statement. For example, "12s" represents the notation "+". f The MOVE function has exactly one input (IN) of type ANY and one output (OUT) of type ANY. g The generic type of the inputs and outputs of these functions is ANY_MAGNITUDE. |  |  |  |  |

##### 2.5.1.5.3  Bit string functions

The standard graphical representation, function names and descriptions of shift functions for a single bit-string variable shall be as defined in table 25.  These functions shall be overloaded on all bit-string types, and can be typed as defined in 2.5.1.4.

The standard graphical representation, function names and symbols, and descriptions of bitwise Boolean functions shall be as defined in table 26. These functions shall be extensible, except for NOT, and overloaded on all bit-string types, and can be typed as defined in 2.5.1.4.

Table 25 - Standard bit shift functions

| Graphical form |  |  | Usage example a |
| --- | --- | --- | --- |
| +-----+ ¦ *** ¦ ANY_BIT ---¦IN ¦--- ANY_BIT ANY_INT ---¦N ¦ +-----+ (***) - Function Name |  |  | A := SHL(IN:=B, N:=5) ; (ST language - see 3.3 ) |
| No. | Name | Description |  |
| 1 | SHL | OUT := IN left-shifted by N bits, zero-filled on right |  |
| 2 | SHR | OUT := IN right-shifted by N bits, zero-filled on left |  |
| 3 | ROR | OUT := IN right-rotated by N bits, circular |  |
| 4 | ROL | OUT := IN left-rotated by N bits, circular |  |
| NOTE The notation OUT refers to the function output. |  |  |  |
| a It shall be an error if the value of the N input is less than zero. |  |  |  |

##### 2.5.1.5.4  Selection and comparison functions

Selection and comparison functions shall be overloaded on all data types.  The standard graphical representations, function names and descriptions of selection functions shall be as shown in table 27.

The standard graphical representation, function names and symbols,and descriptions of comparison functions shall be as defined in table 28.  All comparison functions (except NE) shall be extensible.

Comparisons of bit string data shall be made bitwise from the most significant to the least significant bit, and shorter bit strings shall be considered to be filled on the left with zeros when compared to longer bit strings; that is, comparison of bit string variables shall have the same result as comparison of unsigned integer variables.

Table 26 - Standard bitwise Boolean functions

| Graphical form |  |  |  | Usage examples |
| --- | --- | --- | --- | --- |
| +-----+ ANY_BIT ---¦ *** ¦--- ANY_BIT ANY_BIT ---¦ ¦ : ---¦ ¦ : ---¦ ¦ ANY_BIT ---¦ ¦ +-----+ (***) - Name or symbol |  |  |  | A := AND(B,C,D) ; or A := B & C & D ; |
| No. a,b | Name | Symbol | Description |  |
| 5 | AND | & (NOTE 1) | OUT := IN1 & IN2 & ... & INn |  |
| 6 | OR | >=1 (NOTE 2) | OUT := IN1 OR IN2 OR ... OR INn |  |
| 7 | XOR | =2k+1 (NOTE 2) | OUT := IN1 XOR IN2 XOR ... XOR INn |  |
| 8 | NOT |  | OUT := NOT IN1 (NOTE 4) |  |
| NOTE 1 This symbol is suitable for use as an operator in textual languages, as shown in tables 52 and 55. NOTE 2 This symbol is not suitable for use as an operator in textual languages. NOTE 3 The notations IN1, IN2, ..., INn refer to the inputs in top-to-bottom order; OUT refers to the output. NOTE 4 Graphic negation of signals of type BOOL can also be accomplished as shown in table 19. NOTE 5 Usage examples and descriptions are given in the ST language defined in 3.3. |  |  |  |  |
| a When the named representation of a function is supported, this shall be indicated by the suffix "n" in the compliance statement. For example, "5n" represents the notation "AND". b When the symbolic representation of a function is supported, this shall be indicated by the suffix "s" in the compliance statement. For example, "5s" represents the notation "&". |  |  |  |  |

Table 27 - Standard selection functionsd

| No. | Graphical form | Explanation/example |
| --- | --- | --- |
| 1 | +-----+ ¦ SEL ¦ BOOL--¦G ¦--ANY ANY---¦IN0 ¦ ANY---¦IN1 ¦ +-----+ | Binary selectionc: OUT := IN0 if G = 0 OUT := IN1 if G = 1 EXAMPLE: A := SEL(G:=0,IN0:=X,IN1:=5) ; |
| 2a | +-----+ ¦ MAX ¦ ANY_ELEMENTARY--¦ ¦--ANY_ELEMENTARY : ---¦ ¦ ANY_ELEMENTARY--¦ ¦ +-----+ | Extensible maximum function: OUT := MAX (IN1,IN2, ...,INn) EXAMPLE: A := MAX(B,C,D) ; |
| 2b | +-----+ ¦ MIN ¦ ANY_ELEMENTARY--¦ ¦--ANY_ELEMENTARY : ---¦ ¦ ANY_ELEMENTARY--¦ ¦ +-----+ | Extensible minimum function: OUT := MIN (IN1,IN2, ...,INn) EXAMPLE: A := MIN(B,C,D) ; |
| 3 | +-------+ ¦ LIMIT ¦ ANY_ELEMENTARY--¦MN ¦--ANY_ELEMENTARY ANY_ELEMENTARY--¦IN ¦ ANY_ELEMENTARY--¦MX ¦ +-------+ | Limiter: OUT := MIN(MAX(IN,MN),MX) EXAMPLE: A := LIMIT(IN:=B,MN:=0,MX:=5); |
| 4e | +-----+ ¦ MUX ¦ ANY_INT--¦K ¦----ANY ANY------¦ ¦ : ------¦ ¦ ANY------¦ ¦ +-----+ | Extensible multiplexer a, b, c: Select one of N inputs depending on input K EXAMPLE: A := MUX(0, B, C, D); would have the same effect as A := B ; |
| NOTE 1 The notations IN1, IN2, ..., INn refer to the inputs in top-to-bottom order; OUT refers to the output. NOTE 2 Usage examples and descriptions are given in the ST language defined in 3.3. |  |  |
| a The unnamed inputs in the MUX function shall have the default names IN0, IN1,...,INn-1 in top-to-bottom order, where n is the total number of these inputs. These names may, but need not, be shown in the graphical representation. b The MUX function can be typed as defined in 2.5.1.4 in the form MUX_*_**, where * is the type of the K input and ** is the type of the other inputs and the output. c It is allowed, but not required, that the manufacturer support selection among variables of derived data types, as defined in 2.3.3, in order to claim compliance to this feature. d It is an error if the inputs and the outputs to one of these functions are not all of the same actual data type, with the exception of the G input of the SEL function and the K input of the MUX function. e It is an error if the actual value of the K input of the MUX function is not within the range {0..n-1}. |  |  |

Table 28 - Standard comparison functions

| Graphical form |  |  |  | Usage examples |
| --- | --- | --- | --- | --- |
| +-----+ ANY_ELEMENTARY --¦ *** ¦--- BOOL : --¦ ¦ ANY_ELEMENTARY --¦ ¦ +-----+ (***) - Name or Symbol |  |  |  | A := GT(B,C,D) ; or A := (B>C) & (C>D) ; |
| No. | Name a | Symbol b | Description |  |
| 5 | GT | > | Decreasing sequence: OUT := (IN1>IN2) & (IN2>IN3) & ... & (INn-1 > INn) |  |
| 6 | GE | >= | Monotonic sequence: OUT := (IN1>=IN2)&(IN2>=IN3)& ... & (INn-1 >= INn) |  |
| 7 | EQ | = | Equality: OUT := (IN1=IN2) & (IN2=IN3) & ... & (INn-1 = INn) |  |
| 8 | LE | <= | Monotonic sequence: OUT := (IN1<=IN2)&(IN2<=IN3)& ... & (INn-1 <= INn) |  |
| 9 | LT | < | Increasing sequence: OUT := (IN1<IN2) & (IN2<IN3) & ... & (INn-1 < INn) |  |
| 10 | NE | <> | Inequality (non-extensible) OUT := (IN1 <> IN2) |  |
| NOTE 1 The notations IN1, IN2, ..., INn refer to the inputs in top-to-bottom order; OUT refers to the output. NOTE 2 All the symbols shown in this table are suitable for use as operators in textual languages, as shown in tables 52 and 55. NOTE 3 Usage examples and descriptions are given in the ST language defined in 3.3. |  |  |  |  |
| a When the named representation of a function is supported, this shall be indicated by the suffix "n" in the compliance statement. For example, "5n" represents the notation "GT". b When the symbolic representation of a function is supported, this shall be indicated by the suffix "s" in the compliance statement. For example, "5s" represents the notation ">". |  |  |  |  |

##### 2.5.1.5.5  Character string functions

All the functions defined in 2.5.1.5.4 shall be applicable to character strings.  For the purposes of comparison of two strings of unequal length, the shorter string shall be considered to be extended on the right to the length of the longer string by characters with the value zero.  Comparison shall proceed from left to right, based on the numeric value of the character codes in the character set defined in 2.1.1 .  For example, the character string 'Z' shall be greater than the character string 'AZ', and 'AZ' shall be greater than 'ABC'.

The standard graphical representations, function names and descriptions of additional functions of character strings shall be as shown in table 29.  For the purpose of these operations, character positions within the string shall be considered to be numbered 1,2,...,L, beginning with the leftmost character position, where L is the length of the string.

It shall be an error if:

-	the actual value of any input designated as ANY_INT in Table 29 is less than zero;

-	evaluation of the function results in an attempt to (1) access a non-existent character position in a string, or (2) produce a string longer than the implementation-dependent maximum string length.

| Table 29 - Standard character string functions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Graphical form a |  |  |  | Explanation/example |
| 1 | +-----+ ANY_STRING--¦ LEN ¦--ANY_INT +-----+ |  |  |  | String length function Example: A := LEN('ASTRING'); is equivalent to A := 7; |
| 2 | +------+ ¦ LEFT ¦ ANY_STRING--¦IN ¦--ANY_STRING ANY_INT-----¦L ¦ +------+ |  |  |  | Leftmost L characters of IN Example: A := LEFT(IN:='ASTR',L:=3); is equivalent to A := 'AST' ; |
| 3 | +-------+ ¦ RIGHT ¦ ANY_STRING--¦IN ¦--ANY_STRING ANY_INT-----¦L ¦ +-------+ |  |  |  | Rightmost L characters of IN Example: A := RIGHT(IN:='ASTR',L:=3); is equivalent to A := 'STR' ; |
| 4 | +-------+ ¦ MID ¦ ANY_STRING--¦IN ¦--ANY_STRING ANY_INT-----¦L ¦ ANY_INT-----¦P ¦ +-------+ |  |  |  | L characters of IN, beginning at the P-th Example: A := MID(IN:='ASTR',L:=2,P:=2); is equivalent to A := 'ST' ; |
| 5 | +--------+ ¦ CONCAT ¦ ANY_STRING---¦ ¦--ANY_STRING : ---¦ ¦ ANY_STRING---¦ ¦ +--------+ |  |  |  | Extensible concatenation Example: A := CONCAT('AB','CD','E') ; is equivalent to A := 'ABCDE' ; |
| 6 | +--------+ ¦ INSERT ¦ ANY_STRING--¦IN1 ¦--ANY_STRING ANY_STRING--¦IN2 ¦ ANY_INT-----¦P ¦ +--------+ |  |  | Insert IN2 into IN1 after the P-th character position Example: A:=INSERT(IN1:='ABC',IN2:='XY',P=2); is equivalent to A := 'ABXYC' ; |  |
| 7 | +--------+ ¦ DELETE ¦ ANY_STRING--¦IN ¦--ANY_STRING ANY_INT-----¦L ¦ ANY_INT-----¦P ¦ +--------+ |  | Delete L characters of IN, beginning at the P-th character position Example: A := DELETE(IN:='ABXYC',L:=2, P:=3) ; is equivalent to A := 'ABC' ; |  |  |
| 8 | +---------+ ¦ REPLACE ¦ ANY_STRING--¦IN1 ¦--ANY_STRING ANY_STRING--¦IN2 ¦ ANY_INT-----¦L ¦ ANY_INT-----¦P ¦ +---------+ |  |  |  | Replace L characters of IN1 by IN2, starting at the P-th character position Example: A := REPLACE(IN1:='ABCDE',IN2:='X', L:=2, P:=3) ; is equivalent to A := 'ABXE' ; |
| 9 | +--------+ ¦ FIND ¦ ANY_STRING--¦IN1 ¦--ANY_INT ANY_STRING--¦IN2 ¦ +--------+ | Find the character position of the beginning of the first occurrence of IN2 in IN1. If no occurrence of IN2 is found, then OUT := 0. Example: A := FIND(IN1:='ABCBC',IN2:='BC') ; is equivalent to A := 2 ; |  |  |  |
| NOTE The examples in this table are given in the Structured Text (ST) language defined in 3.3. |  |  |  |  |  |

##### 2.5.1.5.6  Functions of time data types

In addition to the comparison and selection functions defined in 2.5.1.5.4, the combinations of input and output time data types shown in table 30 shall be allowed with the associated functions.

It shall be an error if the result of evaluating one of these functions exceeds the implementation-dependent range of values for the output data type.

| Table 30 - Functions of time data types |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Numeric and concatenation functions |  |  |  |  |  |  |  |
| No. | Name | Symbol |  | IN1 | IN2 |  | OUT |  |
| 1ac,d | ADD | + |  | TIME |  | TIME |  | TIME |
| 1bc,d | ADD_TIME | + |  | TIME |  | TIME |  | TIME |
| 2a | ADD b | + b |  | TIME_OF_DAY | TIME |  | TIME_OF_DAY |  |
| 2b | ADD_TOD_TIME | + b |  | TIME_OF_DAY | TIME |  | TIME_OF_DAY |  |
| 3a | ADD b | + b |  | DATE_AND_TIME | TIME |  | DATE_AND_TIME |  |
| 3b | ADD_DT_TIME | + b |  | DATE_AND_TIME | TIME |  | DATE_AND_TIME |  |
| 4ac,d | SUB | - |  | TIME | TIME |  | TIME |  |
| 4bc,d | SUB_TIME | - |  | TIME | TIME |  | TIME |  |
| 5a | SUB b | - b |  | DATE | DATE |  | TIME |  |
| 5b | SUB_DATE_DATE | - b |  | DATE | DATE |  | TIME |  |
| 6a | SUB b | - b |  | TIME_OF_DAY | TIME |  | TIME_OF_DAY |  |
| 6b | SUB_TOD_TIME | - b |  | TIME_OF_DAY | TIME |  | TIME_OF_DAY |  |
| 7a | SUB b | - b |  | TIME_OF_DAY | TIME_OF_DAY |  | TIME |  |
| 7b | SUB_TOD_TOD | - b |  | TIME_OF_DAY | TIME_OF_DAY |  | TIME |  |
| 8a | SUB b | - b |  | DATE_AND_TIME | TIME |  | DATE_AND_TIME |  |
| 8b | SUB_DT_TIME | - b |  | DATE_AND_TIME | TIME |  | DATE_AND_TIME |  |
| 9a | SUB b | - b |  | DATE_AND_TIME | DATE_AND_TIME |  | TIME |  |
| 9b | SUB_DT_DT | - b |  | DATE_AND_TIME | DATE_AND_TIME |  | TIME |  |
| 10a | MUL b | * b |  | TIME | ANY_NUM |  | TIME |  |
| 10b | MULTIME | * b |  | TIME | ANY_NUM |  | TIME |  |
| 11a | DIV b | / b |  | TIME | ANY_NUM |  | TIME |  |
| 11b | DIVTIME | / b |  | TIME | ANY_NUM |  | TIME |  |
| 12 | CONCAT_DATE_TOD |  |  | DATE | TIME_OF_DAY |  | DATE_AND_TIME |  |
|  | Type conversion functions |  |  |  |  |  |  |  |
| 13a 14a | DT_TO_TOD DT_TO_DATE |  |  |  |  |  |  |  |
| NOTE 1 Non-blank entries in the Symbol column are suitable for use as operators in textual languages, as shown in tables 52 and 55. NOTE 2 The notations IN1, IN2, ..., INn refer to the inputs in top-to-bottom order; OUT refers to the output. NOTE 3 It is possible to type the functions MULTIME and DIVTIME, e.g., the operands of MULTIME_REAL would be of type TIME and REAL, respectively. NOTE 4 The effects of conversion between time data types and types STRING and WSTRING are defined in footnote (e) of Table 22. NOTE 5 The effects of type conversions between time data types and other data types not defined in this Table are implementation-dependent. |  |  |  |  |  |  |  |  |
| a The type conversion functions shall have the effect of "extracting" the appropriate data, e.g., the ST language statements X := DT#1986-04-28-08:40:00 ; Y := DT_TO_TOD(X) ; W := DT_TO_DATE(X); shall have the same result as the statements X := DT#1986-04-28-08:40:00 ; W := DATE#1986-04-28 ; Y := TIME_OF_DAY#08:40:00;. b This usage is deprecated and will not be included in future editions of this Standard. c When the named representation of a function is supported, this shall be indicated by the suffix "n" in the compliance statement. For example, "1n" represents the notation "ADD". d When the symbolic representation of a function is supported, this shall be indicated by the suffix "s" in the compliance statement. For example, "1s" represents the notation "+". |  |  |  |  |  |  |  |  |

##### 2.5.1.5.7  Functions of enumerated data types

The selection and comparison functions listed in table 31 can be applied to inputs which are of an enumerated data type as defined in 2.3.3.1.

Table 31 - Functions of enumerated data types

| No. | Name | Symbol | Feature No. in Tables 27 and 28 |
| --- | --- | --- | --- |
| 1 | SEL |  | 1 |
| 2 | MUX |  | 4 |
| 3a | EQ | = | 7 |
| 4a | NE | <> | 10 |
| NOTE - The provisions of NOTES 1-2 of table 28 apply to this table. |  |  |  |
| a The provisions of footnotes (a) and (b) of Table 28 apply to this feature. |  |  |  |

### 2.5.2  Function blocks

For the purposes of programmable controller programming languages, a function block is a program organization unit which, when executed, yields one or more values.  Multiple, named instances (copies) of a function block can be created.  Each instance shall have an associated identifier (the instance name), and a data structure containing its output and internal variables, and, depending on the implementation, values of or references to its input variables.  All the values of the output variables and the necessary internal variables of this data structure shall persist from one execution of the function block to the next; therefore, invocation of a function block with the same arguments (input variables) need not always yield the same output values.

Only the input and output variables shall be accessible outside of an instance of a function block, i.e., the function block's internal variables shall be hidden from the user of the function block.

Execution of the operations of a function block shall be invoked as defined in clause 3 for textual languages, according to the rules of network evaluation given in clause 4 for graphic languages, or under the control of sequential function chart (SFC) elements as defined in 2.6.

Any function block type which has already been declared can be used in the declaration of another function block type or program type as shown in figure 3.

The scope of an instance of a function block shall be local to the program organization unit in which it is instantiated, unless it is declared to be global in a VAR_GLOBAL block as defined in 2.7.1.

As illustrated in 2.5.2.2, the instance name of a function block instance can be used as the input to a function or function block if declared as an input variable in a VAR_INPUT declaration, or as an input/output variable of a function block in a VAR_IN_OUT declaration, as defined in 2.4.3.

The maximum number of function block types and instantiations for a given resource are implementation-dependent parameters.

#### 2.5.2.1  Representation

As illustrated in figure 9, an instance of a function block can be created textually, by declaring a data element using the declared function block type in a VAR...END_VAR construct, identically to the use of a structured data type, as defined in 2.4.3.

As further illustrated in figure 9, an instance of a function block can be created graphically, by using a graphic representation of the function block, with the function block type name inside the block, and the instance name above the block, following the rules for representation of functions given in 2.5.1.1.

As shown in figure 9, input and output variables of an instance of a function block can be represented as elements of structured data types as defined in 2.3.6.1.

If either of the two graphical negation features defined in table 19 is supported for function blocks, it shall also be supported for functions as defined in 2.5.1, and vice versa.

| Graphical (FBD language) | Textual (ST language) |
| --- | --- |
| FF75 +------+ ¦ SR ¦ %IX1---¦S1 Q1¦---%QX3 %IX2---¦R ¦ +------+ | VAR FF75: SR; END_VAR (* Declaration *) FF75(S1:=%IX1, R:=%IX2); (* Invocation *) %QX3 := FF75.Q1 ; (* Assign Output *) |
| MyTon +-------+ +----+ ¦ TON ¦ a--¦ NE ¦---O¦EN ENO¦-- b--¦ ¦ r--¦IN Q¦O-out +----+ --¦PT ET¦-- +-------+ | VAR a,b,r,out : BOOL; MyTon : TON; END_VAR MyTon(EN := NOT (a <> b), IN := r, NOT Q => out); |

Figure 9 - Function block instantiation examples

Assignment of a value to an output variable of a function block is not allowed except from within the function block.  The assignment of a value to the input of a function block is permitted only as part of the invocation of the function block.  Allowable usages of function block inputs and outputs are summarized in table 32, using the function block FF75 of type SR shown in figure 9.  The examples are shown in the ST language.

Table 32 - Examples of function block I/O variable usage

| Usage | Inside function block | Outside function block |
| --- | --- | --- |
| Input read | IF IN1 THEN ... | Not allowed (Notes 1 and 2) |
| Input assignment | Not allowed (Note 1) | FB_INST(IN1:=A,IN2:=B); |
| Output read | OUT := OUT AND NOT IN2; | C := FB_INST.OUT; |
| Output assignment | OUT := 1; | Not Allowed (Note 1) |
| In-out read | IF INOUT THEN ... | IF FB1.INOUT THEN... |
| In-out assignment | INOUT := OUT OR IN1; (Note 3) | FB_INST(INOUT:=D); |
| NOTE 1 Those usages listed as "not allowed" in this table could lead to implementation-dependent, unpredictable side effects. NOTE 2 Reading and writing of input, output and internal variables of a function block may be performed by the "communication function", "operator interface function", or the "programming, testing, and monitoring functions" defined in IEC 61131-1. NOTE 3 As illustrated in 2.5.2.2, modification within the function block of a variable declared in a VAR_IN_OUT block is permitted. |  |  |

#### 2.5.2.1a  Use of EN and ENO in function blocks

As shown in table 20 for functions, for function blocks also an additional Boolean EN (Enable) input or ENO (Enable Out)output, or both, can be provided by the manufacturer or user according to the declarations

VAR_INPUT   EN: BOOL := 1;  END_VAR
VAR_OUTPUT  ENO: BOOL;  END_VAR

When these variables are used, the execution of the operations defined by the function block shall be controlled according to the following rules:

1)	If the value of EN is FALSE (0) when the function block instance is invoked, the assignments of actual values to the function block inputs may or may not be made in an implementation-dependent fashion, the operations defined by the function block body shall not be executed and the value of ENO shall be reset to FALSE (0) by the programmable controller system.

2)	Otherwise, the value of ENO shall be set to TRUE (1) by the programmable controller system, the assignments of actual values to the function block inputs shall be made and the operations defined by the function block body shall be executed.  These operations can include the assignment of a Boolean value to ENO.

3)	If the ENO output is evaluated to FALSE (0), the values of the function block outputs (VAR_OUTPUT) keep their states from the previous invocation.

NOTE	It is a consequence of these rules that the ENO output of a function block must be explicitly examined by the invoking entity if necessary to account for possible error conditions.

EXAMPLES - The figures below illustrate the use of EN and ENO in association with the standard TP, TON and TOF blocks (represented by T**)  defined in subclause 2.5.2.3.4, and the CTU and CTD blocks (represented by CT*) defined in subclause 2.5.2.3.3.  In accordance with the above rules, a FALSE value of the EN input may be used to "freeze" the operation of the associated function block; that is, the output values do not change irrespective of changes in any of the other input values. When the EN input value becomes TRUE, normal operation of the function block may resume. The value of the ENO output is FALSE after each evaluation of the function block for which the EN input is FALSE. When EN is TRUE, a TRUE value of ENO reflects a normal evaluation of the block, and a FALSE value of ENO may be used to indicate an implementation-dependent error condition.

+-------+                      +-------+
          |  T**  |                      |  CT*  |
   BOOL---|EN  ENO|---BOOL        BOOL---|EN  ENO|---BOOL
   BOOL---|IN    Q|---BOOL        BOOL--->CU    Q|---BOOL
   TIME---|PT   ET|---TIME        BOOL---|R    CV|---INT
          +-------+                INT---|PV     |
                                         +-------+

#### 2.5.2.2  Declaration

As illustrated in figure 10, a function block shall be declared textually or graphically in the same manner as defined for functions in 2.5.1.3, with the differences described below and summarized in table 33:

- The delimiting keywords for declaration of function blocks shall be FUNCTION_BLOCK...END_FUNCTION_BLOCK.

- The RETAIN qualifier defined in 2.4.3 can be used for internal and output variables of a function block, as shown in features 1, 2, and 3 in table 33.

- The values of variables which are passed to the function block via a VAR_EXTERNAL construct can be modified from within the function block, as shown in feature 10 of table 33.

- The output values of a function block instance whose name is passed into the function block via a VAR_INPUT, VAR_IN_OUT, or VAR_EXTERNAL construct can be accessed, but not modified, from within the function block, as shown in features 5, 6, and 7 of table 33.

- A function block whose instance name is passed into the function block via a VAR_IN_OUT or VAR_EXTERNAL construction can be invoked from inside the function block, as shown in features 6 and 7 of table 33.

- In textual declarations, the R_EDGE and F_EDGE qualifiers can be used to indicate an edge-detection function on Boolean inputs.  This shall cause the implicit declaration of a function block of type R_TRIG or F_TRIG, respectively, as defined in 2.5.2.3.2, to perform the required edge detection.  For an example of this construction, see features 8a and 8b of table 33 and the accompanying NOTE.

- The construction illustrated in table 33, features 9a and 9b shall be used in graphical declarations for rising and falling edge detection.  When the character set defined in 2.1.1 is used, the "greater than" (>) or "less than" (<) character shall be in line with the edge of the function block.  When graphic or semigraphic representations are employed, the notation of IEC 617, Part 12 for dynamic inputs shall be used.

- If the generic data types given in table 11 are used in the declaration of standard function block inputs and outputs, then the rules for inferring the actual types of the outputs of such function block types shall be part of the function block type definition.  In textual invocations of such function blocks assignments of the outputs to variables shall be made directly in the invocation statement (using the operator ‘=>‘).

- The asterisk notation (feature No. 10 in table 15) can be used in the declaration of internal variables of a function block.

- EN/ENO inputs and outputs shall be declared and used as described in 2.5.1.2.

- It shall be an error if no value is specified for: (i) an in-out variable of a function block instance; (ii) a function block instance used as an input variable of another function block instance.

As illustrated in figure 12, only variables or function block instance names can be passed into a function block via the VAR_IN_OUT construct, i.e., function or function block outputs cannot be passed via this construction.  This is to prevent the inadvertent modifications of such outputs.  However, "cascading" of VAR_IN_OUT constructions is permitted, as illustrated in figure 12c.

(* a) Textual declaration in ST language (see 3.3) *) FUNCTION_BLOCK DEBOUNCE (*** External Interface ***) VAR_INPUT IN : BOOL ; (* Default = 0 *) DB_TIME : TIME := t#10ms ; (* Default = t#10ms *) END_VAR VAR_OUTPUT OUT : BOOL ; (* Default = 0 *) ET_OFF : TIME ; (* Default = t#0s *) END_VAR VAR DB_ON : TON ; (** Internal Variables **) DB_OFF : TON ; (** and FB Instances **) DB_FF : SR ; END_VAR (** Function Block Body **) DB_ON(IN := IN, PT := DB_TIME) ; DB_OFF(IN := NOT IN, PT:=DB_TIME) ; DB_FF(S1 :=DB_ON.Q, R := DB_OFF.Q) ; OUT := DB_FF.Q ; ET_OFF := DB_OFF.ET ; END_FUNCTION_BLOCK

(* b) Graphical declaration in FBD language (see 4.3) *) FUNCTION_BLOCK (** External Interface **) +---------------+ ¦ DEBOUNCE ¦ BOOL---¦IN OUT¦---BOOL TIME---¦DB_TIME ET_OFF¦---TIME +---------------+ (** Function Block Body **) DB_ON DB_FF +-----+ +----+ ¦ TON ¦ ¦ SR ¦ IN----+------¦IN Q¦-----¦S1 Q¦---OUT ¦ +---¦PT ET¦ +--¦R ¦ ¦ ¦ +-----+ ¦ +----+ ¦ ¦ ¦ ¦ ¦ DB_OFF ¦ ¦ ¦ +-----+ ¦ ¦ ¦ ¦ TON ¦ ¦ +--¦--O¦IN Q¦--+ DB_TIME--+---¦PT ET¦--------------ET_OFF +-----+ END_FUNCTION_BLOCK

Figure 10 - Examples of function block declarations

| Table 33 - Function block declaration and usage features |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. | Description |  | Example |  |  |  |
| 1a | RETAIN qualifier on internal variables |  | VAR RETAIN X : REAL ; END_VAR |  |  |  |
| 1b | NON_RETAIN qualifier on internal variables |  | VAR NON_RETAIN X : REAL ; END_VAR |  |  |  |
| 2a | RETAIN qualifier on output variables |  | VAR_OUTPUT RETAIN X : REAL ; END_VAR |  |  |  |
| 2b | RETAIN qualifier on input variables |  | VAR_INPUT RETAIN X : REAL ; END_VAR |  |  |  |
| 2c | RETAIN qualifier on output variables |  | VAR_OUTPUT NON_RETAIN X : REAL ; END_VAR |  |  |  |
| 2d | RETAIN qualifier on input variables |  | VAR_INPUT NON_RETAIN X : REAL ; END_VAR |  |  |  |
| 3a | RETAIN qualifier on internal function blocks |  |  |  | VAR RETAIN TMR1: TON ; END_VAR |  |
| 3b | NON_RETAIN qualifier on internal function blocks |  |  |  | VAR NON_RETAIN TMR1: TON ; END_VAR |  |
| 4a | VAR_IN_OUT declaration (textual) |  | VAR_IN_OUT A: INT ; END_VAR |  |  |  |
| 4b | VAR_IN_OUT declaration and usage(graphical) |  |  |  |  | See figure 12 |
| 4c | VAR_IN_OUT declaration with assignment to different variables (graphical) |  |  |  |  | See figure 12d |
| 5a | Function block instance name as input (textual) |  |  | VAR_INPUT I_TMR: TON ; END_VAR EXPIRED := I_TMR.Q; (* Note 1 *) |  |  |
| 5b | Function block instance name as input (graphical) |  |  |  |  | See figure 11a |
| 6a | Function block instance name as VAR_IN_OUT (textual) |  |  | VAR_IN_OUT IO_TMR: TOF ; END_VAR IO_TMR(IN:=A_VAR, PT:=T#10S); EXPIRED := IO_TMR.Q; (* Note 1 *) |  |  |
| 6b | Function block instance name as VAR_IN_OUT (graphical) |  |  |  |  | See figure 11b |
| 7a | Function block instance name as external variable (textual) |  |  | VAR_EXTERNAL EX_TMR : TOF ;END_VAR EX_TMR(IN:=A_VAR, PT:=T#10S); EXPIRED := EX_TMR.Q; (* Note 1 *) |  |  |
| 7b | Function block instance name as external variable (graphical) |  |  |  |  | See figure 11c |
| 8a 8b | Textual declaration of: rising edge inputs falling edge inputs | FUNCTION_BLOCK AND_EDGE (* Note 2 *) VAR_INPUT X : BOOL R_EDGE; Y : BOOL F_EDGE; END_VAR VAR_OUTPUT Z : BOOL ; END_VAR Z := X AND Y ; (* ST language example *) END_FUNCTION_BLOCK (*- see 3.3 *) |  |  |  |  |
| 9a 9b | Graphical declaration of: rising edge inputs falling edge inputs | FUNCTION_BLOCK (* Note 2 *) +-----------+ (* External interface *) ¦ AND_EDGE ¦ BOOL---->X Z¦---BOOL ¦ ¦ BOOL----<Y ¦ ¦ ¦ +-----------+ +---+ (* Function block body *) X---¦ & ¦---Z (* FBD language example *) Y---¦ ¦ (* - see 4.3 *) +---+ END_FUNCTION_BLOCK |  |  |  |  |
| 10a | VAR_EXTERNAL declarations within function block type declarations |  |  |  |  |  |
| 10b | VAR_EXTERNAL CONSTANT declarations within function block type declarations |  |  |  |  |  |
| 11 | VAR_TEMP declarations (see 2.4.3) within function block type declarations |  |  |  |  |  |
| NOTE 1 It is assumed in these examples that the variables EXPIRED and A_VAR have been declared of type BOOL. NOTE 2 The declaration of function block AND_EDGE in the above examples is equivalent to: FUNCTION_BLOCK AND_EDGE VAR INPUT X : BOOL; Y : BOOL; END_VAR VAR X_TRIG : R_TRIG ; Y_TRIG : F_TRIG ; END_VAR X_TRIG(CLK := X) ; Y_TRIG(CLK := Y) ; Z := X_TRIG.Q AND Y_TRIG.Q; END_FUNCTION_BLOCK See 2.5.2.3.2 for the definition of the edge detection function blocks R_TRIG and F_TRIG. |  |  |  |  |  |  |

FUNCTION_BLOCK +--------------+ (* External interface *) ¦ INSIDE_A ¦ TON---¦I_TMR EXPIRED¦---BOOL +--------------+ +------+ (* Function Block body *) ¦ MOVE ¦ I_TMR.Q---¦ ¦---EXPIRED +------+ END_FUNCTION_BLOCK

FUNCTION_BLOCK +--------------+ (* External interface *) ¦ EXAMPLE_A ¦ BOOL---¦GO DONE¦---BOOL +--------------+ E_TMR (* Function Block body *) +-----+ I_BLK ¦ TON ¦ +--------------+ GO---¦IN Q¦ ¦ INSIDE_A ¦ t#100ms---¦PT ET¦ E_TMR---¦I_TMR EXPIRED¦---DONE +-----+ +--------------+ END_FUNCTION_BLOCK

Figure 11a - Graphical use of a function block name as an input variable

(table 33, feature 5b)

NOTE	I_TMR is not represented graphically in this figure since this would imply invocation of I_TMR within INSIDE_A, which is forbidden by rules 4) and 5) of 2.5.2.2.  See also Feature No. 5a of Table 33.

FUNCTION_BLOCK +--------------+ (* External interface *) ¦ INSIDE_B ¦ TON---¦I_TMR----I_TMR¦---TON BOOL--¦TMR_GO EXPIRED¦---BOOL +--------------+ I_TMR (* Function Block body *) +-----+ ¦ TON ¦ TMR_GO--¦IN Q¦---EXPIRED ¦PT ET¦ +-----+ END_FUNCTION_BLOCK

FUNCTION_BLOCK +--------------+ (* External interface *) ¦ EXAMPLE_B ¦ BOOL---¦GO DONE¦---BOOL +--------------+ E_TMR (* Function Block body *) +-----+ I_BLK ¦ TON ¦ +---------------+ ¦IN Q¦ ¦ INSIDE_B ¦ t#100ms---¦PT ET¦ E_TMR---¦I_TMR-----I_TMR¦ +-----+ GO------¦TMR_GO EXPIRED¦---DONE +---------------+ END_FUNCTION_BLOCK

Figure 11b - Graphical use of a function block name as an in-out variable

(table 33, feature 6b)

FUNCTION_BLOCK +--------------+ (* External interface *) ¦ INSIDE_C ¦ BOOL--¦TMR_GO EXPIRED¦---BOOL +--------------+ VAR_EXTERNAL X_TMR : TON ; END_VAR X_TMR (* Function Block body *) +-----+ ¦ TON ¦ TMR_GO---¦IN Q¦---EXPIRED ¦PT ET¦ +-----+ END_FUNCTION_BLOCK

PROGRAM +--------------+ (* External interface *) ¦ EXAMPLE_C ¦ BOOL---¦GO DONE¦---BOOL +--------------+ VAR_GLOBAL X_TMR : TON ; END_VAR I_BLK (* Program body *) +---------------+ ¦ INSIDE_C ¦ GO------¦TMR_GO EXPIRED¦---DONE +---------------+ END_PROGRAM

Figure 11c - Graphical use of a function block name as an external variable

(table 33, feature 7b)

NOTE	The PROGRAM declaration mechanism is defined in 2.5.3.

| 12a) |  | +-------+ ¦ ACCUM ¦ INT---¦A-----A¦---INT INT---¦X ¦ +-------+ +---+ A---¦ + ¦---A X---¦ ¦ +---+ | FUNCTION_BLOCK ACCUM VAR_IN_OUT A : INT ; END_VAR VAR_INPUT X : INT ; END_VAR A := A+X ; END_FUNCTION_BLOCK |  |
| --- | --- | --- | --- | --- |
| 12b) |  | ACC1 +-------+ ¦ ACCUM ¦ ACC----------¦A-----A¦---ACC +---+ ¦ ¦ X1---¦ * ¦---¦X ¦ X2---¦ ¦ +-------+ +---+ | A declaration such as VAR ACC : INT ; X1 : INT ; X2 : INT ; END_VAR is assumed: the effect of execution is ACC := ACC+X1*X2 ; |  |
| 12c) | ACC1 ACC2 +-------+ +-------+ ¦ ACCUM ¦ ¦ ACCUM ¦ ACC----------¦A-----A¦----------------¦A-----A¦---ACC +---+ ¦ ¦ +---+ ¦ ¦ X1---¦ * ¦---¦X ¦ X3---¦ * ¦---¦X ¦ X2---¦ ¦ +-------+ X4---¦ ¦ +-------+ +---+ +---+ |  |  | Declarations as in 12b) are assumed for ACC, X1, X2, X3, and X4.; the effect of execution is ACC := ACC+X1*X2+X3*X4; |
| 12d) |  | ACC1 +-------+ ¦ ACCUM ¦ X3-----------¦A-----A¦---X4 +---+ ¦ ¦ X1---¦ * ¦---¦X ¦ X2---¦ ¦ +-------+ +---+ | A declaration such as VAR X1 : INT ; X2 : INT ; ....X3 : INT ; ... X4 : INT ; END_VAR is assumed: the effect of execution is X3 := X3+X1*X2 ; X4 := X3 ; |  |
| 12e) |  | ACC1 +---+ +-------+ X1---¦ * ¦ ¦ ACCUM ¦ X2---¦ ¦---¦A-----A¦---ACC +---+ ¦ ¦ X3-----------¦X ¦ +-------+ | ILLEGAL USAGE!!! Connection to in-out variable A is not a variable or function block name (see preceding text). |  |

Figure 12 - Declaration and usage of in-out variables in function blocks

a) Graphical and textual declarations
b), c), d)  Legal usage,  e) Illegal usage

#### 2.5.2.3  Standard function blocks

Definitions of function blocks common to all programmable controller programming languages are given in this subclause.

Where graphical declarations of standard function blocks are shown in this subclause, equivalent textual declarations, as specified in 2.5.2.2, can also be written, as for example in table 35.

Standard function blocks may be overloaded and may have extensible inputs and outputs.  The definitions of such function block types shall describe any constraints on the number and data types of such inputs and outputs.  The use of such capabilities in non-standard function blocks is beyond the scope of this Standard.

##### 2.5.2.3.1  Bistable elements

The graphical form and function block body of standard bistable elements are shown in table 34.  The notation for these elements is chosen to be as consistent as possible with symbols 12-09-01 and 12-09-02 of IEC 617-12.

Table 34 - Standard bistable function blocks a

| No. | Graphical form | Function block body |
| --- | --- | --- |
| 1 | Bistable Function Block (set dominant) |  |
|  | +-----+ ¦ SR ¦ BOOL---¦S1 Q1¦---BOOL BOOL---¦R ¦ +-----+ | +-----+ S1----------------¦ >=1 ¦---Q1 +---+ ¦ ¦ R------O¦ & ¦----¦ ¦ Q1------¦ ¦ ¦ ¦ +---+ +-----+ |
| 2 | Bistable Function Block (reset dominant) |  |
|  | +-----+ ¦ RS ¦ BOOL---¦S Q1¦---BOOL BOOL---¦R1 ¦ +-----+ | +---+ R1----------------O¦ & ¦---Q1 +-----+ ¦ ¦ S-------¦ >=1 ¦----¦ ¦ Q1------¦ ¦ ¦ ¦ +-----+ +---+ |
| NOTE The function block body is specified in the Function Block Diagram (FBD) language defined in 4.3. |  |  |
| a The initial state of the output variable Q1 shall be the normal default value of zero for Boolean variables. |  |  |

##### 2.5.2.3.2  Edge detection

The graphic representation of standard rising- and falling-edge detecting function blocks shall be as shown in table 35. The behaviors of these blocks shall be equivalent to the definitions given in this table.  This behavior corresponds to the following rules:

1) The Q output  of an R_TRIG function block shall stand at the BOOL#1 value from one execution of the function block to the next, following the 0 to 1 transition of the CLK input, and shall return to 0 at the next execution.

2) The Q output of an F_TRIG function block shall stand at the BOOL#1 value from one execution of the function block to the next, following the 1 to 0 transition of the CLK input, and shall return to 0 at the next execution.

Table 35 - Standard edge detection function blocks

| No. | Graphical form | Definition (ST language - see 3.3) |
| --- | --- | --- |
| 1 | Rising edge detector |  |
|  | +--------+ ¦ R_TRIG ¦ BOOL---¦CLK Q¦---BOOL +--------+ | FUNCTION_BLOCK R_TRIG VAR_INPUT CLK: BOOL; END_VAR VAR_OUTPUT Q: BOOL; END_VAR VAR M: BOOL; END_VAR Q := CLK AND NOT M; M := CLK; END_FUNCTION_BLOCK |
| 2 | Falling edge detector |  |
|  | +--------+ ¦ F_TRIG ¦ BOOL---¦CLK Q¦---BOOL +--------+ | FUNCTION_BLOCK F_TRIG VAR_INPUT CLK: BOOL; END_VAR VAR_OUTPUT Q: BOOL; END_VAR VAR M: BOOL; END_VAR Q := NOT CLK AND NOT M; M := NOT CLK; END_FUNCTION_BLOCK |
| NOTE When the CLK input of an instance of the R_TRIG type is connected to a value of BOOL#1, its Q output will stand at BOOL#1 after its first execution following a "cold restart" as described in 2.4.2. The Q output will stand at BOOL#0 following all subsequent executions. The same applies to an F_TRIG instance whose CLK input is disconnected or is connected to a value of FALSE. |  |  |

##### 2.5.2.3.3  Counters

The graphic representations of standard counter function blocks, with the types of the associated inputs and outputs, shall be as shown in table 36.  The operation of these function blocks shall be as specified in the corresponding function block bodies.

| Table 36 - Standard counter function blocks |  |  |
| --- | --- | --- |
| No. | Graphical form | Function block body (ST language - see 3.3) |
|  | Up-counter |  |
| 1a | +-----+ ¦ CTU ¦ BOOL--->CU Q¦---BOOL BOOL---¦R ¦ INT---¦PV CV¦---INT +-----+ | IF R THEN CV := 0 ; ELSIF CU AND (CV < PVmax) THEN CV := CV+1; END_IF ; Q := (CV >= PV) ; |
| 1b | +----------+ ¦ CTU_DINT ¦ BOOL--->CU Q¦---BOOL BOOL---¦R ¦ DINT---¦PV CV¦---DINT +----------+ | Same as 1a. |
| 1c | +----------+ ¦ CTU_LINT ¦ BOOL--->CU Q¦---BOOL BOOL---¦R ¦ LINT---¦PV CV¦---LINT +----------+ | Same as 1a. |
| 1d | +-----------+ ¦ CTU_UDINT ¦ BOOL--->CU Q¦---BOOL BOOL---¦R ¦ UDINT---¦PV CV¦---UDINT +-----------+ | Same as 1a. |
| 1e | +-----------+ ¦ CTU_ULINT ¦ BOOL--->CU Q¦---BOOL BOOL---¦R ¦ ULINT---¦PV CV¦---ULINT +-----------+ | Same as 1a. |
|  | Down-counter |  |
| 2a | +-----+ ¦ CTD ¦ BOOL--->CD Q¦---BOOL BOOL---¦LD ¦ INT---¦PV CV¦---INT +-----+ | IF LD THEN CV := PV ; ELSIF CD AND (CV > PVmin) THEN CV := CV-1; END_IF ; Q := (CV <= 0) ; |
| 2b | +----------+ ¦ CTD_DINT ¦ BOOL--->CD Q¦---BOOL BOOL---¦LD ¦ DINT---¦PV CV¦---DINT +----------+ | Same as 2a. |
| 2c | +----------+ ¦ CTD_LINT ¦ BOOL--->CD Q¦---BOOL BOOL---¦LD ¦ LINT---¦PV CV¦---LINT +----------+ | Same as 2a. |
| 2d | +-----------+ ¦ CTD_UDINT ¦ BOOL--->CD Q¦---BOOL BOOL---¦LD ¦ UDINT---¦PV CV¦---UDINT +-----------+ | Same as 2a. |
| 2e | +-----------+ ¦ CTD_ULINT ¦ BOOL--->CD Q¦---BOOL BOOL---¦LD ¦ ULINT---¦PV CV¦---ULINT +-----------+ | Same as 2a. |
|  | Up-down counter |  |
| 3a | +------+ ¦ CTUD ¦ BOOL--->CU QU¦---BOOL BOOL--->CD QD¦---BOOL BOOL---¦R ¦ BOOL---¦LD ¦ INT---¦PV CV¦---INT +------+ | IF R THEN CV := 0 ; ELSIF LD THEN CV := PV ; ELSE IF NOT (CU AND CD) THEN IF CU AND (CV < PVmax) THEN CV := CV+1; ELSIF CD AND (CV > PVmin) THEN CV := CV-1; END_IF; END_IF; END_IF ; QU := (CV >= PV) ; QD := (CV <= 0) ; |
| 3b | +-----------+ ¦ CTUD_DINT ¦ BOOL--->CU QU¦---BOOL BOOL--->CD QD¦---BOOL BOOL---¦R ¦ BOOL---¦LD ¦ DINT---¦PV CV¦---DINT +-----------+ | Same as 3a. |
| 3c | +-----------+ ¦ CTUD_LINT ¦ BOOL--->CU QU¦---BOOL BOOL--->CD QD¦---BOOL BOOL---¦R ¦ BOOL---¦LD ¦ LINT---¦PV CV¦---LINT +-----------+ | Same as 3a. |
| 3d | +------------+ ¦ CTUD_ULINT ¦ BOOL--->CU QU¦---BOOL BOOL--->CD QD¦---BOOL BOOL---¦R ¦ BOOL---¦LD ¦ ULINT---¦PV CV¦---ULINT +------------+ | Same as 3a. |
| NOTE The numerical values of the limit variables PVmin and PVmax are implementation-dependent. |  |  |

##### 2.5.2.3.4  Timers

The graphic form for standard timer function blocks shall be as shown in table 37.  The operation of these function blocks shall be as defined in the timing diagrams given in table 38.

Table 37 - Standard timer function blocks

| No. | Description | Graphical form |
| --- | --- | --- |
| 1 2a 2b a 3a 3b a | *** is: TP (Pulse) TON (On-delay) T---0 (On-delay) TOF (Off-delay) 0---T (Off-delay) | +-------+ ¦ *** ¦ BOOL---¦IN Q¦---BOOL TIME---¦PT ET¦---TIME +-------+ |
| NOTE The effect of a change in the value of the PT input during the timing operation, e.g., the setting of PT to t#0s to reset the operation of a TP instance, is an implementation-dependent parameter. |  |  |
| a In textual languages, features 2b and 3b shall not be used. |  |  |

Table 38 - Standard timer function blocks - timing diagrams

Pulse (TP) timing

+--------+ ++ ++ +--------+ IN ¦ ¦ ¦¦ ¦¦ ¦ ¦ --+ +-----++-++---+ +--------- t0 t1 t2 t3 t4 t5 +----+ +----+ +----+ Q ¦ ¦ ¦ ¦ ¦ ¦ --+ +---------+ +--+ +------------- t0 t0+PT t2 t2+PT t4 t4+PT PT +---+ + +---+ : / ¦ /¦ / ¦ ET : / ¦ / ¦ / ¦ : / ¦ / ¦ / ¦ : / ¦ / ¦ / ¦ 0-+ +-----+ +--+ +--------- t0 t1 t2 t4 t5

On-delay (TON) timing

+--------+ +---+ +--------+ IN ¦ ¦ ¦ ¦ ¦ ¦ --+ +--------+ +---+ +------------- t0 t1 t2 t3 t4 t5

+---+ +---+ Q ¦ ¦ ¦ ¦ -------+ +---------------------+ +------------- t0+PT t1 t4+PT t5

PT +---+ +---+ : / ¦ + / ¦ ET : / ¦ /¦ / ¦ : / ¦ / ¦ / ¦ : / ¦ / ¦ / ¦ 0-+ +--------+ +---+ +------------- t0 t1 t2 t3 t4 t5

Off-delay (TOF) timing

+--------+ +---+ +--------+ IN ¦ ¦ ¦ ¦ ¦ ¦ ---+ +--------+ +---+ +----------- t0 t1 t2 t3 t4 t5

+-------------+ +---------------------+ Q ¦ ¦ ¦ ¦ ---+ +---+ +------ t0 t1+PT t2 t5+PT

PT +---+ +------ : / ¦ + / ET : / ¦ /¦ / : / ¦ / ¦ / : / ¦ / ¦ / 0------------+ +---+ +--------+ t1 t3 t5

##### 2.5.2.3.5  Communication function blocks

Standard communication function blocks for programmable controllers are defined in IEC 61131-5.  These function blocks provide programmable communications functionality such as device verification, polled data acquisition, programmed data acquisition, parametric control, interlocked control, programmed alarm reporting, and connection management and protection.

### 2.5.3  Programs

A program is defined in IEC 61131-1 as a "logical assembly of all the programming language elements and constructs necessary for the intended signal processing required for the control of a machine or process by a programmable controller system."

Subclause 1.4.1 of this part describes the place of programs in the overall software model of a programmable controller; subclause 1.4.2 describes the means available for inter- and intra-program communication; and subclause 1.4.3 describes the overall process of program development.

The declaration and usage of programs is identical to that of function blocks as defined in 2.5.2.1 and 2.5.2.2, with the additional features shown in table 39 and the following differences:

- The delimiting keywords for program declarations shall be PROGRAM...END_PROGRAM.

- A program can contain a VAR_ACCESS...END_VAR construction, which provides a means of specifying named variables which can be accessed by some of the communication services specified in IEC 61131-5.  An access path associates each such variable with an input, output or internal variable of the program.  The format and usage of this declaration shall be as described in 2.7.1 and in IEC 61131-5.

- Programs can only be instantiated within resources, as defined in 2.7.1, while function blocks can only be instantiated within programs or other function blocks.

- A program can contain location assignments as described in 2.4.3.1 and 2.4.3.2 in the declarations of its global and internal variables. Location assignments with not fully specified direct representation as described in 2.4.1.1 and 2.4.3.1 can only be used in the declaration of internal variables of a program.

The declaration and use of programs are illustrated in figure 19, and in examples F.7 and F.8 of annex F.

Limitations on the size of programs in a particular resource are implementation-dependent parameters.

| Table 39 - Program declaration features |  |
| --- | --- |
| No. | DESCRIPTION |
| 1 to 9b | Same as features 1 to 9b, respectively, of table 33 |
| 10 | Formal input and output variables |
| 11 to 14 | Same as features 1 to 4, respectively, of table 17 |
| 15 to 18 | Same as features 1 to 4, respectively, of table 18 |
| 19 | Use of directly represented variables (subclause 2.4.1.1) |
| 20 | VAR_GLOBAL...END_VAR declaration within a PROGRAM (see 2.4.3 and 2.7.1) |
| 21 | VAR_ACCESS...END_VAR declaration within a PROGRAM |
| 22a | VAR_EXTERNAL declarations within PROGRAM type declarations |
| 22b | VAR_EXTERNAL CONSTANT declarations within PROGRAM type declarations |
| 23 | VAR_GLOBAL CONSTANT declarations within PROGRAM type declarations |
| 24 | VAR_TEMP declarations (see 2.4.3) within PROGRAM type declarations |

## 2.6  Sequential Function Chart (SFC) elements

### 2.6.1  General

This subclause defines sequential function chart (SFC) elements for use in structuring the internal organization of a programmable controller program organization unit, written in one of the languages defined in this standard, for the purpose of performing sequential control functions.  The definitions in this subclause are derived from IEC 848, with the changes necessary to convert the representations from a documentation standard to a set of execution control elements for a programmable controller program organization unit.

The SFC elements provide a means of partitioning a programmable controller program organization unit into a set of steps and transitions interconnected by directed links.  Associated with each step is a set of actions, and with each transition is associated a transition condition.

Since SFC elements require storage of state information, the only program organization units which can be structured using these elements are function blocks and programs.

If any part of a program organization unit is partitioned into SFC elements, the entire program organization unit shall be so partitioned.  If no SFC partitioning is given for a program organization unit, the entire program organization unit shall be considered to be a single action which executes under the control of the invoking entity.

### 2.6.2  Steps

A step represents a situation in which the behavior of a program organization unit with respect to its inputs and outputs follows a set of rules defined by the associated actions of the step.  A step is either active or inactive.  At any given moment, the state of the program organization unit is defined by the set of active steps and the values of its internal and output variables.

As shown in table 40, a step shall be represented graphically by a block containing a step name in the form of an identifier as defined in 2.1.2, or textually by a STEP...END_STEP construction.  The directed link(s) into the step can be represented graphically by a vertical line attached to the top of the step.  The directed link(s) out of the step can be represented by a vertical line attached to the bottom of the step.  Alternatively, the directed links can be represented textually by the TRANSITION... END_TRANSITION construction defined in 2.6.3.

The step flag (active or inactive state of a step) can be represented by the logic value of a Boolean structure element ***.X, where *** is the step name, as shown in table 40.  This Boolean variable has the value 1 when the corresponding step is active, and 0 when it is inactive.  The state of this variable is available for graphical connection at the right side of the step as shown in table 40.

Similarly, the elapsed time, ***.T, since initiation of a step can be represented by a structure element of type TIME, as shown in table 40.  When a step is deactivated, the value of the step elapsed time shall remain at the value it had when the step was deactivated. When a step is activated, the value of the step elapsed time shall be reset to t#0s.

The scope of step names, step flags, and step times shall be local to the program organization unit in which the steps appear.

The initial state of the program organization unit is represented by the initial values of its internal and output variables, and by its set of initial steps, i.e., the steps which are initially active.  Each SFC network, or its textual equivalent, shall have exactly one initial step.

An initial step can be drawn graphically with double lines for the borders, and with the character set defined in 2.1.1 shall be drawn as shown in table 40.

For system initialization as defined in 2.4.2, the default initial elapsed time for steps is t#0s, and the default initial state is BOOL#0 for ordinary steps and BOOL#1 for initial steps.  However, when an instance of a function block or a program is declared to be retentive (for instance, as in feature 3 of table 33), the states and (if supported) elapsed times of all steps contained in the program or function block shall be treated as retentive for system initialization as defined in 2.4.2.

The maximum number of steps per SFC and the precision of step elapsed time are implementation-dependent parameters.

It shall be an error if: 1) an SFC network does not contain exactly one initial step; 2) a user program attempts to assign a value directly to the step state or the step time.

| Table 40 - Step features |  |  |
| --- | --- | --- |
| No. | REPRESENTATION | DESCRIPTION |
| 1 | ¦ +-----+ ¦ *** ¦ +-----+ ¦ | Step - Graphical form with directed links "***" = step name |
|  | ¦ +=======+ ¦¦ *** ¦¦ ¦¦ ¦¦ +=======+ ¦ | Initial step - Graphical form with directed links "***" = Name of initial step |
| 2 | STEP *** : (* Step body *) END_STEP | Step - Textual form without directed links (see 2.6.3) "***" = Step name |
|  | INITIAL_STEP *** : (* Step body *) END_STEP | Initial step - Textual form without directed links (see 2.6.3) "***" = Name of initial step |
| 3a a | ***.X | Step flag - General form "***" = Step name ***.X = BOOL#1 when *** is active, BOOL#0 otherwise |
| 3b a | ¦ +-----+ ¦ *** ¦---- +-----+ ¦ | Step flag - Direct connection of Boolean variable ***.X to right side of step "***" |
| 4 a | ***.T | Step elapsed time - General form "***" = Step name ***.T = A variable of type TIME (See 2.6.2) |
| NOTE The upper directed link to an initial step is not present if it has no predecessors. |  |  |
| a When feature 3a, 3b, or 4 is supported, it shall be an error if the user program attempts to modify the associated variable. For example, if S4 is a step name, then the following statements would be errors in the ST language defined in 3.3: S4.X := 1 ; (* ERROR *) S4.T := t#100ms ; (* ERROR *) |  |  |

### 2.6.3  Transitions

A transition represents the condition whereby control passes from one or more steps preceding the transition to one or more successor steps along the corresponding directed link.  The transition shall be represented by a horizontal line across the vertical directed link.

The direction of evolution following the directed links shall be from the bottom of the predecessor step(s) to the top of the successor step(s).

Each transition shall have an associated transition condition which is the result of the evaluation of a single Boolean expression.  A transition condition which is always true shall be represented by the symbol 1 or the keyword TRUE.

A transition condition can be associated with a transition by one of the following means, as shown in table 41:

1) By placing the appropriate Boolean expression in the ST language defined in 3.3 physically or logically adjacent to the vertical directed link.

2) By a ladder diagram network in the LD language defined in 4.2, physically or logically adjacent to the vertical directed link.

3) By a network in the FBD language defined in 4.3, physically or logically adjacent to the vertical directed link.

4) By a LD or FBD network whose output intersects the vertical directed link via a connector as defined in 4.1.1.

5) By a TRANSITION...END_TRANSITION construct using the ST language.  This shall consist of:

- The keywords TRANSITION FROM followed by the step name of the predecessor step (or, if there is more than one predecessor, by a parenthesized list of predecessor steps);

- The keyword TO followed by the step name of the successor step (or, if there is more than one successor, by a parenthesized list of successor steps);

- The assignment operator (:=), followed by a Boolean expression in the ST language, specifying the transition condition;

- The terminating keyword END_TRANSITION.

6) By a TRANSITION...END_TRANSITION construct using the IL language defined in 3.2.  This shall consist of:

- The keywords TRANSITION FROM followed by the step name of the predecessor step (or, if there is more than one predecessor, by a parenthesized list of predecessor steps), followed by a colon (:);

- The keyword TO followed by the step name of the successor step (or, if there is more than one successor, by a parenthesized list of successor steps);

- Beginning on a separate line, a list of instructions in the IL language, the result of whose evaluation determines the transition condition;

- The terminating keyword END_TRANSITION on a separate line.

7) By the use of a transition name in the form of an identifier to the right of the directed link.  This identifier shall refer to a TRANSITION...END_TRANSITION construction defining one of the following entities, whose evaluation shall result in the assignment of a Boolean value to the variable denoted by the transition name:

- A network in the LD or FBD language;

- A list of instructions in the IL language;

- An assignment of a Boolean expression in the ST language.

The scope of a transition name shall be local to the program organization unit in which the transition is located.

It shall be an error in the sense of 1.5.1 if any "side effect" (for instance, the assignment of a value to a variable other than the transition name) occurs during the evaluation of a transition condition.

The maximum number of transitions per SFC and per step are implementation-dependent parameters.

| Table 41 - Transitions and transition conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Example |  | Description |  |  |
| 1a | ¦ +-----+ ¦STEP7¦ +-----+ ¦ + %IX2.4 & %IX2.3 ¦ +-----+ ¦STEP8¦ +-----+ ¦ |  | Predecessor step Transition condition physically or logically adjacent to the transition using ST language (see 3.3) Successor step |  |  |
| 2a | ¦ +-----+ ¦STEP7¦ +-----+ ¦ %IX2.4 %IX2.3 ¦ +---¦¦-----¦¦--------+ ¦ ¦ +-----+ ¦STEP8¦ +-----+ ¦ |  | Predecessor step Transition condition physically or logically adjacent to the transition using LD language (see 4.2) Successor step |  |  |
| 3a | ¦ +-----+ ¦STEP7¦ +-------+ +-----+ ¦ & ¦ ¦ %IX2.4---¦ ¦-----+ %IX2.3---¦ ¦ ¦ +-------+ +-----+ ¦STEP8¦ +-----+ ¦ |  | Predecessor step Transition condition physically or logically adjacent to the transition using FBD language (see 4.3) Successor step |  |  |
| 4a |  | ¦ +-----+ ¦STEP7¦ +-----+ ¦ >TRANX>-------------+ ¦ +-----+ ¦STEP8¦ +-----+ ¦ |  |  | Use of connector: Predecessor step Transition connector Successor step |
| 4a 4b |  | ¦ %IX2.4 %IX2.3 +---¦¦-----¦¦---->TRANX> ¦ +-------+ ¦ & ¦ %IX2.4---¦ ¦-->TRANX> %IX2.3---¦ ¦ +-------+ |  |  | Transition condition: Using LD language (see 4.2) Using FBD language (see 4.3) |
| 5b |  | STEP STEP7: END_STEP TRANSITION FROM STEP7 TO STEP8 := %IX2.4 & %IX2.3 ; END_TRANSITION STEP STEP8: END_STEP |  |  | Textual equivalent of feature 1 using ST language (see 3.3) |
| 6b |  | STEP STEP7: END_STEP TRANSITION FROM STEP7 TO STEP 8: LD %IX2.4 AND %IX2.3 END_TRANSITION STEP STEP8: END_STEP |  |  | Textual equivalent of feature 1 using IL language (see 3.2) |
| 7a | ¦ +-----+ ¦STEP7¦ +-----+ ¦ + TRAN78 ¦ +-----+ ¦STEP8¦ +-----+ ¦ |  |  | Use of transition name: Predecessor step Transition name Successor step |  |
| 7a | TRANSITION TRAN78 FROM STEP7 TO STEP8: ¦ ¦ ¦ %IX2.4 %IX2.3 TRAN78 ¦ +---¦¦-----¦¦------( )---+ ¦ ¦ END_TRANSITION |  |  | Transition condition using LD language (see 4.2) |  |
| 7b | TRANSITION TRAN78 FROM STEP7 TO STEP8: +-------+ ¦ & ¦ %IX2.4---¦ ¦--TRAN78 %IX2.3---¦ ¦ +-------+ END_TRANSITION |  |  | Transition condition using FBD language (see 4.3) |  |
| 7c | TRANSITION TRAN78 FROM STEP7 TO STEP8: LD %IX2.4 AND %IX2.3 END_TRANSITION |  |  | Transition condition using IL language (see 3.2) |  |
| 7d | TRANSITION TRAN78 FROM STEP7 TO STEP8 := %IX2.4 & %IX2.3 ; END_TRANSITION |  |  | Transition condition using ST language (see 3.3) |  |
| a If feature 1 of table 40 is supported, then one or more of features 1, 2, 3, 4, or 7 of this table shall be supported. b If feature 2 of table 40 is supported, then feature 5 or 6 of this table, or both, shall be supported. |  |  |  |  |  |

### 2.6.4  Actions

Zero or more actions shall be associated with each step.  A step which has zero associated actions shall be considered as having a WAIT function, that is, waiting for a successor transition condition to become true.

An action can be a Boolean variable, a collection of instructions in the IL language defined in 3.2, a collection of statements in the ST language defined in 3.3, a collection of rungs in the LD language defined in 4.2, a collection of networks in the FBD language defined in 4.3, or a sequential function chart (SFC) organized as defined in this subclause (2.6).

Actions shall be declared via one or more of the mechanisms defined in 2.6.4.1, and shall be associated with steps via textual step bodies or graphical action blocks, as defined in 2.6.4.2.  The details of action block representation are defined in 2.6.4.3.  Control of actions shall be expressed by action qualifiers as defined in 2.6.4.4.

#### 2.6.4.1  Declaration

A programmable controller implementation which supports SFC elements shall provide one or more of the mechanisms defined in table 42 for the declaration of actions.  The scope of the declaration of an action shall be local to the program organization unit containing the declaration.

| Table 42 - Declaration of actions a,b |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. |  | Feature |  |  |  |  |  |
| 1 |  | Any Boolean variable declared in a VAR or VAR_OUTPUT block, or their graphical equivalents, can be an action. |  |  |  |  |  |
| No. |  | Example |  |  | Feature |  |  |
| 2l |  | +----------------------------------------+ ¦ ACTION_4 ¦ +----------------------------------------+ ¦ ¦ %IX1 %MX3 S8.X %QX17 ¦ ¦ ¦ +---¦¦-----¦¦----¦¦-----()---+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +------+ ¦ ¦ ¦ +----¦EN ENO¦ %MX10 ¦ ¦ ¦ ¦ C--¦ LT ¦----------(S)---+ ¦ ¦ ¦ D--¦ ¦ ¦ ¦ ¦ ¦ +------+ ¦ ¦ +----------------------------------------+ |  |  | Graphical declaration in LD language (see 4.2) |  |  |
| 2s | +----------------------------------------+ ¦ OPEN_VALVE_1 ¦ +----------------------------------------+ ¦ ¦ ... ¦ ¦ +=================+ ¦ ¦ ¦¦ VALVE_1_READY ¦¦ ¦ ¦ +=================+ ¦ ¦ ¦ ¦ ¦ + STEP8.X ¦ ¦ ¦ ¦ ¦ +-----------------+ +---+-----------+ ¦ ¦ ¦ VALVE_1_OPENING ¦--¦ N ¦VALVE_1_FWD¦ ¦ ¦ +-----------------+ +---+-----------+ ¦ ¦ ¦ ... ¦ +----------------------------------------+ |  |  | Inclusion of SFC elements in action |  |  |  |
| 2f | +----------------------------------------+ ¦ ACTION_4 ¦ +----------------------------------------+ ¦ +---+ ¦ ¦ %IX1--¦ & ¦ ¦ ¦ %MX3--¦ ¦--%QX17 ¦ ¦ S8.X---------¦ ¦ ¦ ¦ +---+ FF28 ¦ ¦ +----+ ¦ ¦ ¦ SR ¦ ¦ ¦ +------+ ¦ Q1¦-%MX10 ¦ ¦ C--¦ LT ¦--¦S1 ¦ ¦ ¦ D--¦ ¦ +----+ ¦ ¦ +------+ ¦ +----------------------------------------+ |  |  | Graphical declaration in FBD language (see 4.3) |  |  |  |
| 3s |  |  | ACTION ACTION_4: %QX17 := %IX1 & %MX3 & S8.X ; FF28(S1 := (C<D)); %MX10 := FF28.Q; END_ACTION |  |  | Textual declaration in ST language (see 3.3) |  |
| 3i |  |  | ACTION ACTION_4: LD S8.X AND %IX1 AND %MX3 ST %QX17 LD C LT D S1 FF28 LD FF28.Q ST %MX10 END_ACTION |  |  | Textual declaration in IL language (see 3.2) |  |
| NOTE The step flag S8.X is used in these examples to obtain the desired result that, when S8 is deactivated, %QX17 := 0. |  |  |  |  |  |  |  |
| a If feature 1 of table 40 is supported, then one or more of the features in this table, or feature 4 of table 43, shall be supported. b If feature 2 of table 40 is supported, then one or more of features 1,3s, or 3i of this table shall be supported. |  |  |  |  |  |  |  |

#### 2.6.4.2  Association with steps

A programmable controller implementation which supports SFC elements shall provide one or more of the mechanisms defined in table 43 for the association of actions with steps.  The maximum number of action blocks per step is an implementation-dependent parameter.

| Table 43 - Step/action association |  |  |
| --- | --- | --- |
| No. | Example | Feature |
| 1 | ¦ +----+ +-----+----------+---+ ¦ S8 ¦--¦ L ¦ ACTION_1 ¦DN1¦ +----+ ¦t#10s¦ ¦ ¦ ¦ +-----+----------+---+ + DN1 ¦ | Action block physically or logically adjacent to the step (see 2.6.4.3) |
| 2 | ¦ +----+ +-----+---------------------+---+ ¦ S8 ¦--¦ L ¦ ACTION_1 ¦DN1¦ +----+ ¦t#10s¦ ¦ ¦ ¦ +-----+---------------------+---+ +DN1 ¦ P ¦ ACTION_2 ¦ ¦ ¦ +-----+---------------------+---+ ¦ ¦ N ¦ ACTION_3 ¦ ¦ ¦ +-----+---------------------+---+ | Concatenated action blocks physically or logically adjacent to the step |
| 3 | STEP S8: ACTION_1(L,t#10s,DN1) ; ACTION_2(P) ; ACTION_3(N) ; END_STEP | Textual step body |
| 4 a | +-----+----------------------+---+ ----¦ N ¦ ACTION_4 ¦ ¦--- +-----+----------------------+---+ ¦ %QX17 := %IX1 & %MX3 & S8.X ; ¦ ¦ FF28 (S1 := (C<D)); ¦ ¦ %MX10 := FF28.Q; ¦ +--------------------------------+ | Action block "d" Field (see 2.6.4.3) |
| a When feature 4 is used, the corresponding action name cannot be used in any other action block. |  |  |

#### 2.6.4.3  Action blocks

As shown in table 44, an action block is a graphical element for the combination of a Boolean variable with one of the action qualifiers specified in subclause 2.6.4.4 to produce an enabling condition, according to the rules given in subclause 2.6.4.5, for an associated action.

The action block provides a means of optionally specifying Boolean "indicator" variables, indicated by the "c" field in table 44, which can be set by the specified action to indicate its completion, timeout, error conditions, etc.  If the "c" field is not present, and the "b" field specifies that the action shall be a Boolean variable, then this variable shall be interpreted as the "c" variable when required. If the (c) field is not defined, and the (b) field does not specify a Boolean variable, then the value of the "indicator" variable is considered to be always FALSE.

When action blocks are concatenated graphically as illustrated in table 43, such concatenations can have multiple indicator variables, but shall have only a single common Boolean input variable, which shall act simultaneously upon all the concatenated blocks.

As well as being associated with a step, an action block can be used as a graphical element in the LD or FBD languages specified in clause 4.  In this case, signal or power flow through an action block shall follow the rules specified in 4.1.1.

Table 44 - Action block features

| No. | Feature | Graphical form |
| --- | --- | --- |
| 1 a 2 3 b 4 5 6 7 | "a" : Qualifier as per 2.6.4.4 "b" : Action name "c" : Boolean "indicator" variables "d" : Action using: - IL language (3.2) - ST language (3.3) - LD language (4.2) - FBD language (4.3) | +-----+--------------+-----+ ---¦ "a" ¦ "b" ¦ "c" ¦--- +-----+--------------+-----+ ¦ "d" ¦ ¦ ¦ +--------------------------+ |
| No. | Feature/Example |  |
| 8 | Use of action blocks in ladder diagrams (see 4.2): |  |
|  | ¦ S8.X %IX7.5 +---+------+---+ OK1 ¦ +--¦ ¦----¦ ¦----¦ N ¦ ACT1 ¦DN1¦--( )--+ ¦ +---+------+---+ ¦ |  |
| 9 | Use of action blocks in function block diagrams (see 4.3): |  |
|  | +---+ +---+------+-----+ S8.X---¦ & ¦-----¦ N ¦ ACT1 ¦ DN1 ¦---OK1 %IX7.5---¦ ¦ +---+------+-----+ +---+ |  |
| a Field "a" can be omitted when the qualifier is "N". b Field "c" can be omitted when no indicator variable is used. |  |  |

#### 2.6.4.4  Action qualifiers

Associated with each step/action association defined in 2.6.4.2, or each occurrence of an action block as defined in 2.6.4.3, shall be an action qualifier.  The value of this qualifier shall be one of the values listed in table 45.  In addition, the qualifiers L, D, SD, DS, and SL shall have an associated duration of type TIME.

NOTE	IEC 848 gives informal definitions and examples of the use of these qualifiers.  This standard formalizes these definitions, redefining the S qualifier and introducing the R qualifier.  The control of actions using these qualifiers is defined in the following subclause, and additional examples of their use are given in annex F.

Table 45 - Action qualifiers

| No. | Qualifier | Explanation |
| --- | --- | --- |
| 1 | None | Non-stored (null qualifier) |
| 2 | N | Non-stored |
| 3 | R | overriding Reset |
| 4 | S | Set (Stored) |
| 5 | L | time Limited |
| 6 | D | time Delayed |
| 7 | P | Pulse |
| 8 | SD | Stored and time Delayed |
| 9 | DS | Delayed and Stored |
| 10 | SL | Stored and time Limited |
| 11 | P1 | Pulse (rising edge) |
| 12 | P0 | Pulse (falling edge) |

#### 2.6.4.5  Action control

The control of actions shall be functionally equivalent to the application of the following rules:

1)	Associated with each action shall be the functional equivalent of an instance of the ACTION_CONTROL function block defined in figures 14 and 15.  If the action is declared as a Boolean variable, as defined in 2.6.4.1, the Q output of this block shall be the state of this Boolean variable.  If the action is declared as a collection of statements or networks, as defined in 2.6.4.1, then this collection shall be executed continually while the A (activation) output of the ACTION_CONTROL function block stands at BOOL#1.  In this case, the state of the output Q (called the "action flag") can be accessed within the action by reading a read-only boolean variable which has the form of a reference to the Q output of a function block instance whose instance name is the same as the corresponding action name, e.g., ACTION1.Q.

NOTE 1	The condition Q=FALSE will ordinarily be used by an action to determine that it is being executed for the final time during its current activation.

NOTE 2	The value of Q will always be FALSE during execution of actions invoked by  P0 and P1 qualifiers.

NOTE 3	The value of A will be TRUE for only one execution of an action invoked by a P1 or P0 qualifier. For all other qualifiers, A will be true for one additional execution following the falling edge of Q.

NOTE 4	Access to the functional equivalent of the Q or A outputs of an ACTION_CONTROL function block from outside of the associated action is an implementation-dependent feature.

NOTE 5	The manufacturer may opt for a simpler implementation as shown in Figure 15(b).  In this case, if the action is declared as a collection of statements or networks, as defined in 2.6.4.1, then this collection shall be executed continually while the Q output of the ACTION_CONTROL function block stands at BOOL#1. In any case the manufacturer shall specify which of the features given in Table 45a is supported.

2)	A Boolean input to the ACTION_CONTROL block for an action shall be said to have an association with a step as defined in 2.6.4.2, or with an action block as defined in 2.6.4.3, if the corresponding qualifier is equivalent to the input name (N, R, S, L, D, P, P0, P1, SD, DS, or SL).  The association shall be said to be active if the associated step is active, or if the associated action block's input has the value BOOL#1.  The active associations of an action are equivalent to the set of active associations of all inputs to its ACTION_CONTROL function block.

A Boolean input to an ACTION_CONTROL block shall have the value BOOL#1 if it has at least one active association, and the value BOOL#0 otherwise.

3)	The value of the T input to an ACTION_CONTROL block shall be the value of the duration portion of a time-related qualifier (L, D, SD, DS, or SL) of an active association.  If no such association exists, the value of the T input shall be t#0s.

4)	It shall be an error in the sense of subclause 1.5.1 if one or more of the following conditions exist:

a) More than one active association of an action has a time-related qualifier (L, D, SD, DS, or SL).

b) The SD input to an ACTION_CONTROL block has the BOOL#1 when the Q1 output of its SL_FF block has the value BOOL#1.

c) The SL input to an ACTION_CONTROL block has the value BOOL#1 when the Q1 output of its SD_FF block has the value BOOL#1.

5)	It is not required that the ACTION_CONTROL block itself be implemented, but only that the control of actions be equivalent to the preceding rules.  Only those portions of the action control appropriate to a particular action need be instantiated, as illustrated in figure 16.  In particular, note that simple MOVE (:=) and Boolean OR functions suffice for control of Boolean variable actions if the latter's associations have only "N" qualifiers.

| a) | b) |
| --- | --- |
| +----------------+ ¦ ACTION_CONTROL ¦ BOOL---¦N Q¦---BOOL BOOL---¦R A¦---BOOL BOOL---¦S ¦ BOOL---¦L ¦ BOOL---¦D ¦ BOOL---¦P ¦ BOOL---¦P1 ¦ BOOL---¦P0 ¦ BOOL---¦SD ¦ BOOL---¦DS ¦ BOOL---¦SL ¦ TIME---¦T ¦ +----------------+ | +----------------+ ¦ ACTION_CONTROL ¦ BOOL---¦N Q¦---BOOL BOOL---¦R ¦ BOOL---¦S ¦ BOOL---¦L ¦ BOOL---¦D ¦ BOOL---¦P ¦ BOOL---¦P1 ¦ BOOL---¦P0 ¦ BOOL---¦SD ¦ BOOL---¦DS ¦ BOOL---¦SL ¦ TIME---¦T ¦ +----------------+ |

Figure 14 - ACTION_CONTROL function block - External interface(Not visible to the user)

a) With "final scan"logic - see Figure 15(a); b) Without "final scan"logic - see Figure 15(b)

+---+ +-----------------------------------------------------O¦ & ¦---Q ¦ +-----+ ¦ ¦ N--¦---------------------------------------------¦ >=1 ¦--¦ ¦ ¦ S_FF ¦ ¦ +---+ R--+ +----+ ¦ ¦ ¦ ¦ RS ¦ ¦ ¦ NOTE 1 S--¦----------------------¦S Q1¦-----------------¦ ¦ Instances of this +----------------------¦R1 ¦ ¦ ¦ function block ¦ +----+ +---+ ¦ ¦ are not visible L--¦---------+--------------------¦ & ¦----------¦ ¦ to the user ¦ ¦ L_TMR +--O¦ ¦ ¦ ¦ ¦ ¦ +-----+ ¦ +---+ ¦ ¦ NOTE 2 ¦ ¦ ¦ TON ¦ ¦ ¦ ¦ The external ¦ +------¦IN Q¦---+ D_TMR ¦ ¦ interface of this ¦ +-------------¦PT ¦ +-----+ ¦ ¦ function block type ¦ ¦ +-----+ ¦ TON ¦ ¦ ¦ is given in Figure D--¦--¦-----------------------------¦IN Q¦------¦ ¦ 14(a) ¦ +-----------------------------¦PT ¦ ¦ ¦ ¦ ¦ P_TRIG +-----+ ¦ ¦ ¦ ¦ +--------+ ¦ ¦ ¦ ¦ ¦ R_TRIG ¦ ¦ ¦ P--¦--¦------------¦CLK Q¦--------------------¦ ¦ ¦ ¦ SD_FF +--------+ SD_TMR ¦ ¦ ¦ ¦ +----+ +-----+ ¦ ¦ ¦ ¦ ¦ RS ¦ ¦ TON ¦ ¦ ¦ SD-¦--¦---¦S Q1¦----------------¦IN Q¦----------¦ ¦ +--¦---¦R1 ¦ +------------¦PT ¦ ¦ ¦ ¦ ¦ +----+ ¦ DS_TMR +-----+ DS_FF ¦ ¦ ¦ +------------+ +-----+ +----+ ¦ ¦ ¦ ¦ ¦ TON ¦ ¦ RS ¦ ¦ ¦ DS-¦--¦----------------¦IN Q¦----------¦S Q1¦---¦ ¦ ¦ +----------------¦PT ¦ +---¦R1 ¦ ¦ ¦ ¦ ¦ +-----+ ¦ +----+ ¦ ¦ +--¦-----------------------------+ ¦ ¦ ¦ ¦ SL_FF ¦ ¦ ¦ ¦ +----+ ¦ ¦ ¦ ¦ ¦ RS ¦ +---+ ¦ ¦ SL-¦--¦--------¦S Q1¦--+------------------¦ & ¦--¦ ¦ +--¦--------¦R1 ¦ ¦ SL_TMR +--O¦ ¦ +-----+ ¦ +----+ ¦ +-----+ ¦ +---+ ¦ ¦ ¦ TON ¦ ¦ ¦ +----¦IN Q¦---+ +-----+ T-----+---------------------¦PT ¦ +--------+ ¦ >=1 ¦ +-----+ ¦ F_TRIG ¦ Q---¦ ¦---A +--------+ Q---¦CLK Q¦---------¦ ¦ ¦ R_TRIG ¦ +--------+ ¦ ¦ P1--------------¦CLK Q¦----------------------------------¦ ¦ +--------+ +--------+ ¦ ¦ ¦ F_TRIG ¦ ¦ ¦ P0-----------------------------¦CLK Q¦-------------------¦ ¦ +--------+ +-----+

Figure 15a - ACTION_CONTROL function block body with "final scan" logic

+---+ +-----------------------------------------------------O¦ & ¦---Q ¦ +-----+ ¦ ¦ N--¦---------------------------------------------¦ >=1 ¦--¦ ¦ ¦ S_FF ¦ ¦ +---+ R--+ +----+ ¦ ¦ ¦ ¦ RS ¦ ¦ ¦ NOTE 1 - S--¦----------------------¦S Q1¦-----------------¦ ¦ instances of this +----------------------¦R1 ¦ ¦ ¦ function block ¦ +----+ +---+ ¦ ¦ are not visible L--¦---------+--------------------¦ & ¦----------¦ ¦ to the user ¦ ¦ L_TMR +--O¦ ¦ ¦ ¦ ¦ ¦ +-----+ ¦ +---+ ¦ ¦ NOTE 2 - ¦ ¦ ¦ TON ¦ ¦ ¦ ¦ The external ¦ +------¦IN Q¦---+ D_TMR ¦ ¦ interface of this ¦ +-------------¦PT ¦ +-----+ ¦ ¦ function block type ¦ ¦ +-----+ ¦ TON ¦ ¦ ¦ is given in Figure D--¦--¦-----------------------------¦IN Q¦------¦ ¦ 14(b) ¦ +-----------------------------¦PT ¦ ¦ ¦ ¦ ¦ P_TRIG +-----+ ¦ ¦ ¦ ¦ +--------+ ¦ ¦ ¦ ¦ ¦ R_TRIG ¦ ¦ ¦ P--¦--¦------------¦CLK Q¦--------------------¦ ¦ ¦ ¦ SD_FF +--------+ SD_TMR ¦ ¦ ¦ ¦ +----+ +-----+ ¦ ¦ ¦ ¦ ¦ RS ¦ ¦ TON ¦ ¦ ¦ SD-¦--¦---¦S Q1¦----------------¦IN Q¦----------¦ ¦ +--¦---¦R1 ¦ +------------¦PT ¦ ¦ ¦ ¦ ¦ +----+ ¦ DS_TMR +-----+ DS_FF ¦ ¦ ¦ +------------+ +-----+ +----+ ¦ ¦ ¦ ¦ ¦ TON ¦ ¦ RS ¦ ¦ ¦ DS-¦--¦----------------¦IN Q¦----------¦S Q1¦---¦ ¦ ¦ +----------------¦PT ¦ +---¦R1 ¦ ¦ ¦ ¦ ¦ +-----+ ¦ +----+ ¦ ¦ +--¦-----------------------------+ ¦ ¦ ¦ ¦ SL_FF ¦ ¦ ¦ ¦ +----+ ¦ ¦ ¦ ¦ ¦ RS ¦ +---+ ¦ ¦ SL-¦--¦--------¦S Q1¦--+------------------¦ & ¦--¦ ¦ +--¦--------¦R1 ¦ ¦ SL_TMR +--O¦ ¦ ¦ ¦ ¦ +----+ ¦ +-----+ ¦ +---+ ¦ ¦ ¦ ¦ ¦ TON ¦ ¦ ¦ ¦ ¦ +----¦IN Q¦---+ ¦ ¦ T-----+---------------------¦PT ¦ ¦ ¦ +--------+ +-----+ ¦ ¦ ¦ R_TRIG ¦ ¦ ¦ P1--------¦CLK Q¦-----------------------------¦ ¦ +--------+ +--------+ ¦ ¦ ¦ F_TRIG ¦ ¦ ¦ P0-----------------------¦CLK Q¦--------------¦ ¦ +--------+ +-----+

Figure 15b - ACTION_CONTROL function block body without "final scan" logic

¦ +-----+ +---+------------+----------------+ ¦ S22 ¦---¦ N ¦ HV_BREAKER ¦ HV_BRKR_CLOSED ¦ +-----+ +---+------------+----------------+ ¦ ¦ S ¦ START_INDICATOR ¦ ¦ +---+-----------------------------+ + HV_BRKR_CLOSED ¦ +-----+ +----+---------------+ ¦ S23 ¦---¦ SL ¦ RUNUP_MONITOR ¦ +-----+ ¦t#1m¦ ¦ ¦ +----+---------------+ ¦ ¦ D ¦ START_WAIT ¦ ¦ ¦t#1s¦ ¦ ¦ +----+---------------+ + START_WAIT ¦ +-----+ +-----+-----------------+------------------+ ¦ S24 ¦---¦ N ¦ ADVANCE_STARTER ¦ STARTER_ADVANCED ¦ +-----+ +-----+-----------------+------------------+ ¦ ¦ L ¦ START_MONITOR ¦ ¦ ¦t#30s¦ ¦ ¦ +-----+------------------------------------+ + STARTER_ADVANCED ¦ +-----+ +-----+-----------------+-------------------+ ¦ S26 ¦---¦ N ¦ RETRACT_STARTER ¦ STARTER_RETRACTED ¦ +-----+ +-----+-----------------+-------------------+ ¦ ¦ + STARTER_RETRACTED ¦ +-----+ +-----+-----------------+ ¦ S27 ¦---¦ R ¦ START_INDICATOR ¦ +-----+ +-----+-----------------+ ¦ ¦ R ¦ RUNUP_MONITOR ¦ ¦ +-----+-----------------+

NOTE The complete SFC network and its associated declarations are not shown in this example.

Figure 16a - Action control example - SFC representation

Table 45a - Action control features

| No. | Description |
| --- | --- |
| 1 | per Figures 14(a) and 15(a) |
| 2 | per Figures 14(b) and 15(b) |

S22.X---------------------------------------------------HV_BREAKER S24.X----------------------------------------------ADVANCE_STARTER S26.X----------------------------------------------RETRACT_STARTER START_INDICATOR_S_FF +----+ ¦ RS ¦ S22.X-----------------------¦S Q1¦-----------------START_INDICATOR S27.X-----------------------¦R1 ¦ +----+ START_WAIT_D_TMR +-----+ ¦ TON ¦ S23.X-----------------------¦IN Q¦---------------------START_WAIT t#1s------------------------¦PT ¦ +-----+ RUNUP_MONITOR_SL_FF +----+ ¦ RS ¦ +---+ S23.X---¦S Q1¦--+-----------------------------¦ & ¦--RUNUP_MONITOR S27.X---¦R1 ¦ ¦ RUNUP_MONITOR_SL_TMR +--O¦ ¦ +----+ ¦ +-----+ ¦ +---+ ¦ ¦ TON ¦ ¦ +---------¦IN Q¦---------+ t#1m----------------------¦PT ¦ +-----+ +---+ S24.X------------+---------------------------¦ & ¦---START_MONITOR ¦ START_MONITOR_L_TMR +---O¦ ¦ ¦ +-----+ ¦ +---+ ¦ ¦ TON ¦ ¦ +--------¦IN Q¦-------+ t#30s---------------------¦PT ¦ +-----+

Figure 16b - Action control example - functional equivalent

### 2.6.5  Rules of evolution

The initial situation of a SFC network is characterized by the initial step which is in the active state upon initialization of the program or function block containing the network.

Evolutions of the active states of steps shall take place along the directed links when caused by the clearing of one or more transitions.

A transition is enabled when all the preceding steps, connected to the corresponding transition symbol by directed links, are active.  The  clearing of a transition occurs when the transition is enabled and when the associated transition condition is true.

The clearing of a transition causes the deactivation (or "resetting") of all the immediately preceding steps connected to the corresponding transition symbol by directed links, followed by the activation of all the immediately following steps.

The alternation Step/Transition and Transition/Step shall always be maintained in SFC element connections, that is:

- Two steps shall never be directly linked; they shall always be separated by a transition.

- Two transitions shall never be directly linked; they shall always be separated by a step.

When the clearing of a transition leads to the activation of several steps at the same time, the sequences to which these steps belong are called simultaneous sequences.  After their simultaneous activation, the evolution of each of these sequences becomes independent.  In order to emphasize the special nature of such constructs, the divergence and convergence of simultaneous sequences shall be indicated by a double horizontal line.

It shall be an error if the possibility can arise that non-prioritized transitions in a selection divergence, as shown in Feature 2a of Table 46, are simultaneously true. The user may make provisions to avoid this error as shown in Features 2b and 2c of Table 46.

Table 46 defines the syntax and semantics of the allowed combinations of steps and transitions.

The clearing time of a transition may theoretically be considered as short as one may wish, but it can never be zero.  In practice, the clearing time will be imposed by the programmable controller implementation.  For the same reason, the duration of a step activity can never be considered to be zero.

Several transitions which can be cleared simultaneously shall be cleared simultaneously, within the timing constraints of the particular programmable controller implementation and the priority constraints defined in table 46.

Testing of the successor transition condition(s) of an active step shall not be performed until the effects of the step activation have propagated throughout the program organization unit in which the step is declared.

Figure 17 illustrates the application of these rules.  In this figure, the active state of a step is indicated by the presence of an asterisk (*) in the corresponding block.  This notation is used for illustration only, and is not a required language feature.

The application of the rules given in this subclause cannot prevent the formulation of "unsafe" SFCs, such as the one shown in figure 18a, which may exhibit uncontrolled proliferation of tokens.  Likewise, the application of these rules cannot prevent the formulation of "unreachable" SFCs, such as the one shown in figure 18b, which may exhibit "locked up" behavior.  The programmable controller system shall treat the existence of such conditions as errors as defined in 1.5.1.

The maximum allowed widths of the "divergence" and "convergence" constructs in Table 46 are implementation-dependent parameters.

| Table 46 - Sequence evolution |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | Example |  |  | Rule |  |  |  |
| 1 | ¦ +----+ ¦ S3 ¦ +----+ ¦ + c ¦ +----+ ¦ S4 ¦ +----+ ¦ |  |  | Single sequence: The alternation step-transition is repeated in series. Example: An evolution from step S3 to step S4 shall take place if and only if step S3 is in the active state and the transition condition c is true. |  |  |  |
| 2a | ¦ +----+ ¦ S5 ¦ +----+ ¦ +-----*----+--... ¦ ¦ + e + f ¦ ¦ +----+ +----+ ¦ S6 ¦ ¦ S8 ¦ +----+ +----+ ¦ ¦ |  |  | Divergence of sequence selection: A selection between several sequences is represented by as many transition symbols, under the horizontal line, as there are different possible evolutions. The asterisk denotes left-to-right priority of transition evaluations. Example: An evolution shall take place from S5 to S6 only if S5 is active and the transition condition e is true, or from S5 to S8 only if S5 is active and f is true and e is false. |  |  |  |
| 2b | ¦ +----+ ¦ S5 ¦ +----+ ¦ +-----*-----+--... ¦2 ¦1 + e + f ¦ ¦ +----+ +----+ ¦ S6 ¦ ¦ S8 ¦ +----+ +----+ ¦ ¦ |  |  | Divergence of sequence selection: The asterisk, followed by numbered branches, indicates a user-defined priority of transition evaluation, with the lowest-numbered branch having the highest priority. Example: An evolution shall take place from S5 to S8 only if S5 is active and the transition condition f is true, or from S5 to S6 only if S5 is active, and e is true, and f is false. |  |  |  |
| 2c | ¦ +----+ ¦ S5 ¦ +----+ ¦ +------+----+--... ¦ ¦ +e +NOT e & f ¦ ¦ +----+ +----+ ¦ S6 ¦ ¦ S8 ¦ +----+ +----+ ¦ ¦ |  |  | Divergence of sequence selection: The connection of the branch indicates that the user must assure that transition conditions are mutually exclusive, as specified by IEC 848. Example: An evolution shall take place from S5 to S6 only if S5 is active and the transition condition e is true, or from S5 to S8 only if S5 is active and e is false and f is true. |  |  |  |
| 3 |  |  | ¦ ¦ +----+ +----+ ¦ S7 ¦ ¦ S9 ¦ +----+ +----+ ¦ ¦ + h + j ¦ ¦ +-----+-----+--... ¦ +----+ ¦S10 ¦ +----+ ¦ |  |  |  | Convergence of sequence selection: The end of a sequence selection is represented by as many transition symbols, above the horizontal line, as there are selection paths to be ended. Example: An evolution shall take place from S7 to S10 only if S7 is active and the transition condition h is true, or from S9 to S10 only if S9 is active and j is true. |
| 4 |  |  | ¦ +----+ ¦S11 ¦ +----+ ¦ + b ¦ ==+=====+=====+==... ¦ ¦ +----+ +----+ ¦ S12¦ ¦ S14¦ +----+ +----+ ¦ ¦ |  |  |  | Simultaneous sequences - divergence: Only one common transition symbol shall be possible, above the double horizontal line of synchronization. Example: An evolution shall take place from S11 to S12, S14,... only if S11 is active and the transition condition "b" associated to the common transition is true. After the simultaneous activation of S12, S14, etc., the evolution of each sequence proceeds independently. |
|  |  |  | ¦ ¦ +----+ +----+ ¦ S13¦ ¦ S15¦ +----+ +----+ ¦ ¦ ==+=====+=====+==... ¦ + d ¦ +----+ ¦S16 ¦ +----+ ¦ |  |  |  | Simultaneous sequences - convergence: Only one common transition symbol shall be possible, under the double horizontal line of synchronization. Example: An evolution shall take place from S13, S15,... to S16 only if all steps above and connected to the double horizontal line are active and the transition condition "d" associated to the common transition is true. |
| 5a 5b 5c |  |  | ¦ +-----+ ¦ S30 ¦ +-----+ ¦ +---*---+ ¦ ¦ + a +d ¦ ¦ +-----+ ¦ ¦ S31 ¦ ¦ +-----+ ¦ ¦ ¦ + b ¦ ¦ ¦ +-----+ ¦ ¦ S32 ¦ ¦ +-----+ ¦ ¦ ¦ + c ¦ ¦ ¦ +---+---+ ¦ +-----+ ¦ S33 ¦ +-----+ ¦ |  |  | Sequence skip: A "sequence skip" is a special case of sequence selection (Feature 2) in which one or more of the branches contain no steps. Features 5a, 5b, and 5c correspond to the representation options given in features 2a, 2b, and 2c, respectively. Example: (Feature 5a shown) An evolution shall take place from S30 to S33 if "a" is false and "d" is true, that is, the sequence (S31, S32) will be skipped. |  |
| 6a 6b 6c |  |  | ¦ +-----+ ¦ S30 ¦ +-----+ ¦ + a ¦ +---------+ ¦ ¦ +-----+ ¦ ¦ S31 ¦ ¦ +-----+ ¦ ¦ ¦ + b ¦ ¦ ¦ +-----+ ¦ ¦ S32 ¦ ¦ +-----+ ¦ ¦ ¦ *-----+ ¦ ¦ ¦ ¦ + c + d ¦ ¦ ¦ ¦ +-----+ +---+ ¦ S33 ¦ +-----+ ¦ |  |  | Sequence loop: A "sequence loop" is a special case of sequence selection (Feature 2) in which one or more of the branches returns to a preceding step. Features 6a, 6b, and 6c correspond to the representation options given in features 2a, 2b, and 2c, respectively. Example: (Feature 6a shown) An evolution shall take place from S32 to S31 if "c" is false and "d" is true, that is, the sequence (S31, S32) will be repeated. |  |
| 7 |  | ¦ +-----+ ¦ S30 ¦ +-----+ ¦ + a ¦ +----<----+ ¦ ¦ +-----+ ¦ ¦ S31 ¦ ¦ +-----+ ¦ ¦ ¦ + b ¦ ¦ ¦ +-----+ ¦ ¦ S32 ¦ ¦ +-----+ ¦ ¦ ¦ *-----+ ¦ ¦ ¦ ¦ + c + d ¦ ¦ ¦ ¦ +-----+ +->-+ ¦ S33 ¦ +-----+ ¦ |  |  | Directional arrows: When necessary for clarity, the "less than" (<) character of the character set defined in 2.1.1 can be used to indicate right-to-left control flow, and the "greater than" (>) character to represent left-to-right control flow. When this feature is used, the corresponding character shall be located between two "-" characters, that is, in the character sequence "-<-" or "->-" as shown in the accompanying example. |  |  |

| Figure 17 - Examples of SFC evolution rules |  |
| --- | --- |
| a) Transition not enabled ( X = Don't care) | ¦ ¦ ¦ ¦ +------+ +-----+ +------+ +------+ ¦STEP10¦ ¦STEP9¦ ¦STEP13¦ ¦STEP22¦ ¦ ¦ ¦ ¦ ¦ * ¦ ¦ * ¦ +------+ +-----+ +------+ +------+ ¦ ¦ ¦ ¦ + X ====+========+=========+==== ¦ ¦ +------+ + X ¦STEP11¦ ¦ ¦ ¦ ====+====+===+==== +------+ ¦ ¦ ¦ +------+ +------+ ¦STEP15¦ ¦STEP16¦ ¦ ¦ ¦ ¦ +------+ +------+ ¦ ¦ |
| b) Transition enabled but not cleared ( X = 0 ) | ¦ ¦ ¦ ¦ +------+ +-----+ +------+ +------+ ¦STEP10¦ ¦STEP9¦ ¦STEP13¦ ¦STEP22¦ ¦ * ¦ ¦ * ¦ ¦ * ¦ ¦ * ¦ +------+ +-----+ +------+ +------+ ¦ ¦ ¦ ¦ + X ===+========+=========+==== ¦ ¦ +------+ + X ¦STEP11¦ ¦ ¦ ¦ ====+====+====+==== +------+ ¦ ¦ ¦ +------+ +------+ ¦STEP15¦ ¦STEP16¦ ¦ ¦ ¦ ¦ +------+ +------+ ¦ ¦ |
| c) Transition cleared ( X = 1 ) | ¦ ¦ ¦ ¦ +------+ +-----+ +------+ +------+ ¦STEP10¦ ¦STEP9¦ ¦STEP13¦ ¦STEP22¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +------+ +-----+ +------+ +------+ ¦ ¦ ¦ ¦ + X ====+========+=========+==== ¦ ¦ +------+ + X ¦STEP11¦ ¦ ¦ * ¦ ====+====+===+==== +------+ ¦ ¦ ¦ +------+ +------+ ¦STEP15¦ ¦STEP16¦ ¦ * ¦ ¦ * ¦ +------+ +------+ ¦ ¦ |

NOTE	In Figure 17, the active state of a step is indicated by the presence of an asterisk (*) in the corresponding block.  This notation is used for illustration only, and is not a required language feature.

+----------------------+ ¦ ¦ ¦ +=====+ ¦ ¦¦ A ¦¦ ¦ +=====+ ¦ ¦ ¦ + t1 ¦ ¦ ¦ ======+==========+============+======= ¦ ¦ ¦ ¦ +-----+ +-----+ ¦ ¦ B ¦ ¦ C ¦ ¦ +-----+ +-----+ ¦ ¦ ¦ ¦ ¦ *--------+ ¦ ¦ ¦ ¦ ¦ ¦ + t2 + t3 ¦ ¦ ¦ ¦ ¦ ¦ +---+ +---+ ¦ ¦ ¦ D ¦ ¦ E ¦ ¦ ¦ +---+ +---+ ¦ ¦ ¦ ¦ ¦ ===+==========+============+=== ¦ ¦ ¦ ¦ ¦ + t4 + t5 ¦ ¦ ¦ ¦ +---+ +---+ ¦ ¦ F ¦ ¦ G ¦ ¦ +---+ +---+ ¦ ¦ ¦ ¦ + t6 + t7 ¦ ¦ ¦ +----------------------+---------------------+

Figure 18a - Examples of SFC errors: an "unsafe" SFC

(see 2.6.5)

+----------------------+ ¦ ¦ ¦ +=====+ ¦ ¦¦ A ¦¦ ¦ +=====+ ¦ ¦ ¦ + t1 ¦ ¦ ¦ ======+==========+============+======= ¦ ¦ ¦ ¦ +-----+ +-----+ ¦ ¦ B ¦ ¦ C ¦ ¦ +-----+ +-----+ ¦ ¦ ¦ ¦ ¦ *--------+ ¦ ¦ ¦ ¦ ¦ ¦ + t2 + t3 ¦ ¦ ¦ ¦ ¦ ¦ +---+ +---+ ¦ ¦ ¦ D ¦ ¦ E ¦ ¦ ¦ +---+ +---+ ¦ ¦ ¦ ¦ ¦ ===+==========+============+=== ¦ ¦ ¦ ¦ ¦ + t4 + t5 ¦ ¦ ¦ ¦ +---+ +---+ ¦ ¦ F ¦ ¦ G ¦ ¦ +---+ +---+ ¦ ¦ ¦ ¦ ====+==========+==========+=== ¦ ¦ ¦ + t6 ¦ ¦ +---------------------------------+

Figure 18b - Examples of SFC errors: an "unreachable" SFC

(see 2.6.5)

### 2.6.6  Compatibility of SFC elements

SFCs can be represented graphically or textually, utilizing the elements defined above.  Table 47 summarizes for convenience those elements which are mutually compatible for graphical and textual representation, respectively.

Table 47 - Compatible SFC features

| Table | Graphical representation | Textual representation |
| --- | --- | --- |
| 40 | 1, 3a, 3b, 4 | 2, 3a, 4 |
| 41 | 1,2,3,4,4a,4b,7,7a,7b | 5, 6, 7c, 7d |
| 42 | 1, 2l, 2s, 2f | 3s,3i |
| 43 | 1, 2, 4 | 3 |
| 44 | 1 to 9 | -- |
| 45 | 1 to 10 | 1 to 10 (textual equivalent) |
| 46 | 1 to 7 | 1 to 6 |
| 57 | All | -- |

### 2.6.7  SFC Compliance requirements

In order to claim compliance with the requirements of 2.6, the elements shown in table 48 shall be supported and the compatibility requirements defined in 2.6.6 shall be observed.

Table 48 - SFC minimal compliance requirements

| Table | Graphical representation | Textual representation |
| --- | --- | --- |
| 40 | 1 | 2 |
| 41 | 1 or 2 or 3 or (4 and (4a or 4b)) or (7 and (7a or 7b or 7c or 7d)) | 5 or 6 |
| 42 | 1 or 2l or 2f | 3s or 3i |
| 43 | 1 or 2 or 4 | 3 |
| 45 | 1 or 2 | 1 or 2 |
| 46 | 1 and (2a or 2b or 2c) and 3 and 4 | Same (textual equivalent) |
| 57 | (1 or 2) and (3 or 4) and (5 or 6) and (7 or 8) and (9 or 10) and (11 or 12) | Not required |

## 2.7  Configuration elements

As described in 1.4.1, a configuration consists of resources, tasks (which are defined within resources), global variables, access paths and instance specific initializations.  Each of these elements is defined in detail in this subclause.

A graphic example of a simple configuration is shown in figure 19a. Skeleton declarations for the corresponding function blocks and programs are given in figure 19b.  This figure serves as a reference point for the examples of configuration elements given in the remainder of this subclause such as in figure 20.

Figure 19a - Graphical example of a configuration

| FUNCTION_BLOCK A VAR_OUTPUT y1 : UINT ; y2 : BYTE ; END_VAR END_FUNCTION_BLOCK | FUNCTION_BLOCK B VAR_INPUT b1 : UINT ; b2 : BYTE ; END_VAR END_FUNCTION_BLOCK |
| --- | --- |
| FUNCTION_BLOCK C VAR_OUTPUT c1 : BOOL ; END_VAR VAR C2 AT %Q*: BYTE; C3: INT; END_VAR END_FUNCTION_BLOCK | FUNCTION_BLOCK D VAR_INPUT d1 : BOOL ; END_VAR VAR_OUTPUT y2 : INT ; END_VAR END_FUNCTION_BLOCK |
| PROGRAM F VAR_INPUT x1 : BOOL ; x2 : UINT ; END_VAR VAR_OUTPUT y1 : BYTE ; END_VAR VAR COUNT: INT; TIME1: TON; END_VAR END_PROGRAM |  |
| PROGRAM G VAR_OUTPUT out1 : UINT ; END_VAR VAR_EXTERNAL z1 : BYTE ; END_VAR VAR FB1 : A ; FB2 : B ; END_VAR FB1(...); out1 := FB1.y1; z1 := FB1.y2; FB2(b1 := FB1.y1, b2 := FB1.y2) ; END_PROGRAM |  |
| PROGRAM H VAR_OUTPUT HOUT1: INT ; END_VAR VAR FB1 : C ; FB2 : D ; END_VAR FB1(...) ; FB2(...); HOUT1 := FB2.y2; END_PROGRAM |  |

Figure 19b - Skeleton function block and program declarations for configuration example

### 2.7.1  Configurations, resources, and access paths

Table 49 enumerates the language features for declaration of configurations, resources, global variables, access paths and instance specific initializations.  Partial enumeration of TASK declaration features is also given; additional information on tasks is provided in 2.7.2.  The formal syntax for these features is given in B.1.7.  Figure 20 provides examples of these features, corresponding to the example configuration shown in figure 19a and the supporting declarations in figure 19b.

The ON qualifier in the RESOURCE...ON...END_RESOURCE construction is used to specify the type of "processing function" and its "man-machine interface" and "sensor and actuator interface" functions upon which the resource and its associated programs and tasks are to be implemented.  The manufacturer shall supply an implementation-dependent resource library of such functions,  as illustrated in figure 3. Associated with each element in this library shall be an identifier (the resource type name) for use in resource declaration.

NOTE	The RESOURCE...ON...END_RESOURCE construction is not required in a configuration with a single resource.  See the production single_resource_declaration in Annex B.1.7 for the syntax to be used in this case.

The scope of a VAR_GLOBAL declaration shall be limited to the configuration or resource in which it is declared, with the exception that an access path can be declared to a global variable in a resource using feature 10d in table 49.

The VAR_ACCESS...END_VAR construction provides a means of specifying variable names which can be used for remote access by some of the communication services specified in IEC 61131-5.  An access path associates each such variable name with a global variable, a directly represented variable as defined in 2.4.1.1, or any input, output, or internal variable of a program or function block.

The association shall be accomplished by qualifying the name of the variable with the complete hierarchical concatenation of instance names, beginning with the name of the resource (if any), followed by the name of the program instance (if any), followed by the name(s) of the function block instance(s) (if any). The name of the variable is concatenated at the end of the chain. All names in the concatenation shall be separated by dots. If such a variable is a multi-element variable (structure or array), an access path can also be specified to an element of the variable.

It shall not be possible to define access paths to variables that are declared in VAR_TEMP, VAR_EXTERNAL or VAR_IN_OUT declarations.

The direction of the access path can be specified as READ_WRITE or READ_ONLY, indicating that the communication services can both read and modify the value of the variable in the first case, or read but not modify the value in the second case. If no direction is specified, the default direction is READ_ONLY.

Access to variables that are declared CONSTANT or to function block inputs that are externally connected to other variables shall be READ_ONLY.

NOTE	The effect of using READ_WRITE access to function block output variables is implementation-dependent.

The VAR_CONFIG...END_VAR construction provides a means to assign instance specific locations to symbolically represented variables, which are nominated for the respective purpose by using the asterisk notation described in 2.4.1.1 and 2.4.3.1, respectively, or to assign instance specific initial values to symbolically represented variables, or both.

The assignment shall be accomplished by qualifying the name of the object to be located or initialized with the complete hierarchical concatenation of instance names, beginning with the name of the resource (if any), followed by the name of the program instance, followed by the name(s) of the function block instance(s) (if any). The name of the object to be located or initialized is concatenated at the end of the chain. All names in the concatenation shall be separated by dots. The location assignment or the initial value assignment follows the syntax and the semantics described in 2.4.3.1 and 2.4.3.2 respectively.

Instance specific initial values provided by the VAR_CONFIG...END_VAR construction always override type specific initial values. It shall not be possible to define instance specific initializations to variables which are declared in VAR_TEMP, VAR_EXTERNAL, VAR CONSTANT or VAR_IN_OUT declarations.

| Table 49 - Configuration and resource declaration features |  |
| --- | --- |
| No. | Description |
| 1 | CONFIGURATION...END_CONFIGURATION construction |
| 2 | VAR_GLOBAL...END_VAR construction within CONFIGURATION |
| 3 | RESOURCE...ON...END_RESOURCE construction |
| 4 | VAR_GLOBAL...END_VAR construction within RESOURCE |
| 5a | Periodic TASK construction(see NOTE 1) |
| 5b | Non-periodic TASK construction (see NOTE 1) |
| 6a | WITH construction for PROGRAM to TASK association (see NOTE 1) |
| 6b | WITH construction for Function Block to TASK association (see NOTE 1) |
| 6c | PROGRAM declaration with no TASK association (see NOTE 1) |
| 7 | Declaration of directly represented variables in VAR_GLOBAL (see NOTE 2) |
| 8a | Connection of directly represented variables to PROGRAM inputs |
| 8b | Connection of GLOBAL variables to PROGRAM inputs |
| 9a | Connection of PROGRAM outputs to directly represented variables |
| 9b | Connection of PROGRAM outputs to GLOBAL variables |
| 10a | VAR_ACCESS...END_VAR construction |
| 10b | Access paths to directly represented variables |
| 10c | Access paths to PROGRAM inputs |
| 10d | Access paths to GLOBAL variables in RESOURCEs |
| 10e | Access paths to GLOBAL variables in CONFIGURATIONs |
| 10f | Access paths to PROGRAM outputs |
| 10g | Access paths to PROGRAM internal variables |
| 10h | Access paths to function block inputs |
| 10i | Access paths to function block outputs |
| 11 | VAR_CONFIG...END_VAR constructiona |
| 12a | VAR_GLOBAL CONSTANT in RESOURCE declarations |
| 12b | VAR_GLOBAL CONSTANT in CONFIGURATION declarations |
| 13a | VAR_EXTERNAL in RESOURCE declarations |
| 13b | VAR_EXTERNAL CONSTANT in RESOURCE declarations |
| NOTE 1 See 2.7.2 for further descriptions of TASK features. NOTE 2 See 2.4.3.1 for further descriptions of related features. |  |
| a This feature shall be supported if feature No. 10 in table 15 is supported. |  |

| Figure 20 - Examples of CONFIGURATION and RESOURCE declaration features |  |
| --- | --- |
| No. | Example |
| 1 | CONFIGURATION CELL_1 |
| 2 | VAR_GLOBAL w: UINT; END_VAR |
| 3 | RESOURCE STATION_1 ON PROCESSOR_TYPE_1 |
| 4 | VAR_GLOBAL z1: BYTE; END_VAR |
| 5a | TASK SLOW_1(INTERVAL := t#20ms, PRIORITY := 2) ; |
| 5a | TASK FAST_1(INTERVAL := t#10ms, PRIORITY := 1) ; |
| 6a 8a | PROGRAM P1 WITH SLOW_1 : F(x1 := %IX1.1) ; |
| 9b | PROGRAM P2 : G(OUT1 => w, |
| 6b | FB1 WITH SLOW_1, |
| 6b | FB2 WITH FAST_1) ; |
| 3 | END_RESOURCE |
| 3 | RESOURCE STATION_2 ON PROCESSOR_TYPE_2 |
| 4 | VAR_GLOBAL z2 : BOOL ; |
| 7 | AT %QW5 : INT ; |
| 4 | END_VAR |
| 5a | TASK PER_2(INTERVAL := t#50ms, PRIORITY := 2) ; |
| 5b | TASK INT_2(SINGLE := z2, PRIORITY := 1) ; |
| 6a 8b | PROGRAM P1 WITH PER_2 : F(x1 := z2, x2 := w) ; |
| 6a 9a | PROGRAM P4 WITH INT_2 : H(HOUT1 => %QW5, |
| 6b | FB1 WITH PER_2); |
| 3 | END_RESOURCE |
| 10a | VAR_ACCESS |
| 10b | ABLE : STATION_1.%IX1.1 : BOOL READ_ONLY ; |
| 10c | BAKER : STATION_1.P1.x2 : UINT READ_WRITE ; |
| 10d | CHARLIE : STATION_1.z1 : BYTE ; |
| 10e | DOG : w : UINT READ_ONLY ; |
| 10f | ALPHA : STATION_2.P1.y1 : BYTE READ_ONLY ; |
| 10f | BETA : STATION_2.P4.HOUT1 : INT READ_ONLY ; |
| 10d | GAMMA : STATION_2.z2 : BOOL READ_WRITE ; |
| 10g | S1_COUNT : STATION_1.P1.COUNT : INT; |
| 10h | THETA : STATION_2.P4.FB2.d1 : BOOL READ_WRITE; |
| 10i | ZETA : STATION_2.P4.FB1.c1 : BOOL READ_ONLY; |
| 10k | OMEGA : STATION_2.P4.FB1.C3 : INT READ_WRITE; |
| 10a | END_VAR |
| 11 | VAR_CONFIG STATION_1.P1.COUNT : INT := 1; STATION_2.P1.COUNT : INT := 100; STATION_1.P1.TIME1 : TON := (PT := T#2.5s); STATION_2.P1.TIME1 : TON := (PT := T#4.5s); STATION_2.P4.FB1.C2 AT %QB25 : BYTE; END_VAR |
| 1 | END_CONFIGURATION |
| NOTE 1 Graphical and semigraphic representation of these features is allowed but is beyond the scope of this Part of IEC 61131. NOTE 2 It is an error if the data type declared for a variable in a VAR_ACCESS statement is not the same as the data type declared for the variable elsewhere, e.g., if variable BAKER is declared of type WORD in the above examples. |  |

### 2.7.2  Tasks

For the purposes of IEC 61131-3, a task is defined as an execution control element which is capable of invoking, either on a periodic basis or upon the occurrence of the rising edge of a specified Boolean variable, the execution of a set of program organization units, which can include programs and function blocks whose instances are specified in the declaration of programs.

The maximum number of tasks per resource and task interval resolution are implementation-dependent parameters.

Tasks and their association with program organization units can be represented graphically or textually using the WITH construction, as shown in table 50, as part of resources within configurations.  A task is implicitly enabled or disabled by its associated resource according to the mechanisms defined in 1.4.1.  The control of program organization units by enabled tasks shall conform to the following rules:

1) The associated program organization units shall be scheduled for execution upon each rising edge of the SINGLE input of the task.

2) If the INTERVAL input is non-zero, the associated program organization units shall be scheduled for execution periodically at the specified interval as long as the SINGLE input stands at zero (0).  If the INTERVAL input is zero (the default value), no periodic scheduling of the associated program organization units shall occur.

3) The PRIORITY input of a task establishes the scheduling priority of the associated program organization units, with zero (0) being highest priority and successively lower priorities having successively higher numeric values.  As shown in table 50, the priority of a program organization unit (that is, the priority of its associated task) can be used for preemptive or non-preemptive scheduling.

a) In non-preemptive scheduling, processing power becomes available on a resource when execution of a program organization unit or operating system function is complete.  When processing power is available, the program organization unit with highest scheduled priority shall begin execution.  If more than one program organization unit is waiting at the highest scheduled priority, then the program organization unit with the longest waiting time at the highest scheduled priority shall be executed.

b) In preemptive scheduling, when a program organization unit is scheduled, it can interrupt the execution of a program organization unit of lower priority on the same resource, that is, the execution of the lower-priority unit can be suspended until the execution of the higher-priority unit is completed.  A program organization unit shall not interrupt the execution of another unit of the same or higher priority.

NOTE	Depending on schedule priorities, a program organization unit might not begin execution at the instant it is scheduled.  However, in the examples shown in table 50, all program organization units meet their deadlines, that is, they all complete execution before being scheduled for re-execution.  The manufacturer shall provide information to enable the user to determine whether all deadlines will be met in a proposed configuration.

4) A program with no task association shall have the lowest system priority.  Any such program shall be scheduled for execution upon "starting" of its  resource, as defined in 1.4.1, and shall be re-scheduled for execution  as soon as its execution terminates.

5) When a function block instance is associated with a task, its execution shall be under the exclusive control of the task, independent of the rules of evaluation of the program organization unit in which the task-associated function block instance is declared.

6) Execution of a function block instance which is not directly associated with a task shall follow the normal rules for the order of evaluation of language elements for the program organization unit (which can itself be under the control of a task) in which the function block instance is declared.

7) The execution of function blocks within a program shall be synchronized to ensure that data concurrency is achieved according to the following rules:

a) If a function block receives more than one input from another function block, then when the former is executed, all inputs from the latter shall represent the results of the same evaluation.  For instance, in the example represented by figure 21a, when Y2 is evaluated, the inputs Y2.A and Y2.B shall represent the outputs Y1.C and Y1.D from the same (not two different) evaluations of Y1.

b) If two or more function blocks receive inputs from the same function block, and if the "destination" blocks are all explicitly or implicitly associated with the same task, then the inputs to all such "destination" blocks at the time of their evaluation shall represent the results of the same evaluation of the "source" block.  For instance, in the example represented by figures 21b and 21c, when Y2 and Y3 are evaluated in the normal course of evaluating program P1, the inputs Y2.A and Y2.B shall be the results of the same evaluation of Y1 as the inputs Y3.A and Y3.B.

Provision shall be made for storage of the outputs of functions or function blocks which have explicit task associations, or which are used as inputs to program organization units which have explicit task associations, as necessary to satisfy the rules given above.

It shall be an error in the sense of subclause 1.5.1 if a task fails to be scheduled or to meet its execution deadline because of excessive resource requirements or other task scheduling conflicts.

| Table 50 - Task features |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| No. |  | Description/Examples |  |  |  |  |
| 1a |  | Textual declaration of periodic TASK (feature 5a of table 49) |  |  |  |  |
| 1b |  | Textual declaration of non-periodic TASK (feature 5b of table 49) |  |  |  |  |
| Graphical representation of TASKs (general form) |  |  |  |  |  |  |
| TASKNAME +---------+ ¦ TASK ¦ BOOL---¦SINGLE ¦ TIME---¦INTERVAL ¦ UINT---¦PRIORITY ¦ +---------+ |  |  |  |  |  |  |
| 2a |  | Graphical representation of periodic TASKs |  |  |  |  |
|  |  | SLOW_1 FAST_1 +---------+ +---------+ ¦ TASK ¦ ¦ TASK ¦ ¦SINGLE ¦ ¦SINGLE ¦ t#20ms---¦INTERVAL ¦ t#10ms---¦INTERVAL ¦ 2---¦PRIORITY ¦ 1---¦PRIORITY ¦ +---------+ +---------+ |  |  |  |  |
| 2b |  | Graphical representation of non-periodic TASK |  |  |  |  |
|  |  | INT_2 +---------+ ¦ TASK ¦ %IX2---¦SINGLE ¦ ¦INTERVAL ¦ 1---¦PRIORITY ¦ +---------+ |  |  |  |  |
| 3a |  | Textual association with PROGRAMs (feature 6a of table 49) |  |  |  |  |
| 3b |  | Textual association with function blocks (feature 6b of table 49) |  |  |  |  |
| 4a |  | Graphical association with PROGRAMs |  |  |  |  |
|  |  | RESOURCE STATION_2 P1 P4 +-------+ +-------+ ¦ F ¦ ¦ H ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +-------+ +-------+ ¦ PER_2 ¦ ¦ INT_2 ¦ +-------+ +-------+ END_RESOURCE |  |  |  |  |
| 4b |  | Graphical association with function blocks within PROGRAMs |  |  |  |  |
|  |  | RESOURCE STATION_1 P2 +---------------------------------------------------+ ¦ G ¦ ¦ ¦ ¦ FB1 FB2 ¦ ¦ +------+ +------+ ¦ ¦ ¦ A ¦ ¦ B ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ +------+ +------+ ¦ ¦ ¦SLOW_1¦ ¦FAST_1¦ ¦ ¦ +------+ +------+ ¦ +---------------------------------------------------+ END_RESOURCE |  |  |  |  |
| 5a |  | Non-preemptive scheduling |  |  |  |  |
|  |  | EXAMPLE 1: |  |  |  |  |
|  |  | - RESOURCE STATION_1 as configured in figure 20 |  |  |  |  |
|  |  | - Execution times: P1 = 2 ms; P2 = 8 ms; - P2.FB1 = P2.FB2 = 2 ms (see NOTE 3) |  |  |  |  |
|  |  | - STATION_1 starts at t = 0 |  |  |  |  |
|  |  | SCHEDULE (repeats every 40 ms) |  |  |  |  |
|  |  | t(ms) |  | Executing | Waiting |  |
|  |  | 0 |  | P2.FB2@1 | P1@2, P2.FB1@2, P2 |  |
|  |  | 2 |  | P1@2 | P2.FB1@2, P2 |  |
|  |  | 4 |  | P2.FB1@2 | P2 |  |
|  |  | 6 |  | P2 |  |  |
|  |  | 10 |  | P2 | P2.FB2@1 |  |
|  |  | 14 |  | P2.FB2@1 | P2 |  |
|  |  | 16 |  | P2 | (P2 restarts) |  |
|  |  | 20 |  | P2 | P2.FB2@1, P1@2, P2.FB1@2 |  |
|  |  | 24 |  | P2.FB2@1 | P1@2, P2.FB1@2, P2 |  |
|  |  | 26 |  | P1@2 | P2.FB1@2, P2 |  |
|  |  | 28 |  | P2.FB1@2 | P2 |  |
|  |  | 30 |  | P2.FB2@1 | P2 |  |
|  |  | 32 |  | P2 |  |  |
|  |  | 40 |  | P2.FB2@1 | P1@2, P2.FB1@2, P2 |  |
| 5a |  | Non-preemptive scheduling |  |  |  |  |
|  |  | EXAMPLE 2: - RESOURCE STATION_2 as configured in figure 20 - Execution times: P1 = 30 ms, P4 = 5 ms, P4.FB1 = 10 ms (see NOTE 4) - INT_2 is triggered at t = 25, 50, 90, ... ms - STATION_2 starts at t = 0 |  |  |  |  |
|  |  | SCHEDULE |  |  |  |  |
|  |  | t(ms) |  | Executing | Waiting |  |
|  |  | 0 |  | P1@2 | P4.FB1@2 |  |
|  |  | 25 |  | P1@2 | P4.FB1@2, P4@1 |  |
|  |  | 30 |  | P4@1 | P4.FB1@2 |  |
|  |  | 35 |  | P4.FB1@2 |  |  |
|  |  | 50 |  | P4@1 | P1@2, P4.FB1@2 |  |
|  |  | 55 |  | P1@2 | P4.FB1@2 |  |
|  |  | 85 |  | P4.FB1@2 |  |  |
|  |  | 90 |  | P4.FB1@2 | P4@1 |  |
|  |  | 95 |  | P4@1 |  |  |
|  |  | 100 |  | P1@2 | P4.FB1@2 |  |
| 5b |  | Preemptive scheduling |  |  |  |  |
|  |  | EXAMPLE 3: - RESOURCE STATION_1 as configured in figure 20 - Execution times: P1 = 2 ms; P2 = 8 ms; P2.FB1 = P2.FB2 = 2 ms ( see NOTE 3) - STATION_1 starts at t = 0 |  |  |  |  |
|  |  | SCHEDULE |  |  |  |  |
|  |  | t(ms) |  | Executing | Waiting |  |
|  |  | 0 |  | P2.FB2@1 | P1@2, P2.FB1@2, P2 |  |
|  |  | 2 |  | P1@2 | P2.FB1@2, P2 |  |
|  |  | 4 |  | P2.FB1@2 | P2 |  |
|  |  | 6 |  | P2 |  |  |
|  |  | 10 |  | P2.FB2@1 | P2 |  |
|  |  | 12 |  | P2 |  |  |
|  |  | 16 |  | P2 | (P2 restarts) |  |
|  |  | 20 |  | P2.FB2@1 | P1@2, P2.FB1@2, P2 |  |
| 5b | Preemptive scheduling |  |  |  |  |  |
|  | EXAMPLE 4: - RESOURCE STATION_2 as configured in figure 20 - Execution times: P1 = 30 ms, P4 = 5 ms, P4.FB1 = 10 ms (NOTE 4) - INT_2 is triggered at t = 25, 50, 90, ... ms - STATION_2 starts at t = 0 |  |  |  |  |  |
|  | SCHEDULE |  |  |  |  |  |
|  | t(ms) |  | Executing |  |  | Waiting |
|  | 0 |  | P1@2 |  |  | P4.FB1@2 |
|  | 25 |  | P4@1 |  |  | P1@2, P4.FB1@2 |
|  | 30 |  | P1@2 |  |  | P4.FB1@2 |
|  | 35 |  | P4.FB1@2 |  |  |  |
|  | 50 |  | P4@1 |  |  | P1@2, P4.FB1@2 |
|  | 55 |  | P1@2 |  |  | P4.FB1@2 |
|  | 85 |  | P4.FB1@2 |  |  |  |
|  | 90 |  | P4@1 |  |  | P4.FB1@2 |
|  | 95 |  | P4.FB1@2 |  |  |  |
|  | 100 |  | P1@2 |  |  | P4.FB1@2 |
| NOTE 1 Details of RESOURCE and PROGRAM declarations are not shown; see 2.7 and 2.7.1. NOTE 2 The notation X@Y indicates that program organization unit X is scheduled or executing at priority Y. NOTE 3 The execution times of P2.FB1 and P2.FB2 are not included in the execution time of P2. NOTE 4 The execution time of P4.FB1 is not included in the execution time of P4. |  |  |  |  |  |  |

|  | RESOURCE R1 fast1 +----------+ ¦ TASK ¦ t#10ms---¦INTERVAL ¦ 1---¦PRIORITY ¦ +----------+ |  | slow1 +----------+ ¦ TASK ¦ t#20ms---¦INTERVAL ¦ 2---¦PRIORITY ¦ +----------+ |  |
| --- | --- | --- | --- | --- |
|  |  | P1 |  |  |
|  |  | PROGRAM X Y1 Y2 +-----+ +-----+ ¦ Y ¦ ¦ Y ¦ ---¦A C¦----+--------¦A C¦--- ---¦B D¦----¦--+-----¦B D¦--- +-----+ ¦ ¦ +-----+ ¦slow1¦ ¦ ¦ ¦fast1¦ +-----+ ¦ ¦ +-----+ ¦ ¦ ¦ ¦ Y3 ¦ ¦ +-----+ ¦ ¦ ¦ Y ¦ +--¦--¦A C¦--- +--¦B D¦--- +-----+ ¦fast1¦ +-----+ END_PROGRAM |  |  |

Figure 21a - Synchronization of function blocks with explicit task associations

|  | RESOURCE R1 fast1 +----------+ ¦ TASK ¦ t#10ms---¦INTERVAL ¦ 1---¦PRIORITY ¦ +----------+ |  | slow1 +----------+ ¦ TASK ¦ t#20ms---¦INTERVAL ¦ 2---¦PRIORITY ¦ +----------+ |  |
| --- | --- | --- | --- | --- |
|  |  | P1 |  |  |
|  |  | PROGRAM X Y1 Y2 +-----+ +-----+ ¦ Y ¦ ¦ Y ¦ ---¦A C¦----+--------¦A C¦--- ---¦B D¦----¦--+-----¦B D¦--- +-----+ ¦ ¦ +-----+ ¦fast1¦ ¦ ¦ +-----+ ¦ ¦ ¦ ¦ ¦ ¦ Y3 ¦ ¦ +-----+ ¦ ¦ ¦ Y ¦ +--¦--¦A C¦--- +--¦B D¦--- +-----+ END_PROGRAM |  |  |
|  |  | slow1 |  |  |

Figure 21b - Synchronization of function blocks with implicit task associations

|  | RESOURCE R1 fast1 +----------+ ¦ TASK ¦ t#10ms---¦INTERVAL ¦ 1---¦PRIORITY ¦ +----------+ |  | slow1 +----------+ ¦ TASK ¦ t#20ms---¦INTERVAL ¦ 2---¦PRIORITY ¦ +----------+ |  |
| --- | --- | --- | --- | --- |
|  |  | P1 |  |  |
|  |  | PROGRAM X Y1 Y2 +-----+ +-----+ ¦ Y ¦ ¦ Y ¦ ---¦A C¦----+--------¦A C¦--- ---¦B D¦----¦--+-----¦B D¦--- +-----+ ¦ ¦ +-----+ ¦fast1¦ ¦ ¦ ¦slow1¦ +-----+ ¦ ¦ +-----+ ¦ ¦ ¦ ¦ Y3 ¦ ¦ +-----+ ¦ ¦ ¦ Y ¦ +--¦--¦A C¦--- +--¦B D¦--- +-----+ ¦slow1¦ +-----+ END_PROGRAM |  |  |

Figure 21c - Explicit task associations equivalent to figure 21b

# 3.  Textual languages

The textual languages defined in this standard are IL (Instruction List) and ST (Structured Text).  The sequential function chart (SFC) elements defined in 2.6 can be used in conjunction with either of these languages.

## 3.1  Common elements

The textual elements specified in clause 2 shall be common to the textual languages (IL and ST) defined in this clause.  In particular, the following program structuring elements shall be common to textual languages:

| TYPE...END_TYPE | (2.3.3) |
| --- | --- |
| VAR...END_VAR | (2.4.3) |
| VAR_INPUT...END_VAR | (2.4.3) |
| VAR_OUTPUT...END_VAR | (2.4.3) |
| VAR_IN_OUT...END_VAR | (2.4.3) |
| VAR_EXTERNAL...END_VAR | (2.4.3) |
| VAR_TEMP...END_VAR | (2.4.3) |
| VAR_ACCESS...END_VAR | (2.4.3) |
| VAR_GLOBAL...END_VAR | (2.4.3) |
| VAR_CONFIG...END_VAR | (2.4.3) |
| FUNCTION ... END_FUNCTION | (2.5.1.3) |
| FUNCTION_BLOCK...END_FUNCTION_BLOCK | (2.5.2.2) |
| PROGRAM...END_PROGRAM | (2.5.3) |
| STEP...END_STEP | (2.6.2) |
| TRANSITION...END_TRANSITION | (2.6.3) |
| ACTION...END_ACTION | (2.6.4) |

## 3.2  Instruction list (IL)

This subclause defines the semantics of the IL (Instruction List) language whose formal syntax is given in B.2.

### 3.2.1  Instructions

As illustrated in table 51, an instruction list is composed of a sequence of instructions.  Each instruction shall begin on a new line and shall contain an operator with optional modifiers, and, if necessary for the particular operation, one or more operands separated by commas.  Operands can be any of the data representations defined in 2.2 for literals, in 2.3.3 for enumerated values, and in 2.4 for variables.

The instruction can be preceded by an identifying label followed by a colon (:).Empty lines can be inserted between instructions.

Table 51a - Examples of instruction fields

| LABEL | OPERATOR | OPERAND | COMMENT |
| --- | --- | --- | --- |
| START: | LD | %IX1 | (* PUSH BUTTON *) |
|  | ANDN | %MX5 | (* NOT INHIBITED *) |
|  | ST | %QX2 | (* FAN ON *) |

### 3.2.2  Operators, modifiers and operands

Standard operators with their allowed modifiers and operands shall be as listed in table 52.  The typing of operators shall conform to the conventions of 2.5.1.4.

Unless otherwise defined in table 52, the semantics of the operators shall be

result := result OP operand

That is, the value of the expression being evaluated is replaced by its current value operated upon by the operator with respect to the operand.  For instance, the instruction AND %IX1 is interpreted as

result := result AND %IX1

The comparison operators shall be interpreted with the current result to the left of the comparison and the operand to the right, with a Boolean result.  For instance, the instruction "GT %IW10" will have the Boolean result 1 if the current result is greater than the value of Input Word 10, and the Boolean result 0 otherwise.

The modifier "N" indicates bitwise Boolean negation (one's complement) of the operand.  For instance, the instruction ANDN %IX2 is interpreted as

result := result AND NOT %IX2

It shall be an error in the sense of subclause 1.5.1 if the current result and operand are not of same data type, or if the result of a numerical operation exceeds the range of values for its data type.

The left parenthesis modifier "(" indicates that evaluation of the operator shall be deferred until a right parenthesis operator ")" is encountered. In table 51b two equivalent forms of a parenthesized sequence of instructions are shown. Both features in table 51b shall be interpreted as

result := result AND (%IX1 OR %IX2)

Table 51b - Parenthesized expression features for IL language

| No. | DESCRIPTION/EXAMPLE |
| --- | --- |
| 1 | Parenthesized expression beginning with explicit operator: |
|  | AND( LD %IX1 (NOTE 1) OR %IX2 ) |
| 2 | Parenthesized expression (short form): |
|  | AND( %IX1 OR %IX2 ) |
| NOTE In form 1 the LD operator may be modified or the LD operation may be replaced by another operation or function invocation respectively. |  |

The modifier "C" indicates that the associated instruction shall be performed only if the value of the currently evaluated result is Boolean 1 (or Boolean 0 if the operator is combined with the "N" modifier).

| Table 52 - Instruction List operators |  |  |  |
| --- | --- | --- | --- |
| No. | OPERATORa | MODIFIERS (Note 1) | SEMANTICS |
| 1 | LD | N | Set current result equal to operand |
| 2 | ST | N | Store current result to operand location |
| 3 | Se |  | Set operand to 1 if current result is Boolean 1 |
|  | Re |  | Reset operand to 0 if current result is Boolean 1 |
| 4 | AND | N, ( | Logical AND |
| 5 | & | N, ( | Logical AND |
| 6 | OR | N, ( | Logical OR |
| 7 | XOR | N, ( | Logical Exclusive OR |
| 7a | NOTd |  | Logical Negation (one's complement) |
| 8 | ADD | ( | Addition |
| 9 | SUB | ( | Subtraction |
| 10 | MUL | ( | Multiplication |
| 11 | DIV | ( | Division |
| 11a | MOD | ( | Modulo-Division |
| 12 | GT | ( | Comparison: > |
| 13 | GE | ( | Comparison: >= |
| 14 | EQ | ( | Comparison: = |
| 15 | NE | ( | Comparison: <> |
| 16 | LE | ( | Comparison: <= |
| 17 | LT | ( | Comparison: < |
| 18 | JMPb | C, N | Jump to label |
| 19 | CALc | C, N | Call function block (See table 53) |
| 20 | RETf | C, N | Return from called function, function block or program |
| 21 | )f |  | Evaluate deferred operation |
| a Unless otherwise noted, these operators shall be either overloaded or typed as defined in 2.5.1.4 and 2.5.1.5.6. b The operand of a JMP instruction shall be the label of an instruction to which execution is to be transferred.When a JMP instruction is contained in an ACTION... END_ACTION construct, the operand shall be a label within the same construct. c The operand of this instruction shall be the name of a function block instance to be invoked. d The result of this operation shall be the bitwise Boolean negation (one's complement) of the current result. e The type of the operand of this instruction shall be BOOL. f This instruction does not have an operand. |  |  |  |
| NOTE See preceding text for explanation of modifiers and evaluation of expressions. |  |  |  |

### 3.2.3  Functions and Function Blocks

Functions as defined in 2.5.1 shall be invoked by placing the function name in the operator field. As shown in features 4 and 5 of table 53, this invocation can take one of two forms. The value returned by a function upon the successful execution of a RET instruction or upon reaching the physical end of the function shall become the "current result" described in 3.2.2.

The argument list of functions (feature 4 in table 53) is equivalent to feature 1 in table 19a . The rules and features defined in 2.5.1.1 and table 19a for function calls apply.

A non-formal input list of functions (feature 5 in table 53) is equivalent to feature 2 in table 19a . The rules and features defined in 2.5.1.1 and table 19a for function calls apply. In contrast to the examples given in table 19a for ST language, the first argument is not contained in the non-formal input list in IL , but the current result shall be used as the first argument of the function. Additional arguments (starting with the 2nd), if required, shall be given in the operand field, separated by commas, in the order of their declaration.

Function blocks as defined in 2.5.2 can be invoked conditionally and unconditionally via the CAL (Call) operator listed in table 52.  As shown in features 1a, 1b, 2 and 3 of table 53, this invocation can take one of four forms.

A formal argument list of a function block invocation (feature 1a in table 53)  is equivalent to feature 1 in table 19a . A non-formal argument list of a function block invocation (feature 1b in table 53)  is equivalent to feature 2 in table 19a . The rules and features defined in 2.5.1.1 and table 19a for function calls apply correspondingly, by replacing each occurrence of the term ‘function’ by the term ‘function block’ in these rules.

All assignments in an argument list of a conditional function block invocation shall only be performed together with the invocation, if the condition is true.

| Table 53 - Function Block invocation and Function invocation features for IL language |  |
| --- | --- |
| No. | DESCRIPTION/EXAMPLE |
| 1a | CAL of Function Block with non-formal argument list: |
|  | CAL C10(%IX10, FALSE, A, OUT, B) CAL CMD_TMR(%IX5, T#300ms, OUT, ELAPSED) |
| 1b | CAL of Function Block with formal argument list: |
|  | CAL C10( CU := %IX10, Q => OUT) CAL CMD_TMR( IN := %IX5, PT := T#300ms, Q => OUT, ET => ELAPSED, ENO => ERR) |
| 2 | CAL of Function Block with load/store of arguments (NOTE 2) |
|  | LD A ADD 5 ST C10.PV LD %IX10 ST C10.CU CAL C10 |
| 3 | Use of Function Block input operators: |
|  | LD A ADD 5 PV C10 LD %IX10 CU C10 |
| 4 | Function invocation with formal argument list: |
|  | LIMIT( EN:= COND, IN:= B, MN:= 1, MX:= 5, ENO=> TEMPL ) ST A |
| 5 | Function invocation with non-formal argument list: |
|  | LD 1 LIMIT B, 5 ST A |
| NOTE 1 A declaration such as VAR C10 : CTU; CMD_TMR : TON; A, B : INT; ELAPSED : TIME; OUT, ERR, TEMPL, COND : BOOL; END_VAR is assumed in the above examples. NOTE 2 This usage is an exception to the rule given in 2.5.2.1 that "The assignment of a value to the inputs of a function block is permitted only as part of the invocation of the function block." |  |

The input operators shown in table 54 can be used in conjunction with feature 3 of table 53. This method of invocation is equivalent to a CAL with an argument list, which contains only one variable with the name of the input operator. Arguments, which are not supplied, are taken from the last assignment or, if not present, from initialization. This feature supports problem situations, where events are predictable and therefore only one variable can change from one call to the next.

EXAMPLE 1
Together with the declaration
VAR C10: CTU; END_VAR
the instruction sequence 
	LD	15
	PV	C10
gives the same result as
	CAL	C10(PV:=15)
The missing inputs R and CU have values previously assigned to them. Since the CU input detects a rising edge, only the PV input value will be set by this call; counting cannot happen because an unsupplied argument cannot change. In contrast to this, the sequence
	LD	%IX10
	CU	C10
results in counting at maximum in every second call, depending on the change rate of the input %IX10. Every call uses the previously set values for PV and R.

EXAMPLE 2
With bistable function blocks, taking a declaration
VAR FORWARD: SR; END_VAR
this results into an implicit conditional behavior. The sequence
	LD	FALSE
	S1	FORWARD
does not change the state of the bistable FORWARD.  A following sequence
	LD	TRUE
	R	FORWARD
resets the bistable.

Table 54 - Standard Function Block input operators for IL language

| No. | Operators | FB Type | Reference |
| --- | --- | --- | --- |
| 4 | S1,R | SR | 2.5.2.3.1 |
| 5 | S,R1 | RS | 2.5.2.3.1 |
| 6 | CLK | TRIGGER | 2.5.2.3.2 |
| 8 | CU,R,PV | CTU | 2.5.2.3.3 |
| 9 | CD,PV | CTD | 2.5.2.3.3 (NOTE 1) |
| 10 | CU,CD,R,PV | CTUD | 2.5.2.3.3 (NOTE 1) |
| 11 | IN,PT | TP | 2.5.2.3.4 |
| 12 | IN,PT | TON | 2.5.2.3.4 |
| 13 | IN,PT | TOF | 2.5.2.3.4 |
| NOTE 1 LD is not necessary as a Standard Function Block input operator, because the LD functionality is included in PV. NOTE 2 The feature numbering in this table is such as to maintain consistency with IEC 61131-3, First Edition. |  |  |  |

## 3.3  Structured Text (ST)

This subclause defines the semantics of the ST (Structured Text) language whose syntax is defined in B.3.  In this language, the end of a textual line shall be treated the same as a space (SP) character, as defined in 2.1.4.

### 3.3.1  Expressions

An expression is a construct which, when evaluated, yields a value corresponding to one of the data types defined in 2.3.1 and 2.3.3.  The maximum allowed length of expressions is an implementation-dependent parameter.

Expressions are composed of operators and operands.  An operand shall be a literal as defined in 2.2, an enumerated value as defined in 2.3.3, a variable as defined in 2.4, a function invocation as defined in 2.5.1, or another expression.

The operators of the ST language are summarized in table 55.  The evaluation of an expression consists of applying the operators to the operands in a sequence defined by the operator precedence shown in table 55.  The operator with highest precedence in an expression shall be applied first, followed by the operator of next lower precedence, etc., until evaluation is complete.  Operators of equal precedence shall be applied as written in the expression from left to right.  For example, if A, B, C, and D are of type INT with values 1, 2, 3, and 4, respectively, then

A+B-C*ABS(D)

shall evaluate to -9, and

(A+B-C)*ABS(D)

shall evaluate to 0 .

When an operator has two operands, the leftmost operand shall be evaluated first.  For example, in the expression

SIN(A)*COS(B)

the expression SIN(A) shall be evaluated first, followed by COS(B), followed by evaluation of the product.

The following conditions in the execution of operators shall be treated as errors in the sense of subclause 1.5.1: 1) An attempt is made to divide by zero. 2)  Operands are not of the correct data type for the operation. 3) The result of a numerical operation exceeds the range of values for its data type.

Boolean expressions may be evaluated only to the extent necessary to determine the resultant value.  For instance, if A<=B, then only the expression (A>B) would be evaluated to determine that the value of the expression

(A>B) & (C<D)

is Boolean zero.

Functions shall be invoked as elements of expressions consisting of the function name followed by a parenthesized list of arguments, as defined in 2.5.1.1.

When an operator in an expression can be represented as one of the overloaded functions defined in 2.5.1.5, conversion of operands and results shall follow the rule and examples given in 2.5.1.4.

| Table 55 - Operators of the ST language |  |  |  |
| --- | --- | --- | --- |
| No. | Operationa | Symbol | Precedence |
| 1 | Parenthesization | (expression) | HIGHEST |
| 2 | Function evaluation | identifier(argument list) |  |
|  | EXAMPLES | LN(A), MAX(X,Y), etc. |  |
| 4 | Negation | - |  |
| 5 | Complement | NOT |  |
| 3 | Exponentiationb | ** |  |
| 6 | Multiply | * |  |
| 7 | Divide | / |  |
| 8 | Modulo | MOD |  |
| 9 | Add | + |  |
| 10 | Subtract | - |  |
| 11 | Comparison | < , > , <= , >= |  |
| 12 | Equality | = |  |
| 13 | Inequality | <> |  |
| 14 | Boolean AND | & |  |
| 15 | Boolean AND | AND |  |
| 16 | Boolean Exclusive OR | XOR |  |
| 17 | Boolean OR | OR | LOWEST |
| NOTE The feature numbering in this table is such as to maintain consistency with IEC 61131-3, First Edition. |  |  |  |
| a The same restrictions apply to the operands of these operators as to the inputs of the corresponding functions defined in 2.5.1.5. b The result of evaluating the expression A**B shall be the same as the result of evaluating the function EXPT(A,B) as defined in table 24. |  |  |  |

### 3.3.2  Statements

The statements of the ST language are summarized in table 56.  Statements shall be terminated by semicolons as specified in the syntax of B.3.  The maximum allowed length of statements is an implementation-dependent parameter.

| Table 56 - ST language statements |  |  |
| --- | --- | --- |
| No. | Statement type/Reference | Examples |
| 1 | Assignment (3.3.2.1) | A := B; CV := CV+1; C := SIN(X); |
| 2 | Function block Invocation and FB output usage (3.3.2.2) | CMD_TMR(IN:=%IX5, PT:=T#300ms) ; A := CMD_TMR.Q ; |
| 3 | RETURN (3.3.2.2) | RETURN ; |
| 4 | IF (3.3.2.3) | D := B*B - 4*A*C ; IF D < 0.0 THEN NROOTS := 0 ; ELSIF D = 0.0 THEN NROOTS := 1 ; X1 := - B/(2.0*A) ; ELSE NROOTS := 2 ; X1 := (- B + SQRT(D))/(2.0*A) ; X2 := (- B - SQRT(D))/(2.0*A) ; END_IF ; |
| 5 | CASE (3.3.2.3) | TW := BCD_TO_INT(THUMBWHEEL); TW_ERROR := 0; CASE TW OF 1,5: DISPLAY := OVEN_TEMP; 2: DISPLAY := MOTOR_SPEED; 3: DISPLAY := GROSS - TARE; 4,6..10: DISPLAY := STATUS(TW - 4); ELSE DISPLAY := 0 ; TW_ERROR := 1; END_CASE; QW100 := INT_TO_BCD(DISPLAY); |
| 6 | FOR (3.3.2.4) | J := 101 ; FOR I := 1 TO 100 BY 2 DO IF WORDS[I] = 'KEY' THEN J := I ; EXIT ; END_IF ; END_FOR ; |
| 7 | WHILE (3.3.2.4) | J := 1; WHILE J <= 100 & WORDS[J] <> 'KEY' DO J := J+2 ; END_WHILE ; |
| 8 | REPEAT (3.3.2.4) | J := -1 ; REPEAT J := J+2 ; UNTIL J = 101 OR WORDS[J] = 'KEY' END_REPEAT ; |
| 9 | EXIT (3.3.2.4)a | EXIT ; |
| 10 | Empty Statement | ; |
| a If the EXIT statement (9) is supported, then it shall be supported for all of the iteration statements (FOR, WHILE, REPEAT) which are supported in the implementation. |  |  |

#### 3.3.2.1  Assignment statements

The assignment statement replaces the current value of a single or multi-element variable by the result of evaluating an expression.  An assignment statement shall consist of a variable reference on the left-hand side, followed by the assignment operator ":=", followed by the expression to be evaluated.  For instance, the statement

A := B ;

would be used to replace the single data value of variable A by the current value of variable B if both were of type INT.  However, if both A and B were of type ANALOG_CHANNEL_CONFIGURATION as described in table 12, then the values of all the elements of the structured variable A would be replaced by the current values of the corresponding elements of variable B.

As illustrated in figure 6, the assignment statement shall also be used to assign the value to be returned by a function, by placing the function name to the left of an assignment operator in the body of the function declaration.  The value returned by the function shall be the result of the most recent evaluation of such an assignment.  It is an error to return from the evaluation of a function with an  ENO value of TRUE, or with a non-existent ENO output, unless at least one such assignment has been made.

#### 3.3.2.2  Function and function block control statements

Function and function block control statements consist of the mechanisms for invoking function blocks and for returning control to the invoking entity before the physical end of a function or function block.

Function evaluation shall be invoked as part of expression evaluation, as specified in 3.3.1.

Function blocks shall be invoked by a statement consisting of the name of the function block instance followed by a parenthesized list of arguments, as illustrated in table 56. The rules and features defined in 2.5.1.1 and Table 19a for function calls apply correspondingly, by replacing each occurrence of the term ‘function’ by the term ‘function block’ in these rules.

The RETURN statement shall provide early exit from a function , function block or program (e.g., as the result of the evaluation of an IF statement).

#### 3.3.2.3  Selection statements

Selection statements include the IF and CASE statements.  A selection statement selects one (or a group) of its component statements for execution, based on a specified condition.  Examples of selection statements are given in table 56.

The IF statement specifies that a group of statements is to be executed only if the associated Boolean expression evaluates to the value 1 (true).  If the condition is false, then either no statement is to be executed, or the statement group following the ELSE keyword (or the ELSIF keyword if its associated Boolean condition is true) is to be executed.

The CASE statement consists of an expression which shall evaluate to a variable of type ANY_INT or of an enumerated data type (the "selector"), and a list of statement groups, each group being labeled by one or more integer or enumerated values or ranges of integer values, as applicable.  It specifies that the first group of statements, one of whose ranges contains the computed value of the selector, shall be executed .  If the value of the selector does not occur in a range of any case, the statement sequence following the keyword ELSE (if it occurs in the CASE statement) shall be executed.  Otherwise, none of the statement sequences shall be executed.

The maximum allowed number of selections in CASE statements is an implementation-dependent parameter.

#### 3.3.2.4  Iteration statements

Iteration statements specify that the group of associated statements shall be executed repeatedly.  The FOR statement is used if the number of iterations can be determined in advance; otherwise, the WHILE or REPEAT constructs are used.

The EXIT statement shall be used to terminate iterations before the termination condition is satisfied.

When the EXIT statement is located within nested iterative constructs, exit shall be from the innermost loop in which the EXIT is located, that is, control shall pass to the next statement after the first loop terminator (END_FOR, END_WHILE, or END_REPEAT) following the EXIT statement. For instance, after executing the statements shown in figure 22, the value of the variable SUM shall be 15 if the value of the Boolean variable FLAG is 0, and 6 if FLAG=1.

SUM := 0 ; FOR I := 1 TO 3 DO FOR J := 1 TO 2 DO IF FLAG THEN EXIT ; END_IF SUM := SUM + J ; END_FOR ; SUM := SUM + I ; END_FOR ;

Figure 22 - EXIT statement example

The FOR statement indicates that a statement sequence shall be repeatedly executed, up to the END_FOR keyword, while a progression of values is assigned to the FOR loop control variable.  The control variable, initial value, and final value shall be expressions of the same integer type (e.g., SINT, INT, or DINT) and shall not be altered by any of the repeated statements.  The FOR statement increments the control variable up or down from an initial value to a final value in increments determined by the value of an expression; this value defaults to 1.  The test for the termination condition is made at the beginning of each iteration, so that the statement sequence is not executed if the initial value exceeds the final value.  The value of the control variable after completion of the FOR loop is implementation-dependent.

An example of the usage of the FOR statement is given in feature 6 of table 56.  In this example, the FOR loop is used to determine the index J of the first occurrence (if any) of the string 'KEY' in the odd-numbered elements of an array of strings WORDS with a subscript range of (1..100). If no occurrence is found, J will have the value 101.

The WHILE statement causes the sequence of statements up to the END_WHILE keyword to be executed repeatedly until the associated Boolean expression is false.  If the expression is initially false, then the group of statements is not executed at all.  For instance, the FOR...END_FOR example given in table 56 can be rewritten using the WHILE...END_WHILE construction shown in table 56.

The REPEAT statement causes the sequence of statements up to the UNTIL keyword to be executed repeatedly (and at least once) until the associated Boolean condition is true.  For instance, the WHILE...END_WHILE example given in table 56 can be rewritten using the REPEAT...END_REPEAT construction shown in table 56.

The WHILE and REPEAT statements shall not be used to achieve interprocess synchronization, for example as a "wait loop" with an externally determined termination condition.  The SFC elements defined in  2.6 shall be used for this purpose.

It shall be an error in the sense of 1.5.1 if a WHILE or REPEAT statement is used in an algorithm for which satisfaction of the loop termination condition or execution of an EXIT statement cannot be guaranteed.

# 4.  Graphic languages

The graphic languages defined in this standard are LD (Ladder Diagram) and FBD (Function Block Diagram).  The sequential function chart (SFC) elements defined in 2.6 can be used in conjunction with either of these languages.

## 4.1  Common elements

The elements defined in this clause apply to both the graphic languages in this Standard, that is, LD (Ladder Diagram) and FBD (Function Block Diagram), and to the graphic representation of sequential function chart (SFC) elements.

### 4.1.1 Representation of lines and blocks

The graphic language elements defined in this clause are drawn with line elements using characters from the character set defined in 2.1.1, or using graphic or semigraphic elements, as shown in table 57.

Lines can be extended by the use of connectors as shown in table 57.  No storage of data or association with data elements shall be associated with the use of connectors; hence, to avoid ambiguity, it shall be an error if the identifier used as a connector label is the same as the name of another named element within the same program organization unit.

Any restrictions on network topology in a particular implementation shall be expressed as implementation-dependent parameters.

### 4.1.2  Direction of flow in networks

A network is defined as a maximal set of interconnected graphic elements, excluding the left and right rails in the case of networks in the LD language defined in 4.2.  Provision shall be made to associate with each network or group of networks in a graphic language a network label delimited on the right by a colon (:).  This label shall have the form of an identifier or an unsigned decimal integer as defined in clause 2 of this Part.  The scope of a network and its label shall be local to the program organization unit in which the network is located.  Examples of networks and network labels are shown in annex F.

Graphic languages are used to represent the flow of a conceptual quantity through one or more networks representing a control plan, that is:

- "Power flow", analogous to the flow of electric power in an electromechanical relay system, typically used in relay ladder diagrams;

- "Signal flow", analogous to the flow of signals between elements of a signal processing system, typically used in function block diagrams;

- "Activity flow", analogous to the flow of control between elements of an organization, or between the steps of an electromechanical sequencer, typically used in sequential function charts.

The appropriate conceptual quantity shall flow along lines between elements of a network according to the following rules:

1) Power flow in the LD language shall be from left to right.

2) Signal flow in the FBD language shall be from the output (right-hand) side of a function or function block to the input (left-hand) side of the function or function block(s) so connected.

3) Activity flow between the SFC elements defined in 2.6 shall be from the bottom of a step through the appropriate transition to the top of the corresponding successor step(s).

Table 57 - Representation of lines and blocks

| No. | Feature | Example |
| --- | --- | --- |
| 1 2 | Horizontal lines: ISO / IEC 10646 "minus" character Graphic or semigraphic | ----- |
| 3 4 | Vertical lines: ISO / IEC 10646 "vertical line" character Graphic or semigraphic | ¦ |
| 5 6 | Horizontal/vertical connection: ISO / IEC 10646 "plus" character Graphic or semigraphic | ¦ --+-- ¦ |
| 7 8 | Line crossings without connection: ISO / IEC 10646 characters Graphic or semigraphic | ¦ ¦ --------¦---- ¦ ¦ |
| 9 10 | Connected and non-connected corners: ISO / IEC 10646 characters Graphic or semigraphic | ¦ ¦ ----+ +---- ¦ ----+-+ +---- ¦ ¦ ¦ |
| 11 12 | Blocks with connecting lines: ISO / IEC 10646 characters Graphic or semigraphic | ¦ +--------+ ---¦ ¦ ¦ ¦--- ---¦ ¦ +--------+ ¦ |
| 13 14 | Connectors using ISO / IEC 10646 characters: Connector Continuation of a connected line Graphic or semigraphic connectors | ---------->OTTO> >OTTO>---------- |

### 4.1.3  Evaluation of networks

The order in which networks and their  elements are evaluated is not necessarily the same as the order in which they are labeled or displayed.  Similarly, it is not necessary that all networks be evaluated before the evaluation of a given network can be repeated.  However, when the body of a program organization unit consists of  one or more networks, the results of network evaluation within said body shall be functionally equivalent to the observance of the following rules:

1) No element of a network shall be evaluated until the states of all of its inputs have been evaluated.

2) The evaluation of a network element shall not be complete until the states of all of its outputs have been evaluated.

3) The evaluation of a network is not complete until the outputs of all of its elements have been evaluated, even if the network contains one of the execution control elements defined in 4.1.4.

(4) The order in which networks are evaluated shall conform to the provisions of 4.2.6 for the LD language and 4.3.3 for the FBD language.

A feedback path is said to exist in a network when the output of a function or function block is used as the input to a function or function block which precedes it in the network; the associated variable is called a feedback variable.  For instance, the Boolean variable RUN is the feedback variable in the example shown in figure 23.  A feedback variable can also be an output element of a function block data structure as defined in 2.5.2.

Feedback paths can be utilized in the graphic languages defined in 4.2 and 4.3, subject to the following rules:

1)	Explicit loops such as the one shown in 23a shall only appear in the FBD language defined in 4.3.

2)	It shall be possible for the user to utilize an implementation-dependent means to determine the order of execution of the elements in an explicit loop, for instance by selection of feedback variables to form an implicit loop as shown in figure 23b.

3)	Feedback variables shall be initialized by one of the mechanisms defined in clause 2.  The initial value shall be used during the first evaluation of the network. It shall be an error if a feedback variable is not initialized.

4)	Once the element with a feedback variable as output has been evaluated, the new value of the feedback variable shall be used until the next evaluation of the element.

| a) | +---+ ENABLE---¦ & ¦-----RUN---+ +---¦ ¦ ¦ +---+ ¦ +---+ ¦ START1---¦>=1¦---+ ¦ START2---¦ ¦ ¦ +--¦ ¦ ¦ ¦ +---+ ¦ +------------------------------+ |
| --- | --- |
| b) | +---+ ENABLE---¦ & ¦-----RUN +---¦ ¦ +---+ ¦ +---+ START1---¦>=1¦---+ START2---¦ ¦ RUN---¦ ¦ +---+ |
| c) | ¦ START1 ENABLE RUN ¦ +---¦ ¦----+---¦ ¦------( )---+ ¦ START2 ¦ ¦ +---¦ ¦----+ ¦ ¦ RUN ¦ ¦ +---¦ ¦----+ ¦ ¦ ¦ |

Figure 23 - Feedback path example

a) Explicit loop
b) Implicit loop
c) LD language equivalent

### 4.1.4  Execution control elements

Transfer of program control in the LD and FBD languages shall be represented by the graphical elements shown in table 58.

Jumps shall be shown by a Boolean signal line terminated in a double arrowhead.  The signal line for a jump condition shall originate at a Boolean variable, at a Boolean output of a function or function block, or on the power flow line of a ladder diagram.  A transfer of program control to the designated network label shall occur when the Boolean value of the signal line is 1 (TRUE); thus, the unconditional jump is a special case of the conditional jump.

The target of a jump shall be a network label within the program organization unit within which the jump occurs.  If the jump occurs within an ACTION...END_ACTION construct, the target of the jump shall be within the same construct.

Conditional returns from functions and function blocks shall be implemented using a RETURN construction as shown in table 58.  Program execution shall be transferred back to the invoking entity when the Boolean input is 1 (TRUE), and shall continue in the normal fashion when the Boolean input is 0 (FALSE).  Unconditional returns shall be provided by the physical end of the function or function block, or by a RETURN element connected to the left rail in the LD language, as shown in table 58.

Table 58 - Graphic execution control elements

| No. | Symbol/Example | Explanation |
| --- | --- | --- |
| 1 2 | 1---->>LABELA ¦ +---->>LABELA ¦ | Unconditional Jump: FBD Language LD Language |
| 3 | X---->>LABELB +---+ %IX20---¦ & ¦--->>NEXT %MX50---¦ ¦ +---+ NEXT: +---+ %IX25---¦>=1¦---%QX100 %MX60---¦ ¦ +---+ | Conditional Jump (FBD Language) Example: Jump Condition Jump Target |
| 4 | ¦ X +-¦ ¦---->>LABELB ¦ ¦ ¦ %IX20 %MX50 +---¦ ¦-----¦ ¦--->>NEXT ¦ ¦ NEXT: ¦ %IX25 %QX100 ¦ +----¦ ¦----+----( )---+ ¦ %MX60 ¦ ¦ +----¦ ¦----+ ¦ ¦ ¦ | Conditional Jump (LD Language) Example: Jump Condition Jump Target |
| 5 6 | ¦ X +--¦ ¦---<RETURN> ¦ X---<RETURN> | Conditional Return: LD Language FBD Language |
| 7 8 | END_FUNCTION END_FUNCTION_BLOCK ¦ +---<RETURN> ¦ | Unconditional Return: from FUNCTION from FUNCTION_BLOCK Alternative representation in LD language |

## 4.2  Ladder Diagram (LD)

This subclause defines the LD language for ladder diagram programming of programmable controllers.

A LD program enables the programmable controller to test and modify data by means of standardized graphic symbols.  These symbols are laid out in networks in a manner similar to a "rung" of a relay ladder logic diagram.  LD networks are bounded on the left and right by power rails.

### 4.2.1  Power rails

As shown in table 59, LD network shall be delimited on the left by a vertical line known as the left power rail, and on the right by a vertical line known as the right power rail. The right power rail may be explicit or implied.

Table 59 - Power rails

| No. | Symbol | Description |
| --- | --- | --- |
| 1 | ¦ +--- ¦ | Left power rail (with attached horizontal link) |
| 2 | ¦ ---+ ¦ | Right power rail (with attached horizontal link) |

### 4.2.2  Link elements and states

As shown in table 60, link elements may be horizontal or vertical.  The state of the link element shall be denoted "ON" or "OFF", corresponding to the literal Boolean values 1 or 0, respectively.  The term link state shall be synonymous with the term power flow.

The state of the left rail shall be considered ON at all times..  No state is defined for the right rail.

A horizontal link element shall be indicated by a horizontal line.  A horizontal link element transmits the state of the element on its immediate left to the element on its immediate right.

The vertical link element shall consist of a vertical line intersecting with one or more horizontal link elements on each side.  The state of the vertical link shall represent the inclusive OR of the ON states of the horizontal links on its left side, that is, the state of the vertical link shall be:

- OFF if the states of all the attached horizontal links to its left are OFF;

- ON if the state of one or more of the attached horizontal links to its left is ON.

The state of the vertical link shall be copied to all of the attached horizontal links on its right.  The state of the vertical link shall not be copied to any of the attached horizontal links on its left.

Table 60 - Link elements

| No. | Symbol | Description |
| --- | --- | --- |
| 1 | ----------- | Horizontal link |
| 2 | ¦ ----+---- ----+ ¦ +---- | Vertical link (with attached horizontal links) |

### 4.2.3  Contacts

A contact is an element which imparts a state to the horizontal link on its right side which is equal to the Boolean AND of the state of the horizontal link at its left side with an appropriate function of an associated Boolean input, output, or memory variable.  A contact does not modify the value of the associated Boolean variable. Standard contact symbols are given in table 61.

### 4.2.4  Coils

A coil copies the state of the link on its left to the link on its right without modification, and stores an appropriate function of the state or transition of the left link into the associated Boolean variable.  Standard coil symbols are given in table 62.

EXAMPLE - In the rung shown below, the value of the Boolean output a is always TRUE, while the value of outputs c, d and e upon completion of an evaluation of the rung is equal to the value of the input b.

|   a    b       c    d    |
+--( )--| |--+--( )---( )--+
|            |      e      |
|            +-----( )-----+
|                          |

### 4.2.5  Functions and function blocks

The representation of functions and function blocks in the LD language shall be as defined in clause 2 of this Part, with the following exceptions:

1) Actual variable connections may optionally be shown by writing the appropriate data or variable outside the block adjacent to the formal variable name on the inside.

2) At least one Boolean input and one Boolean output shall be shown on each block to allow for power flow through the block.

### 4.2.6  Order of network evaluation

Within a program organization unit written in LD, networks shall be evaluated in top to bottom order as they appear in the ladder diagram, except as this order is modified by the execution control elements defined in 4.1.4.

Table 61 - Contacts a

| Static contacts |  |  |
| --- | --- | --- |
| No. | Symbol | Description |
| 1 2 | *** --¦ ¦-- or *** --! !-- | Normally open contact The state of the left link is copied to the right link if the state of the associated Boolean variable (indicated by "***") is ON. Otherwise, the state of the right link is OFF. |
| 3 4 | *** --¦/¦-- or *** --!/!-- | Normally closed contact The state of the left link is copied to the right link if the state of the associated Boolean variable is OFF. Otherwise, the state of the right link is OFF. |
| Transition-sensing contacts |  |  |
| 5 6 | *** --¦P¦-- or *** --!P!-- | Positive transition-sensing contact The state of the right link is ON from one evaluation of this element to the next when a transition of the associated variable from OFF to ON is sensed at the same time that the state of the left link is ON. The state of the right link shall be OFF at all other times. |
| 7 8 | *** --¦N¦-- or *** --!N!-- | Negative transition-sensing contact The state of the right link is ON from one evaluation of this element to the next when a transition of the associated variable from ON to OFF is sensed at the same time that the state of the left link is ON. The state of the right link shall be OFF at all other times. |
| a As specified in 2.1.1, the exclamation mark "!" shall be used when a national character set does not support the vertical bar "¦". |  |  |

| Table 62 - Coils |  |  |
| --- | --- | --- |
| No. | Symbol | Description |
| Momentary coils |  |  |
| 1 | *** --( )-- | Coil The state of the left link is copied to the associated Boolean variable and to the right link. |
| 2 | *** --(/)-- | Negated coil The state of the left link is copied to the right link. The inverse of the state of the left link is copied to the associated Boolean variable, that is, if the state of the left link is OFF, then the state of the associated variable is ON, and vice versa. |
| Latched Coils |  |  |
| 3 | *** --(S)-- | SET (latch) coil The associated Boolean variable is set to the ON state when the left link is in the ON state, and remains set until reset by a RESET coil. |
| 4 | *** --(R)-- | RESET (unlatch) coil The associated Boolean variable is reset to the OFF state when the left link is in the ON state, and remains reset until set by a SET coil. |
| Transition-sensing coils |  |  |
| 8 | *** --(P)-- | Positive transition-sensing coil The state of the associated Boolean variable is ON from one evaluation of this element to the next when a transition of the left link from OFF to ON is sensed. The state of the left link is always copied to the right link. |
| 9 | *** --(N)-- | Negative transition-sensing coil The state of the associated Boolean variable is ON from one evaluation of this element to the next when a transition of the left link from ON to OFF is sensed. The state of the left link is always copied to the right link. |
| NOTE Features 5, 6 and 7 of the first Edition of this Part are deleted in this Edition. |  |  |

## 4.3  Function Block Diagram (FBD)

### 4.3.1  General

This subclause defines FBD, a graphic language for the programming of programmable controllers which is consistent, as far as possible, with IEC 617-12.  Where conflicts exist between this standard and IEC 617, the provisions of this standard shall apply for the programming of programmable controllers in the FBD language.

The provisions of clauses 2 and 4.1 shall apply to the construction and interpretation of programmable controller programs in the FBD language.

Examples of the use of the FBD language are given in annex F.

### 4.3.2  Combination of elements

Elements of the FBD language shall be interconnected by signal flow lines following the conventions of 4.1.2.

Outputs of function blocks shall not be connected together.  In particular, the "wired-OR" construct of the LD language is not allowed in the FBD language; an explicit Boolean "OR" block is required instead, as shown in figure 24.

| a) | b) |
| --- | --- |
| ¦ a c ¦ +---¦¦--+--()--+ ¦ b ¦ ¦ +--¦¦---+ ¦ ¦ ¦ | +-----+ a---¦ >=1 ¦---c b---¦ ¦ +-----+ |

Figure 24 - Boolean OR Examples

a) "Wired-OR" in LD language
b) Function in FBD language

### 4.3.3  Order of network evaluation

When a program organization unit written in the FBD language contains more than one network, the manufacturer shall provide implementation-dependent means by which the user may determine the order of execution of networks.

# ANNEX A - Specification method for textual languages (normative)

Programming languages are specified in terms of a syntax, which specifies the allowable combinations of symbols which can be used to define a program; and a set of semantics, which specify the relationship between programmed operations and the symbol combinations defined by the syntax.

## A.1  Syntax

A syntax is defined by a set of terminal symbols to be utilized for program specification; a set of non-terminal symbols defined in terms of the terminal symbols; and a set of production rules specifying those definitions.

### A.1.1    Terminal symbols

The terminal symbols for textual programmable controller programs shall consist of combinations of the characters in the character set defined in 2.1.1.

For the purposes of this part, terminal textual symbols consist of the appropriate character string enclosed in paired single or double quotes.  For example, a terminal symbol represented by the character string ABC can be represented by either

"ABC"

or

'ABC'

This allows the representation of strings containing either single or double quotes; for instance, a terminal symbol consisting of the double quote itself would be represented by '"'.

A special terminal symbol utilized in this syntax is the end-of-line delimiter, which is represented by the unquoted character string EOL.  This symbol shall normally consist of the "paragraph separator" character defined as hexadecimal code 2029 by ISO/IEC 10646.

A second special terminal symbol utilized in this syntax is the "null string", that is, a string containing no characters.  This is represented by the terminal symbol NIL.

The case of letters shall not be significant in terminal symbols.

### A.1.2    Non-terminal symbols

Non-terminal textual symbols shall be represented by strings of lower-case letters, numbers, and the underline character (_), beginning with a lower-case letter.  For instance, the strings

nonterm1

and

non_term_2

are valid non-terminal symbols, while the strings

3nonterm

and

_nonterm4

are not.

### A.1.3   Production rules

The production rules for textual programmable controller programming languages shall form an extended grammar in which each rule has the form

non_terminal_symbol ::= extended_structure

This rule can be read as:

"A non_terminal_symbol can consist of an extended_structure."

Extended structures can be constructed according to the following rules:

1) The null string, NIL, is an extended structure.

2) A terminal symbol is an extended structure.

3) A non-terminal symbol is an extended structure.

4) If S is an extended structure, then the following expressions are also extended structures:

(S), meaning S itself.

{S}, closure, meaning zero or more concatenations of S.

[S], option, meaning zero or one occurrence of S.

5) If S1 and S2 are extended structures, then the following expressions are extended structures:

S1 | S2, alternation, meaning a choice of S1 or S2.

S1 S2, concatenation, meaning S1 followed by S2.

6) Concatenation precedes alternation, that is, S1 | S2 S3 is equivalent to S1 | (S2 S3),
 and S1 S2 | S3 is equivalent to (S1 S2) | S3.

## A.2  Semantics

Programmable controller textual programming language semantics are defined in this Part by appropriate natural language text, accompanying the production rules, which references the descriptions provided in the appropriate clauses.  Standard options available to the user and manufacturer are specified in these semantics.

In some cases it is more convenient to embed semantic information in an extended structure.  In such cases, this information is delimited by paired angle brackets, for example, <semantic information>.

# ANNEX B - Formal specifications of language elements (normative)

## B.0  Programming model

The contents of this annex are normative in the sense that a compiler which is capable of recognizing all the syntax in this annex shall be capable of recognizing the syntax of any textual language implementation complying with this standard.

PRODUCTION RULES:

library_element_name ::= data_type_name | function_name 
| function_block_type_name | program_type_name 
| resource_type_name | configuration_name

library_element_declaration ::= data_type_declaration 
| function_declaration | function_block_declaration 
| program_declaration | configuration_declaration

SEMANTICS: These productions reflect the basic programming model defined in 1.4.3, where declarations are the basic mechanism for the production of named library elements.  The syntax and semantics of the non-terminal symbols given above are defined in the subclauses listed below.

| Non-terminal symbol | Syntax | Semantics |
| --- | --- | --- |
| data_type_name data_type_declaration | B.1.3 | 2.3 |
| function_name function_declaration | B.1.5.1 | 2.5.1 |
| function_block_type_name function_block_declaration | B.1.5.2 | 2.5.2 |
| program_type_name program_declaration | B.1.5.3 | 2.5.3 |
| resource_type_name |  |  |
| configuration_name | B.1.7 | 2.7 |
| configuration_declaration |  |  |

## B.1  Common elements

### B.1.1 Letters, digits and identifiers

PRODUCTION RULES:

letter  ::=  'A' | 'B' | <...> | 'Z' | 'a' | 'b' | <...> | 'z'

digit ::= '0' | '1' | '2' | '3' | '4' | '5' | '6' | '7' | '8' | '9'

octal_digit ::= '0' | '1' | '2' | '3' | '4' | '5' | '6' | '7'

hex_digit ::= digit | 'A'|'B'|'C'|'D'|'E'|'F'

identifier ::= (letter | ('_' (letter | digit))) {['_'] (letter | digit)}

SEMANTICS:

The ellipsis <...> here indicates the ISO/IEC 10646 sequence of 26 letters.

Characters from national character sets can be used; however, international portability of the printed representation of programs cannot be guaranteed in this case.

### B.1.2  Constants

PRODUCTION RULE:

constant ::= numeric_literal | character_string | time_literal | bit_string_literal | boolean_literal

SEMANTICS:

The external representations of data described in 2.2 are designated as "constants" in this annex.

### B.1.2.1   Numeric literals

PRODUCTION RULES:

numeric_literal ::= integer_literal | real_literal

integer_literal ::= [ integer_type_name '#' ]
( signed_integer | binary_integer | octal_integer | hex_integer)

signed_integer ::= ['+' |'-'] integer

integer ::= digit {['_'] digit}

binary_integer ::= '2#' bit {['_'] bit}

bit ::= '1' | '0'

octal_integer ::= '8#' octal_digit {['_'] octal_digit}

hex_integer ::= '16#' hex_digit {['_'] hex_digit}

real_literal ::= [ real_type_name '#' ]
signed_integer  '.' integer [exponent]

exponent ::= ('E' | 'e') ['+'|'-'] integer

bit_string_literal ::= 
[ ('BYTE' | 'WORD' | 'DWORD' | 'LWORD') '#' ]
( unsigned_integer | binary_integer | octal_integer | hex_integer)

boolean_literal ::= 
( [ 'BOOL#' ] ( '1' | '0' )  )| 'TRUE' | 'FALSE'

SEMANTICS:  See 2.2.1.

#### B.1.2.2  Character strings

PRODUCTION RULES:

character_string ::= 
single_byte_character_string | double_byte_character_string

single_byte_character_string ::= "'" {single_byte_character_representation} "'"

double_byte_character_string ::= '"' {double_byte_character_representation} '"'

single_byte_character_representation ::= common_character_representation | "$'" | '"' | '$' hex_digit hex_digit

double_byte_character_representation ::= common_character_representation |  '$"' | "'"| '$' hex_digit hex_digit hex_digit hex_digit

common_character_representation ::= 
<any printable character except '$', '"' or "'"> 
| '$$' | '$L' | '$N' | '$P' | '$R' | '$T' 
| '$l' | '$n' | '$p' | '$r' | '$t'

SEMANTICS:  See 2.2.2.

#### B.1.2.3 Time literals

PRODUCTION RULE:

time_literal ::= duration | time_of_day | date | date_and_time

SEMANTICS: See 2.2.3.

##### B.1.2.3.1  Duration

PRODUCTION RULES:

duration ::= ('T' | 'TIME') '#' ['-'] interval

interval ::= days | hours | minutes | seconds | milliseconds

days ::= fixed_point ('d') | integer ('d') ['_'] hours

fixed_point ::= integer [ '.' integer]

hours ::= fixed_point ('h') | integer ('h') ['_'] minutes

minutes ::= fixed_point ('m')  | integer ('m') ['_'] seconds

seconds ::= fixed_point ('s') | integer ('s') ['_'] milliseconds

milliseconds ::= fixed_point ('ms')

SEMANTICS:  See 2.2.3.1.

NOTE	The semantics of 2.2.3.1 impose additional constraints on the allowable values of hours, minutes, seconds, and milliseconds.

##### B.1.2.3.2  Time of day and date

PRODUCTION RULES:

time_of_day ::= ('TIME_OF_DAY' | 'TOD')  '#' daytime

daytime ::= day_hour ':' day_minute ':' day_second

day_hour ::= integer

day_minute ::= integer

day_second ::= fixed_point

date ::= ('DATE' | 'D') '#' date_literal

date_literal ::= year '-' month '-' day

year ::= integer

month ::= integer

day ::= integer

date_and_time ::= ('DATE_AND_TIME' | 'DT') '#' date_literal '-' daytime

SEMANTICS:  See 2.2.3.2.

NOTE	The semantics of 2.2.3.2 impose additional constraints on the allowable values of day_hour, day_minute, day_second, year, month, and day.

### B.1.3   Data types

PRODUCTION RULES:

data_type_name ::= non_generic_type_name | generic_type_name

non_generic_type_name ::=  elementary_type_name | derived_type_name

SEMANTICS:  See 2.3.

#### B.1.3.1  Elementary data types

PRODUCTION RULES:

elementary_type_name ::= numeric_type_name | date_type_name | bit_string_type_name | 'STRING' | 'WSTRING' | 'TIME'

numeric_type_name ::= integer_type_name | real_type_name

integer_type_name ::= signed_integer_type_name | unsigned_integer_type_name

signed_integer_type_name ::= 'SINT' | 'INT' | 'DINT' | 'LINT'

unsigned_integer_type_name ::= 'USINT' | 'UINT' | 'UDINT'  | 'ULINT'

real_type_name ::= 'REAL' | 'LREAL'

date_type_name ::= 'DATE' | 'TIME_OF_DAY' | 'TOD'  | 'DATE_AND_TIME' | 'DT'

bit_string_type_name ::= 'BOOL' | 'BYTE' | 'WORD' | 'DWORD' | 'LWORD'

SEMANTICS:  See 2.3.1.

#### B.1.3.2  Generic data types

PRODUCTION RULE:

generic_type_name ::= 'ANY' | 'ANY_DERIVED' | 'ANY_ELEMENTARY' | 'ANY_MAGNITUDE' | 'ANY_NUM' | 'ANY_REAL' | 'ANY_INT' | 'ANY_BIT' | 'ANY_STRING' | 'ANY_DATE'

SEMANTICS:  See 2.3.2.

#### B.1.3.3  Derived data types

PRODUCTION RULES:

derived_type_name ::= single_element_type_name | array_type_name | structure_type_name | string_type_name

single_element_type_name ::= simple_type_name | subrange_type_name 
| enumerated_type_name

simple_type_name ::= identifier

subrange_type_name ::= identifier

enumerated_type_name ::= identifier

array_type_name ::= identifier

structure_type_name ::= identifier

data_type_declaration ::= 
'TYPE' type_declaration ';' 
{type_declaration ';'} 
'END_TYPE'

type_declaration ::= single_element_type_declaration | array_type_declaration 
| structure_type_declaration | string_type_declaration

single_element_type_declaration ::= simple_type_declaration | subrange_type_declaration | enumerated_type_declaration

simple_type_declaration ::= simple_type_name ':' simple_spec_init

simple_spec_init := simple_specification [':=' constant]

simple_specification ::= elementary_type_name | simple_type_name

subrange_type_declaration ::= subrange_type_name ':' subrange_spec_init

subrange_spec_init ::= subrange_specification [':=' signed_integer]

subrange_specification ::= integer_type_name '(' subrange')' | subrange_type_name

subrange ::= signed_integer '..' signed_integer

enumerated_type_declaration ::= enumerated_type_name ':' enumerated_spec_init

enumerated_spec_init ::= enumerated_specification [':=' enumerated_value]

enumerated_specification ::=
( '(' enumerated_value {',' enumerated_value} ')' ) | enumerated_type_name

enumerated_value ::= [enumerated_type_name '#'] identifier

array_type_declaration ::= array_type_name ':' array_spec_init

array_spec_init ::= array_specification [':=' array_initialization]

array_specification ::= array_type_name 
| 'ARRAY' '[' subrange {',' subrange} ']' 'OF' non_generic_type_name

array_initialization ::=
'[' array_initial_elements {',' array_initial_elements} ']'

array_initial_elements ::=
array_initial_element | integer '(' [array_initial_element] ')'

array_initial_element ::= constant | enumerated_value | structure_initialization | array_initialization

structure_type_declaration ::=
structure_type_name ':' structure_specification

structure_specification ::= structure_declaration | initialized_structure

initialized_structure ::= structure_type_name [':=' structure_initialization]

structure_declaration ::= 
'STRUCT' structure_element_declaration ';' 
{structure_element_declaration ';'} 
'END_STRUCT'

structure_element_declaration ::= structure_element_name ':' 
(simple_spec_init | subrange_spec_init  | enumerated_spec_init | array_spec_init | initialized_structure)

structure_element_name ::= identifier

structure_initialization ::= 
'(' structure_element_initialization 
{','  structure_element_initialization} ')'

structure_element_initialization ::= 
structure_element_name ':=' (constant | enumerated_value | array_initialization | structure_initialization)

string_type_name ::= identifier

string_type_declaration ::= string_type_name ':' 
('STRING'|'WSTRING') ['[' integer ']'] [':=' character_string]

SEMANTICS:  See 2.3.3.

### B.1.4  Variables

PRODUCTION RULES:

variable ::= direct_variable | symbolic_variable

symbolic_variable ::= variable_name | multi_element_variable

variable_name ::= identifier

SEMANTICS:  See 2.4.1.

#### B.1.4.1  Directly represented variables

PRODUCTION RULES:

direct_variable ::= '%' location_prefix size_prefix integer {'.' integer}

location_prefix ::= 'I' | 'Q' | 'M'

size_prefix ::= NIL | 'X' | 'B' | 'W' | 'D' | 'L'

SEMANTICS:  See 2.4.1.1.

#### B.1.4.2  Multi-element variables

PRODUCTION RULES:

multi_element_variable ::= array_variable | structured_variable

array_variable ::= subscripted_variable subscript_list

subscripted_variable ::= symbolic_variable

subscript_list ::= '[' subscript {',' subscript} ']'

subscript ::= expression

structured_variable ::= record_variable '.' field_selector

record_variable ::= symbolic_variable

field_selector ::= identifier

SEMANTICS:  See 2.4.1.2.

#### B.1.4.3  Declaration and initialization

PRODUCTION RULES:

input_declarations ::= 
'VAR_INPUT' ['RETAIN' | 'NON_RETAIN']
input_declaration ';' 
{input_declaration ';'} 
'END_VAR'

input_declaration ::= var_init_decl | edge_declaration

edge_declaration ::= var1_list ':' 'BOOL' ('R_EDGE' | 'F_EDGE')

var_init_decl ::= var1_init_decl | array_var_init_decl | structured_var_init_decl | fb_name_decl | string_var_declaration

var1_init_decl ::=  var1_list ':' 
(simple_spec_init | subrange_spec_init | enumerated_spec_init)

var1_list ::= variable_name {',' variable_name}

array_var_init_decl ::= var1_list ':' array_spec_init

structured_var_init_decl ::= var1_list ':' initialized_structure

fb_name_decl ::= fb_name_list ':' function_block_type_name 
[ ’:=’ structure_initialization ]

fb_name_list ::= fb_name {',' fb_name}

fb_name ::= identifier

output_declarations ::= 
'VAR_OUTPUT' ['RETAIN' | 'NON_RETAIN'] 
  var_init_decl ';' 
  {var_init_decl ';'} 
'END_VAR'

input_output_declarations ::= 
'VAR_IN_OUT' 
var_declaration ';' 
{var_declaration ';'} 
'END_VAR'

var_declaration ::=  temp_var_decl | fb_name_decl

temp_var_decl ::= var1_declaration | array_var_declaration | structured_var_declaration | string_var_declaration

var1_declaration ::=  var1_list  ':' (simple_specification | subrange_specification | enumerated_specification)

array_var_declaration ::= var1_list ':' array_specification

structured_var_declaration ::= var1_list ':' structure_type_name

var_declarations ::= 
'VAR' ['CONSTANT'] 
var_init_decl ';' 
{(var_init_decl ';')} 
'END_VAR'

retentive_var_declarations ::= 
'VAR' 'RETAIN' 
var_init_decl ';' 
{var_init_decl ';'} 
'END_VAR'

located_var_declarations ::= 
'VAR' ['CONSTANT' | 'RETAIN' | 'NON_RETAIN']
  located_var_decl ';' 
  {located_var_decl ';'} 
'END_VAR'

located_var_decl ::= [variable_name] location ':' located_var_spec_init

external_var_declarations := 
'VAR_EXTERNAL' ['CONSTANT']
external_declaration ';' 
{external_declaration ';'} 
'END_VAR'

external_declaration ::= global_var_name ':' 
(simple_specification | subrange_specification | enumerated_specification | array_specification | structure_type_name | function_block_type_name)

global_var_name ::= identifier

global_var_declarations := 
'VAR_GLOBAL' ['CONSTANT' | 'RETAIN']
global_var_decl ';' 
{global_var_decl ';'}  
'END_VAR'

global_var_decl ::= global_var_spec ':' 
[ located_var_spec_init | function_block_type_name ]

global_var_spec ::= global_var_list | [global_var_name] location

located_var_spec_init ::= simple_spec_init | subrange_spec_init | enumerated_spec_init | array_spec_init | initialized_structure | single_byte_string_spec | double_byte_string_spec

location ::= 'AT' direct_variable

global_var_list ::= global_var_name {',' global_var_name}

string_var_declaration ::= single_byte_string_var_declaration | double_byte_string_var_declaration

single_byte_string_var_declaration ::= 
var1_list ':' single_byte_string_spec

single_byte_string_spec ::=
'STRING' ['[' integer ']'] [':=' single_byte_character_string]

double_byte_string_var_declaration ::= 
var1_list ':' double_byte_string_spec

double_byte_string_spec ::=
'WSTRING' ['[' integer ']'] [':=' double_byte_character_string]

incompl_located_var_declarations ::= 
'VAR' ['RETAIN'|'NON_RETAIN']
   incompl_located_var_decl ';'
   {incompl_located_var_decl ';'}
'END_VAR'

incompl_located_var_decl ::= variable_name incompl_location ':' var_spec

incompl_location ::= 'AT' '%' ('I' | 'Q' | 'M') '*'

var_spec ::= simple_specification 
| subrange_specification | enumerated_specification
| array_specification | structure_type_name | 'STRING' ['[' integer ']'] | 'WSTRING' ['[' integer ']']

SEMANTICS: See 2.4.2. The non-terminal function_block_type_name is defined in B.1.5.2.

### B.1.5  Program organization units

#### B.1.5.1  Functions

PRODUCTION RULES:

function_name ::= standard_function_name | derived_function_name

standard_function_name ::= <as defined in 2.5.1.5>

derived_function_name ::= identifier

function_declaration ::= 
'FUNCTION' derived_function_name ':' 
		(elementary_type_name | derived_type_name)
	{ io_var_declarations | function_var_decls }
	function_body
'END_FUNCTION'

io_var_declarations ::= input_declarations | output_declarations | 
input_output_declarations

function_var_decls ::= 'VAR' ['CONSTANT']
var2_init_decl ';' {var2_init_decl ';'} 'END_VAR'

function_body ::= ladder_diagram | function_block_diagram | instruction_list | statement_list | <other languages>

var2_init_decl ::= var1_init_decl | array_var_init_decl 
| structured_var_init_decl | string_var_declaration

SEMANTICS:  See 2.5.1.

NOTE 1	This syntax does not reflect the fact that each function must have at least one input declaration.

NOTE 2	This syntax does not reflect the fact that edge declarations, function block references and invocations are not allowed in function bodies.

NOTE 3	Ladder diagrams and function block diagrams are graphically represented as defined in Clause 4.  The non-terminals instruction_list and statement_list are defined in B.2.1 and B.3.2, respectively.

#### B.1.5.2  Function blocks

PRODUCTION RULES:

function_block_type_name ::= standard_function_block_name | derived_function_block_name

standard_function_block_name ::= <as defined in 2.5.2.3>

derived_function_block_name ::= identifier

function_block_declaration ::=
'FUNCTION_BLOCK' derived_function_block_name
   { io_var_declarations | other_var_declarations }
   function_block_body
'END_FUNCTION_BLOCK'

other_var_declarations ::= external_var_declarations | var_declarations | retentive_var_declarations | non_retentive_var_declarations | temp_var_decls | incompl_located_var_declarations

temp_var_decls ::= 
'VAR_TEMP'
  temp_var_decl ';' 
  {temp_var_decl ';'} 
'END_VAR'

non_retentive_var_decls ::= 
'VAR' 'NON_RETAIN'
  var_init_decl ';' 
  {var_init_decl ';'} 
'END_VAR'

function_block_body ::= sequential_function_chart | ladder_diagram | function_block_diagram | instruction_list | statement_list | <other languages>

SEMANTICS:  See 2.5.2.

NOTE 1	Ladder diagrams and function block diagrams are graphically represented as defined in clause 4.

NOTE 2	The non-terminals sequential_function_chart, instruction_list, and statement_list are defined in  B.1.6, B.2.1, and B.3.2, respectively.

#### B.1.5.3  Programs

PRODUCTION RULES:

program_type_name :: = identifier

program_declaration ::= 
'PROGRAM' program_type_name
  { io_var_declarations | other_var_declarations
    | located_var_declarations | program_access_decls }
  function_block_body
'END_PROGRAM'

program_access_decls ::= 
'VAR_ACCESS' program_access_decl ';'
   {program_access_decl ';' }
'END_VAR'

program_access_decl ::= access_name ':' symbolic_variable ':' non_generic_type_name [direction]

SEMANTICS:  See 2.5.3.

### B.1.6  Sequential function chart elements

PRODUCTION RULES:

sequential_function_chart ::= sfc_network {sfc_network}

sfc_network ::= initial_step {step | transition | action}

initial_step ::= 
'INITIAL_STEP' step_name ':' {action_association ';'} 'END_STEP'

step ::= 'STEP' step_name ':' {action_association ';'} 'END_STEP'

step_name ::= identifier

action_association ::= 
action_name '(' [action_qualifier] {',' indicator_name} ')'

action_name ::= identifier

action_qualifier ::= 
'N' | 'R' | 'S' | 'P' | timed_qualifier ',' action_time

timed_qualifier ::= 'L' | 'D' | 'SD' | 'DS' | 'SL'

action_time ::= duration | variable_name

indicator_name ::= variable_name

transition ::= ‘TRANSITION’ 
[transition_name] ['(' 'PRIORITY' ':=' integer ')']
'FROM' steps 'TO' steps 
transition_condition
'END_TRANSITION'

transition_name ::= identifier

steps ::= step_name | '(' step_name ',' step_name {',' step_name} ')'

transition_condition ::= ':' simple_instruction_list | ':=' expression ';' | ':' (fbd_network | rung)

action ::= 'ACTION' action_name ':'
          function_block_body
       'END_ACTION'

SEMANTICS:  See 2.6.  The use of function block diagram networks and ladder diagram rungs, denoted by the non-terminals fbd_network and rung, respectively, for the expression of transition conditions shall be as defined in 2.6.3.

NOTE 1	The non-terminals simple_instruction_list and expression are defined in B.2.1 and B.3.1, respectively.

NOTE 2	The term [transition_name] can only be used in the production for transition when feature No.7 of table 41 is supported.  The resulting production is the textual equivalent of this feature.

### B.1.7  Configuration elements

PRODUCTION RULES:

configuration_name ::= identifier

resource_type_name ::= identifier

configuration_declaration ::= 
'CONFIGURATION' configuration_name
   [global_var_declarations]
   (single_resource_declaration 
      | (resource_declaration {resource_declaration}))
   [access_declarations]
   [instance_specific_initializations]
'END_CONFIGURATION'

resource_declaration ::= 
'RESOURCE' resource_name 'ON' resource_type_name
   [global_var_declarations]
   single_resource_declaration
 'END_RESOURCE'

single_resource_declaration ::= 
{task_configuration ';'}
program_configuration ';'
{program_configuration ';'}

resource_name ::= identifier

access_declarations ::= 
'VAR_ACCESS'
 access_declaration ';'
 {access_declaration ';'} 
'END_VAR'

access_declaration ::= access_name ':' access_path ':' non_generic_type_name [direction]

access_path ::= [resource_name '.'] direct_variable 
| [resource_name '.'] [program_name '.']
    {fb_name'.'} symbolic_variable

global_var_reference ::= 
[resource_name '.'] global_var_name ['.' structure_element_name]

access_name ::= identifier

program_output_reference ::= program_name '.' symbolic_variable

program_name ::= identifier

direction ::= 'READ_WRITE' | 'READ_ONLY'

task_configuration ::= 'TASK' task_name task_initialization

task_name := identifier

task_initialization ::= 
'(' ['SINGLE' ':=' data_source ',']
    ['INTERVAL' ':=' data_source ','] 
    'PRIORITY' ':=' integer ')'

data_source ::= constant | global_var_reference | program_output_reference | direct_variable

program_configuration ::= 
'PROGRAM' [RETAIN | NON_RETAIN] 
  program_name ['WITH' task_name] ':' program_type_name
  ['(' prog_conf_elements ')']

prog_conf_elements ::= prog_conf_element {',' prog_conf_element}

prog_conf_element ::= fb_task | prog_cnxn

fb_task ::= fb_name 'WITH' task_name

prog_cnxn ::= symbolic_variable ':=' prog_data_source | symbolic_variable '=>' data_sink

prog_data_source ::= 
constant | enumerated_value | global_var_reference | direct_variable

data_sink ::= global_var_reference | direct_variable

instance_specific_initializations ::= 
'VAR_CONFIG'
  instance_specific_init ';'
  {instance_specific_init ';'}
'END_VAR'

instance_specific_init ::= 
resource_name '.' program_name '.' {fb_name '.'} 
((variable_name [location] ':' located_var_spec_init) | 
 (fb_name ':' function_block_type_name ':=' structure_initialization))

SEMANTICS:  See 2.7.

NOTE	This syntax does not reflect the fact that location assignments are only allowed for references to variables which are marked by the asterisk notation at type declaration level.

## B.2  Language IL (Instruction List)

### B.2.1  Instructions and operands

PRODUCTION RULES:

instruction_list ::= il_instruction {il_instruction}

il_instruction ::= [label':'] [  il_simple_operation
 | il_expression
 | il_jump_operation
 | il_fb_call
 | il_formal_funct_call
 | il_return_operator      ]     EOL {EOL}

label ::= identifier

il_simple_operation ::= ( il_simple_operator [il_operand] )
| ( function_name [il_operand_list] )

il_expression ::= il_expr_operator '(' [il_operand] EOL {EOL} [simple_instr_list] ')'

il_jump_operation ::= il_jump_operator label

il_fb_call ::= il_call_operator fb_name ['('
(EOL {EOL} [ il_param_list ]) | [ il_operand_list ] ')']

il_formal_funct_call ::= function_name '(' EOL {EOL} [il_param_list] ')'

il_operand ::= constant | variable | enumerated_value

il_operand_list ::= il_operand {',' il_operand}

simple_instr_list ::= il_simple_instruction {il_simple_instruction}

il_simple_instruction ::= 
(il_simple_operation | il_expression | il_formal_funct_call)
EOL {EOL}

il_param_list ::= {il_param_instruction} il_param_last_instruction

il_param_instruction ::= (il_param_assignment | il_param_out_assignment) ',' EOL {EOL}

il_param_last_instruction ::= 
( il_param_assignment | il_param_out_assignment ) EOL {EOL}

il_param_assignment ::= il_assign_operator ( il_operand | ( '(' EOL {EOL} simple_instr_list ')' ) )

il_param_out_assignment ::= il_assign_out_operator variable

### B.2.2  Operators

PRODUCTION RULES:

il_simple_operator ::=   'LD' | 'LDN' | 'ST' | 'STN' | 'NOT' | 'S'
| 'R' | 'S1' | 'R1' | 'CLK' | 'CU' | 'CD' | 'PV'
| 'IN' | 'PT' | il_expr_operator

il_expr_operator ::=  'AND' | '&' | 'OR' | 'XOR' | 'ANDN' | '&N' | 'ORN' | 'XORN' | 'ADD' | 'SUB' | 'MUL' | 'DIV' | 'MOD' | 'GT' | 'GE' | 'EQ ' | 'LT' | 'LE' | 'NE'

il_assign_operator ::= variable_name':='

il_assign_out_operator ::= ['NOT'] variable_name'=>'

il_call_operator ::= 'CAL' | 'CALC' | 'CALCN'

il_return_operator ::= 'RET' | 'RETC' | 'RETCN'

il_jump_operator ::= 'JMP' | 'JMPC' | 'JMPCN'

SEMANTICS: See subclause 3.2. This syntax does not reflect the possibility for typing IL operators as noted in Table 52.

## B.3  Language ST (Structured Text)

### B.3.1  Expressions

PRODUCTION RULES:

expression ::= xor_expression {'OR' xor_expression}

xor_expression ::= and_expression {'XOR' and_expression}

and_expression ::= comparison {('&' | 'AND') comparison}

comparison ::= equ_expression { ('=' | '<>') equ_expression}

equ_expression ::= add_expression {comparison_operator add_expression}

comparison_operator ::= '<' | '>' | '<=' | '>=' '

add_expression ::= term {add_operator term}

add_operator ::= '+' | '-'

term ::= power_expression  {multiply_operator power_expression}

multiply_operator ::= '*' | '/' | 'MOD'

power_expression ::= unary_expression {'**' unary_expression}

unary_expression ::= [unary_operator] primary_expression

unary_operator ::= '-' | 'NOT'

primary_expression ::= 
constant | enumerated_value | variable | '(' expression ')' | function_name '(' param_assignment {',' param_assignment} ')'

SEMANTICS:  These definitions have been arranged to show a top-down derivation of expression structure.  The precedence of operations is then implied by a "bottom-up" reading of the definitions of the various kinds of expressions.  Further discussion of the semantics of these definitions is given in 3.3.1.  See 2.5.1.1 for details of the semantics of function calls.

### B.3.2  Statements

PRODUCTION RULE:

statement_list ::= statement ';' {statement ';'}

statement ::= NIL | assignment_statement | subprogram_control_statement | selection_statement | iteration_statement

SEMANTICS:  See 3.3.2.

#### B.3.2.1  Assignment statements

PRODUCTION RULE:

assignment_statement ::= variable ':=' expression

SEMANTICS:  See 3.3.2.1.

#### B.3.2.2  Subprogram control statements

PRODUCTION RULES:

subprogram_control_statement ::= fb_invocation | 'RETURN'

fb_invocation ::= fb_name '(' [param_assignment {',' param_assignment}] ')'

param_assignment ::= ([variable_name ':='] expression) | (['NOT'] variable_name '=>' variable)

SEMANTICS:  See 3.3.2.2.

#### B.3.2.3  Selection statements

PRODUCTION RULES:

selection_statement ::= if_statement | case_statement

if_statement ::= 
'IF' expression 'THEN' statement_list
  {'ELSIF' expression 'THEN' statement_list}
  ['ELSE' statement_list]
'END_IF'

case_statement ::= 
'CASE' expression 'OF'
  case_element 
  {case_element}
  ['ELSE' statement_list]
'END_CASE'

case_element ::= case_list ':' statement_list

case_list ::= case_list_element {',' case_list_element}

case_list_element ::= subrange | signed_integer | enumerated_value

SEMANTICS:  See 3.3.2.3.

#### B.3.2.4  Iteration statements

PRODUCTION RULES:

iteration_statement ::= 
for_statement | while_statement | repeat_statement | exit_statement

for_statement ::= 
'FOR' control_variable ':=' for_list 'DO' statement_list 'END_FOR'

control_variable ::= identifier

for_list ::= expression 'TO' expression ['BY' expression]

while_statement ::= 'WHILE' expression 'DO' statement_list 'END_WHILE'

repeat_statement ::= 
'REPEAT' statement_list 'UNTIL' expression 'END_REPEAT'

exit_statement ::= 'EXIT'

SEMANTICS:  See 3.3.2.4.

# ANNEX C - Delimiters and Keywords (normative)

The usages of delimiters and keywords in IEC 61131-3 is summarized in tables C.1 and C.2.  National standards organizations can publish tables of translations for the textual portions of the delimiters listed in table C.1 and the keywords listed in table C.2.

| Table C.1 - Delimiters |  |  |
| --- | --- | --- |
| Delimiters | Clause | Usage |
| Space | 2.1.4 | As specified in 2.1.4. |
| (* | 2.1.5 | Begin comment |
| *) |  | End comment |
| + | 2.2.1 | Leading sign of decimal literal |
|  | 3.3.1 | Addition operator |
|  | 2.2.1 | Leading sign of decimal literal |
| - | 2.2.3.2 | Year-month-day separator |
|  | 3.3.1 | Subtraction, negation operator |
|  | 4.1.1 | Horizontal line |
| # | 2.2.1 | Based number separator |
|  | 2.2.3 | Time literal separator |
| . | 2.2.1 | Integer/fraction separator |
|  | 2.4.1.1 | Hierarchical address separator |
|  | 2.4.1.2 | Structure element separator |
|  | 2.5.2.1 | Function block structure separator |
| e or E | 2.2.1 | Real exponent delimiter |
| ' | 2.2.2 | Start and end of character string |
| $ | 2.2.2 | Start of special character in strings |
| 2.2.3 - Time literal delimiters, including: T#, D, H, M, S, MS, DATE#, D#, TIME_OF_DAY#, TOD#, DATE_AND_TIME#, DT# |  |  |
|  | 2.2.3.2 | Time of day separator |
|  | 2.3.3.1 | Type name/specification separator |
|  | 2.4.2 | Variable/type separator |
|  | 2.6.2 | Step name terminator |
| : | 2.7 | RESOURCE name/type separator |
|  | 2.7 | PROGRAM name/type separator |
|  | 2.7 3.2.1 4.1.2 | Access name/path/type separator Instruction label terminator Network label terminator |
| := | 2.3.3.1 2.7.1 3.3.2.1 | Initialization operator Input connection operator Assignment operator |
| () | 2.3.3.1 | Enumeration list delimiters |
| () | 2.3.3.1 | Subrange delimiters |
| [] | 2.4.1.2 | Array subscript delimiters |
| [] | 2.4.2 | String length delimiters |
| () | 2.4.2 | Multiple initialization |
| () | 3.2.2 | Instruction List modifier/operator |
| () | 3.3.1 | Function arguments |
| () | 3.3.1 | Subexpression hierarchy |
| () | 3.3.2.2 | Function block input list delimiters |
|  | 2.3.3.1 | Enumeration list separator |
|  | 2.3.3.2 | Initial value separator |
|  | 2.4.1 | Array subscript separator |
|  | 2.4.2 | Declared variable separator |
| , | 2.5.2.1 | Function block initial value separator |
|  | 2.5.2.1 | Function block input list separator |
|  | 3.2.1 | Operand list separator |
|  | 3.3.1 | Function argument list separator |
|  | 3.3.2.3 | CASE value list separator |
| ; | 2.3.3.1 | Type declaration separator |
|  | 3.3 | Statement separator |
| .. | 2.3.3.1 | Subrange separator |
|  | 3.3.2.3 | CASE range separator |
| % | 2.4.1.1 | Direct representation prefix |
| => | 2.7.1 | Output connection operator |
| 3.3.1 - Infix operators, including: **, NOT, *, /, MOD, +, -, <, >, <= >=, =, <>, &, AND, XOR, OR |  |  |
| ¦ or ! | 4.1.1 | Vertical lines |

| Table C.2 - Keywords |  |
| --- | --- |
| Keywords | Clause |
| ACTION...END_ACTION | 2.6.4.1 |
| ARRAY...OF | 2.3.3.1 |
| AT | 2.4.3 |
| CASE...OF...ELSE...END_CASE | 3.3.2.3 |
| CONFIGURATION...END_CONFIGURATION | 2.7.1 |
| CONSTANT | 2.4.3 |
| Data type names | 2.3 |
| EN, ENO | 2.5.1.2 |
| EXIT | 3.3.2.4 |
| FALSE | 2.2.1 |
| F_EDGE | 2.5.2.2 |
| FOR...TO...BY...DO...END_FOR | 3.3.2.4 |
| FUNCTION...END_FUNCTION | 2.5.1.3 |
| Function names | 2.5.1 |
| FUNCTION_BLOCK...END_FUNCTION_BLOCK | 2.5.2.2 |
| Function Block names | 2.5.2 |
| IF...THEN...ELSIF...ELSE...END_IF | 3.3.2.3 |
| INITIAL_STEP...END_STEP | 2.6.2 |
| NOT, MOD, AND, XOR, OR | 3.3.1 a |
| PROGRAM...WITH... | 2.7.1 |
| PROGRAM...END_PROGRAM | 2.5.3 |
| R_EDGE | 2.5.2.2 |
| READ_ONLY, READ_WRITE | 2.7.1 |
| REPEAT...UNTIL...END_REPEAT | 3.3.2.4 |
| RESOURCE...ON...END_RESOURCE | 2.7.1 |
| RETAIN, NON_RETAIN | 2.4.3 |
| RETURN | 3.3.2.2 |
| STEP...END_STEP | 2.6.2 |
| STRUCT...END_STRUCT | 2.3.3.1 |
| TASK | 2.7.2 |
| TRANSITION...FROM...TO...END_TRANSITION | 2.6.3 |
| TRUE | 2.2.1 |
| TYPE...END_TYPE | 2.3.3.1 |
| VAR...END_VAR | 2.4.3 |
| VAR_INPUT...END_VAR | 2.4.3 |
| VAR_OUTPUT...END_VAR | 2.4.3 |
| VAR_IN_OUT...END_VAR | 2.4.3 |
| VAR_TEMP...END_VAR | 2.4.3 |
| VAR_EXTERNAL...END_VAR | 2.4.3 |
| VAR_ACCESS...END_VAR | 2.7.1 |
| VAR_CONFIG...END_VAR | 2.7.1 |
| VAR_GLOBAL...END_VAR | 2.7.1 |
| WHILE...DO...END_WHILE | 3.3.2.4 |
| WITH | 2.7.1 |

# ANNEX D - Implementation-dependent parameters (normative)

The implementation-dependent parameters defined in IEC 61131-3, and the primary reference clause for each, are listed in table D.1.

NOTE	Other implementation-dependent parameters such as the accuracy, precision and repeatability of timing and execution control features may have significant effects on the portability of programs but are beyond the scope of this Part of the Standard.

| Table D.1 - Implementation-dependent parameters |  |
| --- | --- |
| Clause | Parameters |
| 2.1.2 | Maximum length of identifiers |
| 2.1.5 | Maximum comment length |
| 2.1.6 | Syntax and semantics of pragmas |
| 2.2.2 | Syntax and semantics for the use of the double-quote character when a particular implementation supports Feature #4 but not Feature #2 of Table 5. |
| 2.3.1 | Range of values and precision of representation for variables of type TIME, DATE, TIME_OF_DAY and DATE_AND_TIME |
|  | Precision of representation of seconds in types TIME, TIME_OF_DAY and DATE_AND_TIME |
| 2.3.3.1 | Maximum number of enumerated values Maximum number of array subscripts Maximum array size Maximum number of structure elements Maximum structure size Maximum range of subscript values Maximum number of levels of nested structures |
| 2.3.3.2 | Default maximum length of STRING and WSTRING variables Maximum allowed length of STRING and WSTRING variables |
| 2.4.1.1 | Maximum number of hierarchical levels Logical or physical mapping |
| 2.4.2 | Initialization of system inputs |
| 2.4.3 | Maximum number of variables per declaration Effect of using AT qualifier in declaration of function block instances Warm start behavior if variable is declared as neither RETAIN nor NON_RETAIN |
| 2.5 | Information to determine execution times of program organization units |
| 2.5.1.2 | Values of outputs when ENO is FALSE |
| 2.5.1.3 | Maximum number of function specifications |
| 2.5.1.5 | Maximum number of inputs of extensible functions |
| 2.5.1.5.1 | Effects of type conversions on accuracy Error conditions during type conversions |
| 2.5.1.5.2 | Accuracy of numerical functions |
| 2.5.1.5.6 | Effects of type conversions between time data types and other data types not defined in Table 30 |
| 2.5.2 | Maximum number of function block specifications and instantiations |
| 2.5.2.1a | Function block input variable assignment when EN is FALSE |
| 2.5.2.3.3 | Pvmin, Pvmax of counters |
| 2.5.2.3.4 | Effect of a change in the value of a PT input during a timing operation |
| 2.5.3 | Program size limitations |
| 2.6.2 | Precision of step elapsed time Maximum number of steps per SFC |
| 2.6.3 | Maximum number of transitions per SFC and per step |
| 2.6.4.2 | Maximum number of action blocks per step |
| 2.6.4.5 | Access to the functional equivalent of the Q or A outputs |
| 2.6.5 | Transition clearing time Maximum width of diverge/converge constructs |
| 2.7.1 | Contents of RESOURCE libraries |
| 2.7.1 | Effect of using READ_WRITE access to function block outputs |
| 2.7.2 | Maximum number of tasks Task interval resolution |
| 3.3.1 | Maximum length of expressions |
| 3.3.2 | Maximum length of statements |
| 3.3.2.3 | Maximum number of CASE selections |
| 3.3.2.4 | Value of control variable upon termination of FOR loop |
| 4.1.1 | Restrictions on network topology |
| 4.1.3 | Evaluation order of feedback loops |

# ANNEX E - Error Conditions (normative)

The error conditions defined in IEC 61131-3, and the primary reference clause for each, are listed in table E.1.  These errors may be detected during preparation of the program for execution or during execution of the program.  The manufacturer shall specify the disposition of these errors according to the provisions of subclause 1.5.1 of this part.

| Table E.1 - Error conditions |  |
| --- | --- |
| Clause | Error conditions |
| 2.1.5 | Nested comments |
| 2.3.3.1 | Ambiguous enumerated value |
| 2.3.3.1 | Value of a variable exceeds the specified subrange |
| 2.4.1.1 | Missing configuration of an incomplete address specification ("*" notation) |
| 2.4.3 | Attempt by a program organization unit to modify a variable which has been declared CONSTANT |
| 2.4.3 | Declaration of a variable as VAR_GLOBAL CONSTANT in a containing element having a contained element in which the same variable is declared VAR_EXTERNAL without the CONSTANT qualifier. |
| 2.5.1 | Improper use of directly represented or external variables in functions |
| 2.5.1.1 | A VAR_IN_OUT variable is not "properly mapped" |
| 2.5.1.1 | Ambiguous value caused by a VAR_IN_OUT connection |
| 2.5.1.5.1 | Type conversion errors |
| 2.5.1.5.2 | Numerical result exceeds range for data type Division by zero |
| 2.5.1.5.3 | N input is less than zero in a bit-shift function |
| 2.5.1.5.4 | Mixed input data types to a selection function Selector (K) out of range for MUX function |
| 2.5.1.5.5 | Invalid character position specified Result exceeds maximum string length ANY_INT input is less than zero in a string function |
| 2.5.1.5.6 | Result exceeds range for data type |
| 2.5.2.2 | No value specified for a function block instance used as input variable |
| 2.5.2.2 | No value specified for an in-out variable |
| 2.6.2 | Zero or more than one initial steps in SFC network User program attempts to modify step state or time |
| 2.6.3 | Side effects in evaluation of transition condition |
| 2.6.4.5 | Action control contention error |
| 2.6.5 | Simultaneously true, non-prioritized transitions in a selection divergence Unsafe or unreachable SFC |
| 2.7.1 | Data type conflict in VAR_ACCESS |
| 2.7.2 | A task fails to be scheduled or to meet its execution deadline |
| 3.2.2 | Numerical result exceeds range for data type Current result and operand not of same data type |
| 3.3.1 | Division by zero Numerical result exceeds range for data type Invalid data type for operation |
| 3.3.2.1 | Return from function without value assigned |
| 3.3.2.4 | Iteration fails to terminate |
| 4.1.1 | Same identifier used as connector label and element name |
| 4.1.3 | Uninitialized feedback variable |

# ANNEX F -  Examples (informative)

## F.1  Function WEIGH

Example function WEIGH provides the functions of BCD-to-binary conversion of a gross-weight input from a scale, the binary integer subtraction of a tare weight which has been previously converted and stored in the memory of the programmable controller, and the conversion of the resulting net weight back to BCD form, e.g., for an output display.  The "EN" input is used to indicate that the scale is ready to perform the weighing operation.

The "ENO" output indicates that an appropriate command exists (e.g., from an operator pushbutton), the scale is in proper condition for the weight to be read, and each function has a correct result.

A textual form of the declaration of this function is:

FUNCTION WEIGH : WORD (* BCD encoded *) VAR_INPUT (* "EN" input is used to indicate "scale ready" *) weigh_command : BOOL; gross_weight : WORD ; (* BCD encoded *) tare_weight : INT ; END_VAR (* Function Body *) END_FUNCTION (* Implicit "ENO" *)

The body of function WEIGH in the IL language is:

|  | LD | weigh_command |  |
| --- | --- | --- | --- |
|  | JMPC | WEIGH_NOW |  |
|  | ST | ENO | (* No weighing, 0 to "ENO" *) |
|  | RET |  |  |
| WEIGH_NOW: | LD | gross_weight |  |
|  | BCD_TO_INT |  |  |
|  | SUB | tare_weight |  |
|  | INT_TO_BCD |  | (* Return evaluated weight *) |
|  | ST | WEIGH |  |

The body of function WEIGH in the ST language is:

IF weigh_command THEN WEIGH := INT_TO_BCD (BCD_TO_INT(gross_weight) - tare_weight); END_IF ;

An equivalent graphical declaration of function WEIGH is:

+-------------------------+ ¦ WEIGH ¦ BOOL---¦EN ENO¦---BOOL BOOL---¦weigh_command ¦---WORD WORD---¦gross_weight ¦ INT----¦tare_weight ¦ +-------------------------+

The function body in the LD language is:

¦ +--------+ +--------+ ¦ ¦ ¦ BCD_ ¦ +-------+ ¦ INT_ ¦ ¦ ¦ weigh_command ¦ TO_INT ¦ ¦ SUB ¦ ¦ TO_BCD ¦ ENO ¦ +-------¦ ¦-----¦EN ENO¦--¦EN ENO¦---¦EN ENO¦----( )------+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ ¦ gross_weight--¦ ¦--¦ ¦---¦ ¦--WEIGH ¦ ¦ +--------+ ¦ ¦ +--------+ ¦ ¦ tare_weight---------------¦ ¦ ¦ ¦ +-------+ ¦

The function body in the FBD language is:

+--------+ +--------+ ¦ BCD_ ¦ +-------+ ¦ INT_ ¦ ¦ TO_INT ¦ ¦ SUB ¦ ¦ TO_BCD ¦ weigh_command---¦EN ENO¦---¦EN ENO¦---¦EN ENO¦---ENO gross_weight----¦ ¦---¦ ¦---¦ ¦--WEIGH +--------+ ¦ ¦ +--------+ tare_weight------------------¦ ¦ +-------+

## F.2  Function block CMD_MONITOR

Example function block CMD_MONITOR illustrates the control of an operative unit which is capable of responding to a Boolean command (the CMD output) and returning a Boolean feedback signal (the FDBK input) indicating successful completion of the commanded action.  The function block provides for manual control via the MAN_CMD input, or automated control via the AUTO_CMD input, depending on the state of the AUTO_MODE input (0 or 1 respectively).  Verification of the MAN_CMD input is provided via the MAN_CMD_CHK input, which must be 0 in order to enable the MAN_CMD input.

If confirmation of command completion is not received on the FDBK input within a predetermined time specified by the T_CMD_MAX input, the command is cancelled and an alarm condition is signalled via the ALRM output.  The alarm condition may be cancelled by the ACK (acknowledge) input, enabling further operation of the command cycle.

A textual form of the declaration of function block CMD_MONITOR is:

FUNCTION_BLOCK CMD_MONITOR VAR_INPUT AUTO_CMD : BOOL ; (* Automated command *) AUTO_MODE : BOOL ; (* AUTO_CMD enable *) MAN_CMD : BOOL ; (* Manual Command *) MAN_CMD_CHK : BOOL ; (* Negated MAN_CMD to debounce *) T_CMD_MAX : TIME ; (* Max time from CMD to FDBK *) FDBK : BOOL ; (* Confirmation of CMD completion by operative unit *) ACK : BOOL ; (* Acknowledge/cancel ALRM *) END_VAR VAR_OUTPUT CMD : BOOL ; (* Command to operative unit *) ALRM : BOOL ; (* T_CMD_MAX expired without FDBK *) END_VAR VAR CMD_TMR : TON ; (* CMD-to-FDBK timer *) ALRM_FF : SR ; (* Note over-riding S input: *) END_VAR (* Command must be cancelled before "ACK" can cancel alarm *) (* Function Block Body *) END_FUNCTION_BLOCK

An equivalent graphical declaration is:

+---------------+ ¦ CMD_MONITOR ¦ BOOL---¦AUTO_CMD CMD¦---BOOL BOOL---¦AUTO_MODE ALRM¦---BOOL BOOL---¦MAN_CMD ¦ BOOL---¦MAN_CMD_CHK ¦ TIME---¦T_CMD_MAX ¦ BOOL---¦FDBK ¦ BOOL---¦ACK ¦ +---------------+

The body of function block CMD_MONITOR in the ST language is:

CMD := AUTO_CMD & AUTO_MODE OR MAN_CMD & NOT MAN_CMD_CHK & NOT AUTO_MODE ; CMD_TMR (IN := CMD, PT := T_CMD_MAX); ALRM_FF (S1 := CMD_TMR.Q & NOT FDBK, R := ACK); ALRM := ALRM_FF.Q1;

The body of function block CMD_MONITOR in the IL language is:

| LD | T_CMD_MAX |  |
| --- | --- | --- |
| ST | CMD_TMR.PT | (* Store an input to the TON FB *) |
| LD | AUTO_CMD |  |
| AND | AUTO_MODE |  |
| OR( | MAN_CMD |  |
| ANDN | AUTO_MODE |  |
| ANDN | MAN_CMD_CHK |  |
| ) |  |  |
| ST | CMD |  |
| IN | CMD_TMR | (* Invoke the TON FB *) |
| LD | CMD_TMR.Q |  |
| ANDN | FDBK |  |
| ST | ALRM_FF.S1 | (* Store an input to the SR FB *) |
| LD | ACK |  |
| R | ALRM_FF | (* Invoke the SR FB *) |
| LD | ALRM_FF.Q1 |  |
| ST | ALRM |  |

The body of function block CMD_MONITOR in the LD language is:

¦ ¦ ¦ AUTO_MODE AUTO_CMD CMD ¦ +--¦ ¦--------¦ ¦-------------------+---( )--+ ¦ ¦ ¦ ¦ AUTO_MODE MAN_CMD MAN_CMD_CHECK ¦ ¦ +--¦/¦-------¦ ¦------¦/¦-----------+ ¦ ¦ ¦ ¦ ACK ALRM ¦ +--¦ ¦---------------------------------(R)---+ ¦ CMD_TMR ¦ ¦ +-----+ ¦ ¦ CMD ¦ TON ¦ FDBK ALRM ¦ +--¦ ¦-------¦IN Q¦------¦/¦----------(S)---+ ¦ T_CMD_MAX--¦PT ET¦ ¦ ¦ +-----+ ¦ ¦ ¦

The body of function block CMD_MONITOR in the FBD language is:

+-+ +---+ AUTO_CMD------¦&¦----¦>=1¦--+-------------------------------CMD AUTO_MODE--+--¦ ¦ +--¦ ¦ ¦ ¦ +-+ ¦ +---+ ¦ ¦ ¦ ¦ ¦ +-+ ¦ ¦ CMD_TMR ALRM_FF +-O¦&¦ ¦ ¦ +-----+ +-----+ MAN_CMD-------¦ ¦-+ ¦ ¦ TON ¦ +-+ ¦ SR ¦ MAN_CMD_CHK--O¦ ¦ +--¦IN Q¦------¦&¦----¦S1 Q1¦--ALRM +-+ ¦ ¦ +--O¦ ¦ +--¦R ¦ T_CMD_MAX----------------------¦PT ET¦ ¦ +-+ ¦ +-----+ +-----+ ¦ ¦ FDBK------------------------------------+ ¦ ACK---------------------------------------------+

## F.3  Function block FWD_REV_MON

Example function block FWD_REV_MON illustrates the control of an operative unit capable of two-way positioning action, e.g., a motor-operated valve.  Both automated and manual control modes are possible, with alarm capabilities provided for each direction of motion, as described for function block CMD_MONITOR above.  In addition, contention between forward and reverse commands causes the cancellation of both commands and signalling of an alarm condition.  The Boolean OR of all alarm conditions is made available as a KLAXON output for operator signaling.

A graphical declaration of this function block is:

+----------------------+ ¦ FWD_REV_MON ¦ BOOL---¦AUTO KLAXON¦---BOOL BOOL---¦ACK FWD_REV_ALRM¦---BOOL BOOL---¦AUTO_FWD FWD_CMD¦---BOOL BOOL---¦MAN_FWD FWD_ALRM¦---BOOL BOOL---¦MAN_FWD_CHK ¦ TIME---¦T_FWD_MAX ¦ BOOL---¦FWD_FDBK ¦ BOOL---¦AUTO_REV REV_CMD¦---BOOL BOOL---¦MAN_REV REV_ALRM¦---BOOL BOOL---¦MAN_REV_CHK ¦ TIME---¦T_REV_MAX ¦ BOOL---¦REV_FDBK ¦ +----------------------+

A textual form of the declaration of function block FWD_REV_MON is:

FUNCTION_BLOCK FWD_REV_MON VAR_INPUT AUTO : BOOL ;(* Enable automated commands *) ACK : BOOL ; (* Acknowledge/cancel all alarms *) AUTO_FWD : BOOL ; (* Automated forward command *) MAN_FWD : BOOL ; (* Manual forward command *) MAN_FWD_CHK : BOOL ; (* Negated MAN_FWD for debouncing *) T_FWD_MAX : TIME ; (* Maximum time from FWD_CMD to FWD_FDBK *) FWD_FDBK : BOOL ; (* Confirmation of FWD_CMD completion *) (* by operative unit *) AUTO_REV : BOOL ; (* Automated reverse command *) MAN_REV : BOOL ; (* Manual reverse command *) MAN_REV_CHK : BOOL ; (* Negated MAN_REV for debouncing *) T_REV_MAX : TIME ; (* Maximum time from REV_CMD to REV_FDBK *) REV_FDBK : BOOL ; (* Confirmation of REV_CMD completion *) END_VAR (* by operative unit *) VAR_OUTPUT KLAXON : BOOL ; (* Any alarm active *) FWD_REV_ALRM : BOOL; (* Forward/reverse command conflict *) FWD_CMD : BOOL ; (* "Forward" command to operative unit *) FWD_ALRM : BOOL ; (* T_FWD_MAX expired without FWD_FDBK *) REV_CMD : BOOL ; (* "Reverse" command to operative unit *) REV_ALRM : BOOL ; (* T_REV_MAX expired without REV_FDBK *) END_VAR VAR FWD_MON : CMD_MONITOR; (* "Forward" command monitor *) REV_MON : CMD_MONITOR; (* "Reverse" command monitor *) FWD_REV_FF : SR ; (* Forward/Reverse contention latch *) END_VAR (* Function Block body *) END_FUNCTION_BLOCK

The body of function block FWD_REV_MON can be written in the ST language as:

(* Evaluate internal function blocks *) FWD_MON (AUTO_MODE := AUTO, ACK := ACK, AUTO_CMD := AUTO_FWD, MAN_CMD := MAN_FWD, MAN_CMD_CHK := MAN_FWD_CHK, T_CMD_MAX := T_FWD_MAX, FDBK := FWD_FDBK); REV_MON (AUTO_MODE := AUTO, ACK := ACK, AUTO_CMD := AUTO_REV, MAN_CMD := MAN_REV, MAN_CMD_CHK := MAN_REV_CHK, T_CMD_MAX := T_REV_MAX, FDBK := REV_FDBK); FWD_REV_FF (S1 := FWD_MON.CMD & REV_MON.CMD, R := ACK); (* Transfer data to outputs *) FWD_REV_ALRM := FWD_REV_FF.Q1; FWD_CMD := FWD_MON.CMD & NOT FWD_REV_ALRM; FWD_ALRM := FWD_MON.ALRM; REV_CMD := REV_MON.CMD & NOT FWD_REV_ALRM; REV_ALRM := REV_MON.ALRM; KLAXON := FWD_ALRM OR REV_ALRM OR FWD_REV_ALRM;

The body of function block FWD_REV_MON in the IL language is:

(* Evaluate internal function blocks *) CAL FWD_MON( AUTO_MODE:= AUTO, ACK:= ACK, AUTO_CMD:= AUTO_FWD, MAN_CMD:= MAN_FWD, MAN_CMD_CHK:= MAN_FWD_CHK, T_CMD_MAX:= T_FWD_MAX, FDBK:= FWD_FDBK ) CAL REV_MON( AUTO_MODE:= AUTO, ACK:= ACK, AUTO_CMD:= AUTO_REV, MAN_CMD:= MAN_REV, MAN_CMD_CHK:= MAN_REV_CHK, T_CMD_MAX:= T_REV_MAX, FDBK:= REV_FDBK ) CAL FWD_REV_FF( S1:=( LD FWD_MON.CMD AND REV_MON.CMD ), R:= ACK, Q => FWD_REV_ALRM (* Contention alarm *) ) (* Transfer data to outputs *) LD FWD_MON.CMD (* "Forward" command and alarm *) ANDN FWD_REV_ALRM ST FWD_CMD LD FWD_MON.ALRM ST FWD_ALRM LD REV_MON.CMD (* "Reverse" command and alarm *) ANDN FWD_REV_ALRM ST REV_CMD LD REV_MON.ALRM ST REV_ALRM OR FWD_ALRM (* OR all alarms *) OR FWD_REV_ALRM ST KLAXON

The body of function block FWD_REV_MON in the FBD language is:

FWD_MON +---------------+ ¦ CMD_MONITOR ¦ AUTO_FWD----------¦AUTO_CMD CMD¦--+ AUTO---------+----¦AUTO_MODE ALRM¦--¦-------FWD_ALRM MAN_FWD------¦----¦MAN_CMD ¦ ¦ MAN_FWD_CHK--¦----¦MAN_CMD_CHK ¦ ¦ FWD_FDBK-----¦----¦FDBK ¦ ¦ ACK----------¦-+--¦ACK ¦ ¦ T_FWD_MAX----¦-¦--¦T_CMD_MAX ¦ ¦ +---+ ¦ ¦ +---------------+ +--¦ & ¦-------------+ ¦ ¦ +--¦ ¦ ¦ ¦ ¦ REV_MON ¦ +---+ ¦ ¦ ¦ +---------------+ ¦ ¦ ¦ ¦ ¦ CMD_MONITOR ¦ ¦ ¦ AUTO_REV-----¦-¦--¦AUTO_CMD CMD¦--+ ¦ +-¦--¦AUTO_MODE ALRM¦---------REV_ALRM ¦ MAN_REV--------¦--¦MAN_CMD ¦ ¦ MAN_REV_CHK----¦--¦MAN_CMD_CHK ¦ ¦ REV_FDBK-------¦--¦FDBK ¦ ¦ +--¦ACK ¦ ¦ T_REV_MAX---------¦T_CMD_MAX ¦ ¦ +---------------+ ¦ +------------------------------------------------+ ¦ FWD_REV_FF ¦ +------+ ¦ ¦ SR ¦ +-----¦S1 Q1¦--+----------------FWD_REV_ALRM ACK------------¦R ¦ ¦ +------+ ¦ +-----+ +---¦ >=1 ¦------KLAXON FWD_MON.ALRM-------------¦---¦ ¦ REV_MON.ALRM-------------¦---¦ ¦ ¦ +-----+ ¦ ¦ +---+ +--O¦ & ¦--------FWD_CMD FWD_MON.CMD--------------¦---¦ ¦ ¦ +---+ ¦ ¦ +---+ +--O¦ & ¦--------REV_CMD REV_MON.CMD------------------¦ ¦ +---+

The body of function block FWD_REV_MON in the LD language is:

¦ FWD_MON ¦ ¦ +---------------+ ¦ ¦ AUTO_FWD ¦ CMD_MONITOR ¦ ¦ +--¦ ¦---------¦AUTO_CMD CMD¦ ¦ ¦ AUTO ¦ ¦ FWD_ALRM ¦ +--¦ ¦---------¦AUTO_MODE ALRM¦-------( )---+ ¦ MAN_FWD ¦ ¦ ¦ +--¦ ¦---------¦MAN_CMD ¦ ¦ ¦ MAN_FWD_CHK ¦ ¦ ¦ +--¦ ¦---------¦MAN_CMD_CHK ¦ ¦ ¦ FWD_FDBK ¦ ¦ ¦ +--¦ ¦---------¦FDBK ¦ ¦ ¦ ACK ¦ ¦ ¦ +--¦ ¦---------¦ACK ¦ ¦ ¦ ¦ ¦ ¦ ¦ T_FWD_MAX---¦T_CMD_MAX ¦ ¦ ¦ +---------------+ ¦ ¦ ¦ ¦ REV_MON ¦ ¦ +---------------+ ¦ ¦ AUTO_REV ¦ CMD_MONITOR ¦ ¦ +--¦ ¦---------¦AUTO_CMD CMD¦ ¦ ¦ AUTO ¦ ¦ REV_ALRM ¦ +--¦ ¦---------¦AUTO_MODE ALRM¦-------( )---+ ¦ MAN_REV ¦ ¦ ¦ +--¦ ¦---------¦MAN_CMD ¦ ¦ ¦ MAN_REV_CHK ¦ ¦ ¦ +--¦ ¦---------¦MAN_CMD_CHK ¦ ¦ ¦ REV_FDBK ¦ ¦ ¦ +--¦ ¦---------¦FDBK ¦ ¦ ¦ ACK ¦ ¦ ¦ +--¦ ¦---------¦ACK ¦ ¦ ¦ ¦ ¦ ¦ ¦ T_REV_MAX---¦T_CMD_MAX ¦ ¦ ¦ +---------------+ ¦ ¦ ¦ ¦ ACK FWD_REV_ALRM ¦ +-----¦ ¦--------------------------(R)-------+ ¦ ¦ ¦ FWD_MON.CMD REV_MON.CMD FWD_REV_ALRM ¦ +-----¦ ¦-----------¦ ¦------------(S)-------+ ¦ ¦ ¦ FWD_MON.CMD FWD_REV_ALRM FWD_CMD ¦ +-----¦ ¦-----------¦/¦-------------( )------+ ¦ ¦ ¦ REV_MON.CMD FWD_REV_ALRM REV_CMD ¦ +-----¦ ¦-----------¦/¦-------------( )------+ ¦ ¦

¦ FWD_REV_ALRM KLAXON ¦ +-----¦ ¦------+--------------------( )------+ ¦ ¦ ¦ ¦ FWD_ALRM ¦ ¦ +-----¦ ¦------+ ¦ ¦ ¦ ¦ ¦ REV_ALRM ¦ ¦ +-----¦ ¦------+ ¦ ¦ ¦

## F.4  Function block STACK_INT

This function block provides a stack of up to 128 integers.  The usual stack operations of PUSH and POP are provided by edge-triggered Boolean inputs.  An overriding reset (R1) input is provided; the maximum stack depth (N) is determined at the time of resetting.  In addition to the top-of-stack data (OUT), Boolean outputs are provided indicating stack empty and stack overflow states.

A textual form of the declaration of this function block is:

FUNCTION_BLOCK STACK_INT VAR_INPUT PUSH, POP: BOOL R_EDGE; (* Basic stack operations *) R1 : BOOL ; (* Over-riding reset *) IN : INT ; (* Input to be pushed *) N : INT ; (* Maximum depth after reset *) END_VAR VAR_OUTPUT EMPTY : BOOL := 1 ; (* Stack empty *) OFLO : BOOL := 0 ; (* Stack overflow *) OUT : INT := 0 ; (* Top of stack data *) END_VAR VAR STK : ARRAY[0..127] OF INT; (* Internal stack *) NI : INT :=128 ; (* Storage for N upon reset *) PTR : INT := -1 ; (* Stack pointer *) END_VAR (* Function Block body *) END_FUNCTION_BLOCK

A graphical declaration of function block STACK_INT is:

+-----------+ ¦ STACK_INT ¦ BOOL--->PUSH EMPTY¦---BOOL BOOL--->POP OFLO¦---BOOL BOOL---¦R1 OUT¦---INT INT----¦IN ¦ INT----¦N ¦ +-----------+ (* Internal variable declarations *) VAR STK : ARRAY[0..127] OF INT ; (* Internal Stack *) NI : INT :=128 ; (* Storage for N upon Reset *) PTR : INT := -1 ; (* Stack Pointer *) END_VAR

The body of function block STACK_INT in the ST language is:

IF R1 THEN OFLO := 0; EMPTY := 1; PTR := -1; NI := LIMIT (MN:=1,IN:=N,MX:=128); OUT := 0; ELSIF POP & NOT EMPTY THEN OFLO := 0; PTR := PTR-1; EMPTY := PTR < 0; IF EMPTY THEN OUT := 0; ELSE OUT := STK[PTR]; END_IF ; ELSIF PUSH & NOT OFLO THEN EMPTY := 0; PTR := PTR+1; OFLO := (PTR = NI); IF NOT OFLO THEN OUT := IN ; STK[PTR] := IN; ELSE OUT := 0; END_IF ; END_IF ;

The body of function block STACK_INT in the LD language is:

¦ ¦ ¦ R1 ¦ +---¦ ¦--->>RESET ¦ ¦ ¦ ¦ POP EMPTY ¦ +--¦ ¦---¦/¦--->>POP_STK ¦ ¦ ¦ ¦ PUSH OFLO ¦ +--¦ ¦---¦/¦--->>PUSH_STK ¦ ¦ ¦ ¦ ¦ +--------------<RETURN> ¦

RESET: ¦ +--------+ +--------+ +-------+ ¦ ¦ ¦ MOVE ¦ ¦ MOVE ¦ ¦ LIMIT ¦ OFLO ¦ +------¦EN ENO¦-----------¦EN ENO¦-----------¦EN ENO¦--+---(R)---+ ¦ 0---¦ ¦--OUT -1 --¦ ¦--PTR 128--¦MX ¦ ¦ EMPTY ¦ ¦ +--------+ +--------+ N--¦IN ¦ +---(S)---+ ¦ 1--¦MN ¦--NI ¦ ¦ +-------+ ¦ +-----------------<RETURN>

POP_STK: ¦ +--------+ +--------+ ¦ ¦ ¦ SUB ¦ ¦ LT ¦ ¦ +------------¦EN ENO¦-------¦EN ENO¦ EMPTY ¦ ¦ PTR--¦ ¦--PTR--¦ ¦----(S)---+ ¦ 1--¦ ¦ 0--¦ ¦ ¦ ¦ +--------+ +--------+ ¦ ¦ ¦ ¦ +-------+ ¦ ¦ ¦ SEL ¦ OFLO ¦ +------------------¦EN ENO¦---------------(R)----+ ¦ EMPTY ¦ ¦ ¦ +---¦ ¦------------¦G ¦---OUT ¦ ¦ STK[PTR]---¦IN0 ¦ ¦ ¦ 0 ---¦IN1 ¦ ¦ ¦ +-------+ ¦ +--------------------------------<RETURN> ¦

PUSH_STK: ¦ ¦ ¦ +--------+ +--------+ ¦ ¦ ¦ ADD ¦ ¦ EQ ¦ ¦ +------------¦EN ENO¦-------¦EN ENO¦ OFLO ¦ ¦ PTR--¦ ¦--PTR--¦ ¦----(S)---+ ¦ 1--¦ ¦ NI--¦ ¦ ¦ ¦ +--------+ +--------+ ¦ ¦ ¦ ¦ +------+ ¦ ¦ OFLO ¦ MOVE ¦ ¦ +---¦/¦-------¦EN ENO¦----------------------------+ ¦ IN---¦ ¦---STK[PTR] ¦ ¦ +------+ ¦ ¦ ¦ ¦ +-------+ ¦ ¦ ¦ SEL ¦ EMPTY ¦ +-------------¦EN ENO¦--------------------(R)----+ ¦ OFLO ¦ ¦ ¦ +---¦ ¦-------¦G ¦---OUT ¦ ¦ IN---¦IN0 ¦ ¦ ¦ 0 ---¦IN1 ¦ ¦ ¦ +-------+ ¦

The body of function block STACK_INT in the IL language is:

|  | LD | R1 | (* Dispatch on operations *) |
| --- | --- | --- | --- |
|  | JMPC | RESET |  |
|  | LD | POP |  |
|  | ANDN | EMPTY | (* Don't pop empty stack *) |
|  | JMPC | POP_STK |  |
|  | LD | PUSH |  |
|  | ANDN | OFLO | (* Don't push overflowed stack *) |
|  | JMPC | PUSH_STK |  |
|  | RET |  | (* Return if no operations active *) |
| RESET: | LD | 0 | (* Stack reset operations *) |
|  | ST | OFLO |  |
|  | LD | 1 |  |
|  | ST | EMPTY |  |
|  | LD | -1 |  |
|  | ST | PTR |  |
|  | LD | 1 |  |
|  | LIMIT | N, 128 |  |
|  | ST | NI |  |
|  | JMP | ZRO_OUT |  |
| POP_STK: | LD | 0 |  |
|  | ST | OFLO | (* Popped stack is not overflowing *) |
|  | LD | PTR |  |
|  | SUB | 1 |  |
|  | ST | PTR |  |
|  | LT | 0 | (* Empty when PTR < 0 *) |
|  | ST | EMPTY |  |
|  | JMPC | ZRO_OUT |  |
|  | LD | STK[PTR] |  |
|  | JMP | SET_OUT |  |
| PUSH_STK: | LD | 0 |  |
|  | ST | EMPTY | (* Pushed stack is not empty *) |
|  | LD | PTR |  |
|  | ADD | 1 |  |
|  | ST | PTR |  |
|  | EQ | NI | (* Overflow when PTR = NI *) |
|  | ST | OFLO |  |
|  | JMPC | ZRO_OUT |  |
|  | LD | IN |  |
|  | ST | STK[PTR] | (* Push IN onto STK *) |
|  | JMP | SET_OUT |  |
| ZRO_OUT: | LD | 0 | (* OUT=0 for EMPTY or OFLO *) |
| SET_OUT: | ST | OUT |  |

The body of function block STACK_INT in the FBD language is:

R1--+-->>RESET ¦ +-+ +-----------------------------O¦&¦--<RETURN> ¦ +-+ +--------------------O¦ ¦ +--O¦&¦ ¦ +--O¦ ¦ POP-----¦ ¦--+-->>POP_STK ¦ +-+ EMPTY--O¦ ¦ ¦ +-+ ¦ +-+ +-----------O¦&¦--+-->>PUSH_STK R1-----------------------O¦ ¦ PUSH----------------------¦ ¦ OFLO---------------------O¦ ¦ +-+

RESET: +------+ +------+ +------+ ¦ := ¦ ¦ := ¦ ¦ := ¦ 1 --¦EN ENO¦-------------¦EN ENO¦-----------¦EN ENO¦--+ 0 --¦ ¦---OUT -1 --¦ ¦---PTR 0--¦ ¦--¦--OFLO +------+ +------+ +------+ ¦ +---------------------------------------------+ ¦ +------+ +-------+ ¦ ¦ := ¦ ¦ LIMIT ¦ +--¦EN ENO¦---------------¦EN ENO¦--<RETURN> 1--¦ ¦---EMPTY 128--¦MX ¦ +------+ N--¦IN ¦--NI 1--¦MN ¦ +-------+

POP_STK: +------+ +------+ +------+ +------+ ¦ - ¦ ¦ < ¦ ¦ SEL ¦ ¦ := ¦ 1----¦EN ENO¦-------¦EN ENO¦---------¦EN ENO¦------¦EN ENO¦--<RETURN> --PTR¦ ¦--PTR--¦ ¦--EMPTY--¦G ¦-+ ¦ ¦ ----1¦ ¦ 0 --¦ ¦ +------¦IN0 ¦ ¦ 0--¦ ¦--OFLO +------+ +------+ ¦ +----¦IN1 ¦ ¦ +------+ STK[PTR]—---------------------+ ¦ +------+ ¦ 0-------------------------------+ +---------------OUT

PUSH_STK: +------+ +------+ +------+ ¦ := ¦ ¦ + ¦ ¦ = ¦ 1--¦EN ENO¦-----------¦EN ENO¦-------¦EN ENO¦-- 0--¦ ¦--EMPTY 1--¦ ¦--PTR--¦G ¦--+--OFLO +------+ +--¦ ¦ NI---¦ ¦ ¦ PTR—---------------+ +------+ +------+ ¦ +----------------------------+------------+ ¦ +------+ ¦ +-----+ ¦ ¦ := ¦ ¦ ¦ SEL ¦ +----¦EN ENO¦ +---¦G ¦-----OUT IN--+-----¦ ¦--STK[PTR] +-------¦IN0 ¦ ¦ +------+ ¦ 0---¦IN1 ¦ +-------------------------+ +-----+

## F.5  Function block MIX_2_BRIX

Function block MIX_2_BRIX is to control the mixing of two bricks of solid material, brought one at a time on a belt, with weighed quantities of two liquid components, A and B, as shown in figure F.1.  A "Start" (ST) command, which may be manual or automatic, initiates a measurement and mixing cycle beginning with simultaneous weighing and brick transport as follows:

- Liquid A is weighed up to mark "a" of the weighing unit, then liquid B is weighed up to mark "b", followed by filling of the mixer from weighing unit C;

- Two bricks are transported by belt into the mixer.

The cycle ends with the mixer rotating and finally tipping after a predetermined time "t1".  Rotation of the mixer continues while it is emptying.

The scale reading "WC" is given as four BCD digits, and will be converted to type INT for internal operations.  It is assumed that the tare (empty weight) "z" has been previously determined.

Figure F.1 - Function block MIX_2_BRIX - Physical model

The textual form of the declaration of this function block is:

FUNCTION_BLOCK MIX_2_BRIX VAR_INPUT ST : BOOL ; (* "Start" command *) d : BOOL ; (* Transit detector *) S0 : BOOL ; (* "Mixer up" limit switch *) S1 : BOOL ; (* "Mixer down" limit switch *) WC : WORD; (* Current scale reading in BCD *) z : INT ; (* Tare (empty) weight *) WA : INT ; (* Desired weight of A *) WB : INT ; (* Desired weight of B *) t1 : TIME ; (* Mixing time *) END_VAR VAR_OUTPUT DONE , VA , (* Valve "A" : 0 - close, 1 - open *) VB , (* Valve "B" : 0 - close, 1 - open *) VC , (* Valve "C" : 0 - close, 1 - open *) MT , (* Feed belt motor *) MR , (* Mixer rotation motor *) MP0 , (* Tipping motor "up" command *) MP1 : BOOL; (* Tipping motor "down" command *) END_VAR (* Function block body *) END_FUNCTION_BLOCK

A graphical declaration is:

+------------+ ¦ MIX_2_BRIX ¦ BOOL---¦ST DONE¦---BOOL BOOL---¦d VA¦---BOOL BOOL---¦S0 VB¦---BOOL BOOL---¦S1 VC¦---BOOL WORD---¦WC MT¦---BOOL INT---¦z MR¦---BOOL INT---¦WA MP0¦---BOOL INT---¦WB MP1¦---BOOL TIME---¦t1 ¦ +------------+

The body of function block MIX_2_BRIX using graphical SFC elements with transition conditions in the ST language is shown below.

+----------->------------+ ¦ ¦ ¦ +====+====+ +---+------+ ¦ ¦¦ START ¦¦---¦ N ¦ DONE ¦ ¦ +====+====+ +---+------+ ¦ ¦ ¦ + ST & S0 & BCD_TO_INT(WC) <= z ¦ ¦ ¦ ===+===============+================+======= ¦ ¦ ¦ ¦ +----+----+ +---+----+ +----+---+ +---+----+ ¦ ¦ WEIGH_A ¦---¦ N ¦ VA ¦ ¦ BRICK1 ¦---¦ S ¦ MT ¦ ¦ +----+----+ +---+----+ +----+---+ +---+----+ ¦ ¦ ¦ ¦ + BCD_TO_INT(WC) >= WA+z + d ¦ ¦ ¦ ¦ +----+----+ +---+----+ +----+---+ ¦ ¦ WEIGH_B ¦---¦ N ¦ VB ¦ ¦ DROP_1 ¦ ¦ +----+----+ +---+----+ +----+---+ ¦ ¦ ¦ ¦ + BCD_TO_INT(WC) >= WA+WB+z + NOT d ¦ ¦ ¦ ¦ +----+----+ +---+----+ +----+---+ ¦ ¦ FILL ¦---¦ N ¦ VC ¦ ¦ BRICK2 ¦ ¦ +----+----+ +---+----+ +----+---+ ¦ ¦ + d ¦ ¦ +----+---+ +---+----+ ¦ ¦ ¦ DROP_2 ¦---¦ R ¦ MT ¦ ¦ ¦ +----+---+ +---+----+ ¦ ¦ ¦ ¦ ====+===============+================+===== ¦ ¦ ¦ + BCD_TO_INT(WC) <= z & NOT d ¦ ¦ ¦ +--+--+ +---+----+ ¦ ¦ MIX ¦---¦ S ¦ MR ¦ ¦ +--+--+ +---+----+ ¦ ¦ ¦ + MIX.T >= t1 ¦ ¦ ¦ +--+--+ +---+-----+----+ ¦ ¦ TIP ¦---¦ N ¦ MP1 ¦ S1 ¦ ¦ +--+--+ +---+-----+----+ ¦ ¦ ¦ + S1 ¦ ¦ ¦ +---+---+ +---+-----+----+ ¦ ¦ RAISE ¦---¦ R ¦ MR ¦ ¦ ¦ +---+---+ +---+-----+----+ ¦ +S0 ¦ N ¦ MP0 ¦ S0 ¦ ¦ ¦ +---+-----+----+ +-----------<------------+

The body of function block MIX_2_BRIX in a textual SFC representation using ST language elements is:

INITIAL_STEP START: DONE(N); END_STEP TRANSITION FROM START TO (WEIGH_A, BRICK1) := ST & S0 & BCD_TO_INT(WC) <= z; END_TRANSITION STEP WEIGH_A: VA(N); END_STEP TRANSITION FROM WEIGH_A TO WEIGH_B := BCD_TO_INT(WC) >= WA+z ; END_TRANSITION STEP WEIGH_B: VB(N); END_STEP TRANSITION FROM WEIGH_B TO FILL := BCD_TO_INT(WC) >= WA+WB+z ; END_TRANSITION STEP FILL: VC(N); END_STEP STEP BRICK1: MT(S); END_STEP TRANSITION FROM BRICK1 TO DROP_1 := d ; END_TRANSITION STEP DROP_1: END_STEP TRANSITION FROM DROP_1 TO BRICK2 := NOT d ; END_TRANSITION STEP BRICK2: END_STEP TRANSITION FROM BRICK2 TO DROP_2 := d ; END_TRANSITION STEP DROP_1: MT(R); END_STEP TRANSITION FROM (FILL,DROP_2) TO MIX := BCD_TO_INT(WC) <= z & NOT d ; END_TRANSITION STEP MIX: MR(S); END_STEP TRANSITION FROM MIX TO TIP := MIX.T >= t1 ; END_TRANSITION STEP TIP: MP1(N); END_STEP TRANSITION FROM TIP TO RAISE := S1 ; END_TRANSITION STEP RAISE: MR(R); MP0(N); END_STEP TRANSITION FROM RAISE TO START := S0 ; END_TRANSITION

## F.6  Analog signal processing

The purpose of this portion of of this annex is to illustrate the application of the programming languages defined in this standard to accomplish the basic measurement and control functions of process-computer aided automation.  The blocks shown below are not restricted to analog signals; they may be used to process any variables of the appropriate types.  Similarly, other functions and function blocks defined in this standard (e.g., mathematical functions) can be used for the processing of variables which may appear as analog signals at the programmable controller's I/O terminals.

These function blocks can be typed with respect to the input and output variables shown below as REAL (e.g., XIN, XOUT) by appending the appropriate data type name, e.g., LAG1_LREAL.  The default data type for these variables is REAL.

These examples are given for illustrative purposes only.  Manufacturers may have varying implementations of analog signal processing elements.  The inclusion of these examples is not intended to preclude the standardization of such elements by the appropriate standards bodies.

### F.6.1  Function block LAG1

This function block implements a first-order lag filter.

+-----------+ ¦ LAG1 ¦ BOOL---¦RUN ¦ REAL---¦XIN XOUT¦---REAL TIME---¦TAU ¦ TIME---¦CYCLE ¦ +-----------+

FUNCTION_BLOCK LAG1 VAR_INPUT RUN : BOOL ; (* 1 = run, 0 = reset *) XIN : REAL ; (* Input variable *) TAU : TIME ; (* Filter time constant *) CYCLE : TIME ; (* Sampling time interval *) END_VAR VAR_OUTPUT XOUT : REAL ; END_VAR (* Filtered output *) VAR K : REAL ; (* Smoothing constant, 0.0<=K<1.0 *) END_VAR IF RUN THEN XOUT := XOUT + K * (XIN - XOUT) ; ELSE XOUT := XIN ; K := TIME_TO_REAL(CYCLE) / TIME_TO_REAL(CYCLE + TAU) ; END_IF ; END_FUNCTION_BLOCK

### F.6.2  Function block DELAY

This function block implements an N-sample delay.

+-----------+ ¦ DELAY ¦ BOOL---¦RUN ¦ REAL---¦XIN XOUT¦---REAL INT----¦N ¦ +-----------+

FUNCTION_BLOCK DELAY (* N-sample delay *) VAR_INPUT RUN : BOOL ; (* 1 = run, 0 = reset *) XIN : REAL ; N : INT ; (* 0 <= N < 128 or manufacturer- *) END_VAR (* specified maximum value *) VAR_OUTPUT XOUT : REAL; END_VAR (* Delayed output *) VAR X : ARRAY [0..127] (* N-Element queue *) OF REAL; (* with FIFO discipline *) I, IXIN, IXOUT : INT := 0; END_VAR IF RUN THEN IXIN := MOD(IXIN + 1, 128) ; X[IXIN] := XIN ; IXOUT := MOD(IXOUT + 1, 128) ; XOUT := X[IXOUT]; ELSE XOUT := XIN ; IXIN := N ; IXOUT := 0; FOR I := 0 TO N DO X[I] := XIN; END_FOR; END_IF ; END_FUNCTION_BLOCK

### F.6.3  Function block AVERAGE

This function block implements a running average over N samples.

+-----------+ ¦ AVERAGE ¦ BOOL---¦RUN ¦ REAL---¦XIN XOUT¦---REAL INT----¦N ¦ +-----------+

FUNCTION_BLOCK AVERAGE VAR_INPUT RUN : BOOL ; (* 1 = run, 0 = reset *) XIN : REAL ; (* Input variable *) N : INT ; (* 0 <= N < 128 or manufacturer- *) END_VAR (* specified maximum value *) VAR_OUTPUT XOUT : REAL ; END_VAR (* Averaged output *) VAR SUM : REAL := 0.0; (* Running sum *) FIFO : DELAY ; (* N-Element FIFO *) END_VAR SUM := SUM - FIFO.XOUT ; FIFO (RUN := RUN , XIN := XIN, N := N) ; SUM := SUM + FIFO.XOUT ; IF RUN THEN XOUT := SUM/N ; ELSE SUM := N*XIN ; XOUT := XIN ; END_IF ; END_FUNCTION_BLOCK

### F.6.4  Function block INTEGRAL

This function block implements integration over time.

+-----------+ ¦ INTEGRAL ¦ BOOL---¦RUN Q¦---BOOL BOOL---¦R1 ¦ REAL---¦XIN XOUT¦---REAL REAL---¦X0 ¦ TIME---¦CYCLE ¦ +-----------+

FUNCTION_BLOCK INTEGRAL VAR_INPUT RUN : BOOL ; (* 1 = integrate, 0 = hold *) R1 : BOOL ; (* Overriding reset *) XIN : REAL ; (* Input variable *) X0 : REAL ; (* Initial value *) CYCLE : TIME ; (* Sampling period *) END_VAR VAR_OUTPUT Q : BOOL ; (* NOT R1 *) XOUT : REAL ; (* Integrated output *) END_VAR Q := NOT R1 ; IF R1 THEN XOUT := X0 ; ELSIF RUN THEN XOUT := XOUT + XIN * TIME_TO_REAL(CYCLE); END_IF ; END_FUNCTION_BLOCK

### F.6.5  Function block DERIVATIVE

This function block implements differentiation with respect to time.

+------------+ ¦ DERIVATIVE ¦ BOOL---¦RUN ¦ REAL---¦XIN XOUT¦---REAL TIME---¦CYCLE ¦ +------------+

FUNCTION_BLOCK DERIVATIVE VAR_INPUT RUN : BOOL ; (* 0 = reset *) XIN : REAL ; (* Input to be differentiated *) CYCLE : TIME ; (* Sampling period *) END_VAR VAR_OUTPUT XOUT : REAL ; (* Differentiated output *) END_VAR VAR X1, X2, X3 : REAL ; END_VAR IF RUN THEN XOUT := (3.0 * (XIN - X3) + X1 - X2) / (10.0 * TIME_TO_REAL(CYCLE)) ; X3 := X2 ; X2 := X1 ; X1 := XIN ; ELSE XOUT := 0.0; X1 := XIN ; X2 := XIN ; X3 := XIN ; END_IF ; END_FUNCTION_BLOCK

### F.6.6  Function block HYSTERESIS

This function block implements Boolean hysteresis on the difference of REAL inputs.

+------------+ ¦ HYSTERESIS ¦ REAL---¦XIN1 Q¦---BOOL REAL---¦XIN2 ¦ REAL---¦EPS ¦ +------------+

FUNCTION_BLOCK HYSTERESIS (* Boolean hysteresis on difference *) (* of REAL inputs, XIN1 - XIN2 *) VAR_INPUT XIN1, XIN2, EPS : REAL; END_VAR VAR_OUTPUT Q : BOOL := 0; END_VAR IF Q THEN IF XIN1 < (XIN2 - EPS) THEN Q := 0; END_IF ; ELSIF XIN1 > (XIN2 + EPS) THEN Q := 1 ; END_IF ; END_FUNCTION_BLOCK

### F.6.7  Function block LIMITS_ALARM

This function block implements a high/low limit alarm with hysteresis on both outputs.

+---------+ ¦ LIMITS_ ¦ ¦ ALARM ¦ (* High limit *) REAL--¦H QH¦--BOOL (* High flag *) (* Variable value *) REAL--¦X Q¦--BOOL (* Alarm output *) (* Lower limit *) REAL--¦L QL¦--BOOL (* Low flag *) (* Hysteresis *) REAL--¦EPS ¦ +---------+

(* Function block body in FBD language *) HIGH_ALARM +------------+ ¦ HYSTERESIS ¦ X------------------------+--¦XIN1 Q¦--+------------QH +---+ ¦ ¦ ¦ ¦ H----------------¦ - ¦------¦XIN2 ¦ ¦ +---¦ ¦ ¦ ¦ ¦ ¦ ¦ +---+ ¦ ¦ ¦ ¦ +--------------¦EPS ¦ ¦ +-----+ +---+ ¦ ¦ +------------+ +--¦ >=1 ¦ EPS---¦ / ¦--+ ¦ ¦ ¦---Q 2.0---¦ ¦ ¦ ¦ LOW_ALARM +--¦ ¦ +---+ ¦ ¦ +------------+ ¦ +-----+ ¦ +---+ ¦ ¦ HYSTERESIS ¦ ¦ L----------------¦ + ¦------¦XIN1 Q¦--+------------QL ¦ ¦ ¦ ¦ ¦ ¦ +---¦ ¦ +--¦XIN2 ¦ ¦ +---+ ¦ ¦ +--------------¦EPS ¦ +------------+

### F.6.8  Structure ANALOG_LIMITS

This data type implements the declarations of parameters for analog signal monitoring.

TYPE ANALOG_LIMITS : STRUCT HS : REAL ; (* High end of signal range *) HM : REAL ; (* High end of measurement range *) HA : REAL ; (* High alarm threshold *) HW : REAL ; (* High warning threshold *) NV : REAL ; (* Nominal value *) EPS : REAL ; (* Hysteresis *) LW : REAL ; (* Low warning threshold *) LA : REAL ; (* Low alarm threshold *) LM : REAL ; (* Low end of measurement range *) LS : REAL ; (* Low end of signal range *) END_STRUCT ; END_TYPE

### F.6.9  Function block ANALOG_MONITOR

This function block implements analog signal monitoring.

+---------+ ¦ ANALOG_ ¦ ¦ MONITOR ¦ REAL--¦X SE¦--BOOL (* Signal error *) ANALOG_LIMITS--¦L ME¦--BOOL (* Measurement error *) ¦ ALRM¦--BOOL (* Alarm *) ¦ WARN¦--BOOL (* Warning *) ¦ QH¦--BOOL (* 1 = Signal high *) +---------+

(* Function block body in FBD language *) SIGNAL_ALARM MEAS_ALARM +--------------+ +--------------+ ¦ LIMITS_ALARM ¦ ¦ LIMITS_ALARM ¦ L.HS---¦H Q¦---SE L.HM---¦H Q¦---ME X------¦X ¦ X------¦X ¦ L.LS---¦L ¦ L.LM---¦L ¦ EPS----¦EPS ¦ EPS----¦EPS ¦ +--------------+ +--------------+ ALARM WARNING +--------------+ +--------------+ ¦ LIMITS_ALARM ¦ ¦ LIMITS_ALARM ¦ L.HA---¦H Q¦---ALRM L.HW---¦H Q¦---WARN X------¦X ¦ X---¦X ¦ L.LA---¦L ¦ L.LW---¦L ¦ EPS----¦EPS ¦ EPS---¦EPS ¦ +--------------+ +--------------+ +------+ SIGNAL_ALARM.QH---¦ >= 1 ¦---QH MEAS_ALARM.QH-----¦ ¦ ALARM.QH----------¦ ¦ WARNING.QH--------¦ ¦ +------+

### F.6.10 Function block PID

This function block implements Proportional + Integral + Derivative control action.  The functionality is derived by functional composition of previously declared function blocks.

+-----------+ ¦ PID ¦ BOOL---¦AUTO ¦ REAL---¦PV XOUT¦---REAL REAL---¦SP ¦ REAL---¦X0 ¦ REAL---¦KP ¦ REAL---¦TR ¦ REAL---¦TD ¦ TIME---¦CYCLE ¦ +-----------+

FUNCTION_BLOCK PID VAR_INPUT AUTO : BOOL ; (* 0 - manual , 1 - automatic *) PV : REAL ; (* Process variable *) SP : REAL ; (* Set point *) X0 : REAL ; (* Manual output adjustment - *) (* Typically from transfer station *) KP : REAL ; (* Proportionality constant *) TR : REAL ; (* Reset time *) TD : REAL ; (* Derivative time constant *) CYCLE : TIME ; (* Sampling period *) END_VAR VAR_OUTPUT XOUT : REAL; END_VAR VAR ERROR : REAL ; (* PV - SP *) ITERM : INTEGRAL ; (* FB for integral term *) DTERM : DERIVATIVE ; (* FB for derivative term *) END_VAR ERROR := PV - SP ; (*** Adjust ITERM so that XOUT := X0 when AUTO = 0 ***) ITERM (RUN := AUTO, R1 := NOT AUTO, XIN := ERROR, X0 := TR * (X0 - ERROR), CYCLE := CYCLE) ; DTERM (RUN := AUTO, XIN := ERROR, CYCLE := CYCLE) ; XOUT := KP * (ERROR + ITERM.XOUT/TR + DTERM.XOUT*TD) ; END_FUNCTION_BLOCK

### F.6.11 Function block DIFFEQ

This function block implements a general difference equation.

+-----------+ ¦ DIFFEQ ¦ BOOL---¦RUN ¦ REAL---¦XIN XOUT¦---REAL ARRAY[1..127] OF REAL---¦A ¦ INT----¦M ¦ ARRAY[0..127] OF REAL---¦B ¦ INT----¦N ¦ +-----------+

FUNCTION_BLOCK DIFFEQ VAR_INPUT RUN : BOOL ; (* 1 = run, 0 = reset *) XIN : REAL ; A : ARRAY[1..127] OF REAL ; (* Input coefficients *) M : INT ; (* Length of input history *) B : ARRAY[0..127] OF REAL ; (* Output coefficients *) N : INT ; (* Length of output history *) END_VAR VAR_OUTPUT XOUT : REAL := 0.0 ; END_VAR VAR (* NOTE : Manufacturer may specify other array sizes *) XI : ARRAY [0..127] OF REAL ; (* Input history *) XO : ARRAY [0..127] OF REAL ; (* Output history *) I : INT ; END_VAR XO[0] := XOUT ; XI[0] := XIN ; XOUT := B[0] * XIN ; IF RUN THEN FOR I := M TO 1 BY -1 DO XOUT := XOUT + A[I] * XO[I] ; XO[I] := XO[I-1]; END_FOR; FOR I := N TO 1 BY -1 DO XOUT := XOUT + B[I] * XI[I] ; XI[I] := XI[I-1]; END_FOR; ELSE FOR I := 1 TO M DO XO[I] := 0.0; END_FOR; FOR I := 1 TO N DO XI[I] := 0.0; END_FOR; END_IF ; END_FUNCTION_BLOCK

### F.6.12  Function block RAMP

This function block implements a time-based ramp.

+-----------+ ¦ RAMP ¦ BOOL---¦RUN BUSY¦---BOOL REAL---¦X0 XOUT¦---REAL REAL---¦X1 ¦ TIME---¦TR ¦ TIME---¦CYCLE ¦ +-----------+

FUNCTION_BLOCK RAMP VAR_INPUT RUN : BOOL ; (* 0 - track X0, 1 - ramp to/track X1 *) X0,X1 : REAL ; TR : TIME ; (* Ramp duration *) CYCLE : TIME ; (* Sampling period *) END_VAR VAR_OUTPUT BUSY : BOOL ; (* BUSY = 1 during ramping period *) XOUT : REAL := 0.0 ; END_VAR VAR XI : REAL ; (* Initial value *) T : TIME := T#0s; (* Elapsed time of ramp *) END_VAR BUSY := RUN ; IF RUN THEN IF T >= TR THEN BUSY := 0 ; XOUT := X1 ; ELSE XOUT := XI + (X1-XI) * TIME_TO_REAL(T) / TIME_TO_REAL(TR) ; T := T + CYCLE ; END_IF ; ELSE XOUT := X0 ; XI := X0 ; T := t#0s ; END_IF ; END_FUNCTION_BLOCK

### F.6.13  Function block TRANSFER

This function block implements a manual transfer station with bumpless transfer.

+-----------+ ¦ TRANSFER ¦ BOOL---¦AUTO ¦ REAL---¦XIN XOUT¦---REAL REAL---¦FAST_RATE ¦ REAL---¦SLOW_RATE ¦ BOOL---¦FAST_UP ¦ BOOL---¦SLOW_UP ¦ BOOL---¦FAST_DOWN ¦ BOOL---¦SLOW_DOWN ¦ TIME---¦CYCLE ¦ +-----------+

FUNCTION_BLOCK TRANSFER VAR_INPUT AUTO : BOOL ; (* 1 - track X0, 0 - ramp or hold *) XIN : REAL ; (* Typically from PID Function Block *) FAST_RATE, SLOW_RATE : REAL ; (* Up/down ramp slopes *) FAST_UP, SLOW_UP, (* Typically pushbuttons *) FAST_DOWN, SLOW_DOWN : BOOL; CYCLE : TIME ; (* Sampling period *) END_VAR VAR_OUTPUT XOUT : REAL ; END_VAR VAR XFER_RAMP : INTEGRAL ; RAMP_RATE : REAL ; END_VAR RAMP_RATE := 0.0 ; IF NOT AUTO THEN IF FAST_UP THEN RAMP_RATE := FAST_RATE; END_IF; IF SLOW_UP THEN RAMP_RATE := RAMP_RATE + SLOW_RATE; END_IF; IF FAST_DOWN THEN RAMP_RATE := RAMP_RATE - FAST_RATE; END_IF; IF SLOW_DOWN THEN RAMP_RATE := RAMP_RATE - SLOW_RATE; END_IF; END_IF ; XFER_RAMP (RUN := 1, CYCLE := CYCLE, R1 := AUTO, XIN := RAMP_RATE, X0 := XIN) ; XOUT := XFER_RAMP.XOUT; END_FUNCTION_BLOCK

## F.7  Program GRAVEL

A control system is to be used to measure an operator-specified amount of gravel from a silo into an intermediate bin, and to convey the gravel after measurement from the bin into a truck.

The quantity of gravel to be transferred is specified via a thumbwheel with a range of 0 to 99 units.  The amount of gravel in the bin is indicated on a digital display.

For safety reasons, visual and audible alarms must be raised immediately when the silo is empty.  The signalling functions are to be implemented in the control program.

A graphic representation of the control problem is shown in figure F.2, while the variable declarations for the control program are given in figure F.3.

As shown in figure F.4, the operation of the system consists of a number of major states, beginning with filling of the bin upon command from the FILL push button.  After the bin is filled, the truck loading sequence begins upon command by the LOAD pushbutton when a truck is present on the ramp.  Loading consists of a "run-in" period for starting the conveyor, followed by dumping of the bin contents onto the conveyor.  After the bin has emptied, the conveyor "runs out" for a predetermined time to assure that all gravel has been loaded to the truck.  The loading sequence is stopped and re-initialized if the truck leaves the ramp or if the automatic control is stopped by the OFF push button.

Figure F.5 shows the OFF/ON sequence of automatic control states, as well as the generation of display blinking pulses and conveyor motor gating when the control is ON.

Bin level monitoring, operator interface and display functions are defined in figure F.6.

A textual version of the body of program GRAVEL is given in figure F.7, using the ST language with SFC elements.

An example configuration for program GRAVEL is given in figure F.8.

| +---------+ | CONTROL PANEL: |  |
| --- | --- | --- |
| ¦ SILO ¦ | INDICATORS | PUSH BUTTONS |
| ¦ ¦ |  | ON |
| ¦ ¦ | CONTROL SYSTEM ON | OFF |
| ¦ ¦ | TRUCK ON RAMP | ACKNOWLEDGE |
| ¦ ¦ | SILO EMPTY | FILL |
| \ / | CONVEYOR RUNNING | LOAD |
| \ / |  | LAMP TEST |
| ¦ ¦ "Silo empty" ¦ o ¦ limit switch | 2-DIGIT BCD: |  |
| ¦ / ¦ Silo valve | DISPLAY | THUMBWHEEL |
| +---+ | BIN LEVEL | SET POINT |
| ¦ BIN ¦ ¦ ¦ | SIREN : SILO EMPTY |  |
| \ / "Bin empty" |  |  |
| ¦ o ¦ limit switch |  |  |
| ¦ / ¦ Bin valve |  |  |
| +---+ |  |  |

Figure F.2 - Gravel measurement and loading system

PROGRAM GRAVEL (* Gravel measurement and loading system *)

VAR_INPUT OFF_PB : BOOL ; ON_PB : BOOL ; FILL_PB : BOOL ; SIREN_ACK : BOOL ; LOAD_PB : BOOL ; (* Load truck from bin *) JOG_PB : BOOL ; LAMP_TEST : BOOL ; TRUCK_ON_RAMP : BOOL ; (* Optical sensor *) SILO_EMPTY_LS : BOOL ; BIN_EMPTY_LS : BOOL ; SETPOINT : BYTE ; (* 2-digit BCD *) END_VAR

VAR_OUTPUT CONTROL_LAMP : BOOL ; TRUCK_LAMP : BOOL ; SILO_EMPTY_LAMP : BOOL ; CONVEYOR_LAMP : BOOL ; CONVEYOR_MOTOR : BOOL ; SILO_VALVE : BOOL ; BIN_VALVE : BOOL ; SIREN : BOOL ; BIN_LEVEL : BYTE ; END_VAR

VAR BLINK_TIME : TIME; (* BLINK ON/OFF time *) PULSE_TIME : TIME; (* LEVEL_CTR increment interval *) RUNOUT_TIME: TIME; (* Conveyor running time after loading *) RUN_IN_TIME: TIME; (* Conveyor running time before loading *) SILENT_TIME: TIME; (* Siren silent time after SIREN_ACK *) OK_TO_RUN : BOOL; (* 1 = Conveyor is allowed to run *)

(* Function Blocks *) BLINK: TON; (* Blinker OFF period timer / ON output *) BLANK: TON; (* Blinker ON period timer / blanking pulse *) PULSE: TON; (* LEVEL_CTR pulse interval timer *) SIREN_FF: RS; SILENCE_TMR: TP; (* Siren silent period timer *) END_VAR

VAR RETAIN LEVEL_CTR : CTU ; END_VAR

(* Program body *)

END_PROGRAM

Figure F.3 - Declarations for Program GRAVEL

+--------------------->-----------------+ ¦ ¦ ¦ +====+====+ ¦ ¦¦ START ¦¦ ¦ +====+====+ ¦ ¦ ¦ + FILL_PB & CONTROL.X ¦ ¦ ¦ +-----+----+ +---+------------+ ¦ ¦ FILL_BIN ¦---¦ N ¦ SILO_VALVE ¦ ¦ +-----+----+ +---+------------+ ¦ ¦ ¦ +-----------------------------------* ¦ ¦ ¦ ¦ + NOT FILL_PB OR NOT CONTROL.X + LEVEL_CTR.Q +---+ ¦ ¦ +------------------>-------------+ ¦ ¦ +-----+-----+ ¦ ¦ ¦ LOAD_WAIT ¦ ¦ ¦ +-----+-----+ ¦ ¦ ¦ ¦ ¦ + LOAD_PB & OK_TO_RUN ¦ ¦ ¦ ¦ ¦ +----+---+ ¦ ¦ ¦ RUN_IN ¦ ¦ ¦ +----+---+ ¦ ¦ ¦ ¦ ¦ +----------------------------* ¦ ¦ ¦ ¦ ¦ ¦ + NOT OK_TO_RUN + RUN_IN.T >= RUN_IN_TIME ¦ ¦ ¦ ¦ ¦ +---+ +-----+----+ +---+-----------+ ¦ ¦ ¦ DUMP_BIN ¦---¦ N ¦ BIN_VALVE ¦ ¦ ¦ +-----+----+ +---+-----------+ ¦ ¦ ¦ ¦ ¦ +----------------------------* ¦ ¦ ¦ ¦ ¦ ¦ + NOT OK_TO_RUN + BIN_EMPTY_LS ¦ ¦ ¦ ¦ ¦ +---+ +----+---+ ¦ ¦ ¦ RUNOUT ¦ ¦ ¦ +----+---+ ¦ ¦ ¦ ¦ ¦ +----------------------------* ¦ ¦ ¦ ¦ ¦ ¦ + NOT OK_TO_RUN + RUNOUT.T >= RUNOUT_TIME ¦ +---+ ¦ +---------------------------------------+

Figure F.4 - SFC of program GRAVEL body

+---------+ ¦ ¦ ¦ + OFF_PB ¦ ¦ ¦ +======+======+ +===========+ +---+----------------+ ¦ ¦¦CONTROL_OFF¦¦ ¦¦ MONITOR ¦¦---¦ N ¦ MONITOR_ACTION ¦ ¦ +======+======+ +===========+ +---+----------------+ ¦ ¦ ¦ + ON_PB & NOT OFF_PB ¦ ¦ ¦ +---+---+ +---+----------------------------------+-----+ ¦ ¦CONTROL¦--¦ N ¦ CONTROL_ACTION ¦ ¦ ¦ +---+---+ +---+----------------------------------+-----+ ¦ ¦ ¦ +---------------------------------+ ¦ +---------+ ¦ ¦ BLINK BLANK ¦ ¦ ¦ ¦ +-+ +-----+ +-----+ ¦ ¦ ¦ +---O¦&¦ ¦ TON ¦ ¦ TON ¦ ¦ ¦ ¦CONTROL.X--¦ ¦-----¦IN Q¦-----¦IN Q¦--+ ¦ ¦ +-+ +--¦PT ¦ +--¦PT ¦ ¦ ¦ ¦ +-----+ ¦ +-----+ ¦ ¦ BLINK_TIME--+-----------+ ¦ ¦ +-+ ¦ ¦CONTROL.X------¦&¦ ¦ ¦TRUCK_ON_RAMP--¦ ¦---+---------OK_TO_RUN ¦ ¦ +-+ ¦ ¦ ¦ ¦ +-+ ¦ ¦ +-----+ +--¦&¦--CONVEYOR_MOTOR ¦ ¦JOG_PB------¦ >=1 ¦-----¦ ¦ ¦ ¦RUN_IN.X----¦ ¦ +-+ ¦ ¦DUMP_BIN.X--¦ ¦ ¦ ¦RUNOUT.X----¦ ¦ ¦ ¦ +-----+ ¦ +--------------------------------------------+

Figure F.5 - Body of program GRAVEL (continued)
Control state sequencing and monitoring

MONITOR_ACTION

+---+ CONVEYOR_MOTOR------------------------¦ & ¦------CONVEYOR_LAMP BLINK.Q-------------------------------¦ ¦ +---+ +-----+ CONTROL.X----------------------------¦ >=1 ¦---CONTROL_LAMP LAMP_TEST------------+---------------¦ ¦ ¦ +-----+ ¦ +-----+ +---¦ >=1 ¦---------------TRUCK_LAMP TRUCK_ON_RAMP--------¦---¦ ¦ ¦ +-----+ ¦ +-----+ +---------------¦ >=1 ¦---SILO_EMPTY_LAMP +---+ ¦ ¦ BLINK.Q------------¦ & ¦-------------¦ ¦ SILO_EMPTY_LS--+---¦ ¦ +-----+ ¦ +---+ SIREN_FF ¦ +------+ ¦ ¦ RS ¦ +-----------¦S Q1¦------------SIREN SILENCE_TMR ¦ ¦ +------+ ¦ ¦ ¦ TP ¦ ¦ ¦ SIREN_ACK-----¦IN Q¦-----¦R1 ¦ SILENT_TIME---¦PT ¦ +------+ +------+ LEVEL_CTR +-----+ ¦ CTU ¦ BIN_EMPTY_LS-----------------¦R Q¦ +------------------+ ¦ ¦ ¦ PULSE ¦ ¦ ¦ ¦ +-+ +-----+ ¦ ¦ ¦ +---O¦&¦ ¦ TON ¦ ¦ ¦ ¦ FILL_BIN.X--¦ ¦--¦IN Q¦--+-->CU ¦ +-+ ¦ ¦ ¦ ¦ PULSE_TIME-------¦PT ¦ ¦ ¦ +-----+ ¦ ¦ +------------+ ¦ ¦ +------------+ SETPOINT----¦ BCD_TO_INT ¦---¦PV CV¦--¦ INT_TO_BCD ¦--BIN_LEVEL +------------+ +-----+ +------------+

Figure F.6 - Body of action MONITOR_ACTION in FBD language

Figure F.7 - Body of program GRAVEL in textual SFC representation using ST language elements

(* Major operating states *) INITIAL_STEP START : END_STEP TRANSITION FROM START TO FILL_BIN := FILL_PB & CONTROL.X ; END_TRANSITION

STEP FILL_BIN: SILO_VALVE(N); END_STEP TRANSITION FROM FILL_BIN TO START := NOT FILL_PB OR NOT CONTROL.X ; END_TRANSITION TRANSITION FROM FILL_BIN TO LOAD_WAIT := LEVEL_CTR.Q ; END_TRANSITION

STEP LOAD_WAIT : END_STEP TRANSITION FROM LOAD_WAIT TO RUN_IN := LOAD_PB & OK_TO_RUN ; END_TRANSITION

STEP RUN_IN : END_STEP TRANSITION FROM RUN_IN TO LOAD_WAIT := NOT OK_TO_RUN ; END_TRANSITION TRANSITION FROM RUN_IN TO DUMP_BIN := RUN_IN.T > RUN_IN_TIME; END_TRANSITION

STEP DUMP_BIN: BIN_VALVE(N); END_STEP TRANSITION FROM DUMP_BIN TO LOAD_WAIT := NOT OK_TO_RUN ; END_TRANSITION TRANSITION FROM DUMP_BIN TO RUNOUT := BIN_EMPTY_LS ; END_TRANSITION

STEP RUNOUT : END_STEP TRANSITION FROM RUNOUT TO LOAD_WAIT := NOT OK_TO_RUN ; END_TRANSITION TRANSITION FROM RUNOUT TO START := RUNOUT.T >= RUNOUT_TIME ; END_TRANSITION

(* Control state sequencing *) INITIAL_STEP CONTROL_OFF: END_STEP TRANSITION FROM CONTROL_OFF TO CONTROL := ON_PB & NOT OFF_PB ; END_TRANSITION

STEP CONTROL: CONTROL_ACTION(N); END_STEP ACTION CONTROL_ACTION: BLINK(EN:=CONTROL.X & NOT BLANK.Q, PT := BLINK_TIME) ; BLANK(EN:=BLINK.Q, PT := BLINK_TIME) ; OK_TO_RUN := CONTROL.X & TRUCK_ON_RAMP ; CONVEYOR_MOTOR := OK_TO_RUN & OR(JOG_PB, RUN_IN.X, DUMP_BIN.X, RUNOUT.X); END_ACTION TRANSITION FROM CONTROL TO CONTROL_OFF := OFF_PB ; END_TRANSITION

(* Monitor Logic *) INITIAL_STEP MONITOR: MONITOR_ACTION(N); END_STEP ACTION MONITOR_ACTION: CONVEYOR_LAMP := CONVEYOR_MOTOR & BLINK.Q ; CONTROL_LAMP := CONTROL.X OR LAMP_TEST ; TRUCK_LAMP := TRUCK_ON_RAMP OR LAMP_TEST ; SILO_EMPTY_LAMP := BLINK.Q & SILO_EMPTY_LS OR LAMP_TEST ; SILENCE_TMR(IN:=SIREN_ACK, PT:=SILENT_TIME) ; SIREN_FF(S:=SILO_EMPTY_LS, R1:=SILENCE_TMR.Q) ; SIREN := SIREN_FF.Q1 ; PULSE(IN:=FILL_BIN.X & NOT PULSE.Q, PT:=PULSE_TIME) ; LEVEL_CTR(R := BIN_EMPTY_LS, CU := PULSE.Q, PV := BCD_TO_INT(SETPOINT)) ; BIN_LEVEL := INT_TO_BCD(LEVEL_CTR.CV) ; END_ACTION

CONFIGURATION GRAVEL_CONTROL RESOURCE PROC1 ON PROC_TYPE_Y PROGRAM G : GRAVEL (* Inputs *) (OFF_PB := %I0.0 , ON_PB := %I0.1 , FILL_PB := %I0.2 , SIREN_ACK := %I0.3 , LOAD_PB := %I0.4 , JOG_PB := %I0.5 , LAMP_TEST := %I0.7 , TRUCK_ON_RAMP := %I1.4 , SILO_EMPTY_LS := %I1.5 , BIN_EMPTY_LS := %I1.6 , SETPOINT := %IB2 , (* Outputs *) CONTROL_LAMP => %Q4.0, TRUCK_LAMP => %Q4.2, SILO_EMPTY_LAMP => %Q4.3, CONVEYOR_LAMP => %Q5.3, CONVEYOR_MOTOR => %Q5.4, SILO_VALVE => %Q5.5, BIN_VALVE => %Q5.6, SIREN => %Q5.7, BIN_LEVEL => %B6) ; END_RESOURCE END_CONFIGURATION

Figure F.8 - Example configuration for program GRAVEL

## F.8  Program AGV

As illustrated in figure F.9, a program is to be devised to control an automatic guided vehicle (AGV).  The AGV is to travel between two extreme positions, left (indicated by limit switch S3) and right (indicated by limit switch S4).  The normal position of the AGV is on the left.

The AGV is to execute one cycle of left-to-right and return motion when the operator actuates pushbutton S1, and two cycles when the operator actuates pushbutton S2.  It is also possible to pass from a single to a double cycle by actuating pushbutton S2 during a single cycle.  Finally, non-repeat locking is to be provided if either S1 or S2 remains actuated.

Figure F.10 illustrates the graphical declaration of program AGV, while figure F.11  shows a typical configuration for this program.  Figure F.12 shows the AGV program body, consisting of a main control sequence and a single-cycle control sequence.

Figure F.9 - Physical model for program AGV

+----------------------+ ¦ AGV ¦ BOOL---¦SINGLE_PB FWD_MOTOR¦---BOOL BOOL---¦DOUBLE_PB REV_MOTOR¦---BOOL BOOL---¦LEFT_LS ¦ BOOL---¦RIGHT_LS ¦ +----------------------+

Figure F.10 - Graphical declaration of program AGV

| CONFIGURATION AGV_CONTROL |  |  |
| --- | --- | --- |
|  | RESOURCE AGV_PROC: SMALL_PC |  |
|  | AGV_1 +--------------------+ ¦ AGV ¦ %IX1---¦SINGLE_PB FWD_MOTOR¦---%QX1 %IX2---¦DOUBLE_PB REV_MOTOR¦---%QX2 %IX3---¦LEFT_LS ¦ %IX4---¦RIGHT_LS ¦ +--------------------+ |  |

Figure F.11 - A graphical configuration of program AGV

+----------------------------+ ¦ ¦ ¦ +===+===+ (* Main sequence *) ¦ ¦¦START¦¦ ¦ +===+===+ ¦ ¦ ¦ +--------------------*----------+ ¦ ¦ ¦ ¦ + READY.X & SINGLE_PB + READY.X & DOUBLE_PB ¦ ¦ ¦ ¦ +--+---+ +-+-----+ +---+----+ +-+-----+ ¦ ¦SINGLE+-¦N¦CYCLE¦ ¦DOUBLE_1+--¦N¦CYCLE¦ ¦ +--+---+ +-+-----+ +---+----+ +-+-----+ ¦ ¦ ¦ ¦ *---------+ + DONE.X ¦ ¦ + DONE.X & DOUBLE_PB ¦ ¦ ¦ +---------------------+ ¦ ¦ ¦ ¦ ¦ +-----+-----+ ¦ + DONE.X & NOT DOUBLE_PB ¦DOUBLE_WAIT¦ ¦ ¦ +-----+-----+ ¦ ¦ ¦ ¦ ¦ + READY.X ¦ ¦ ¦ ¦ ¦ +---+----+ +-+-----+ ¦ ¦ ¦DOUBLE_2+--¦N¦CYCLE¦ ¦ ¦ +---+----+ +-+-----+ ¦ ¦ ¦ ¦ ¦ + DONE.X ¦ ¦ ¦ ¦ +---------------------+---------+ ¦ ¦ ¦ +----+-----+ ¦ ¦NON_REPEAT¦ ¦ +----+-----+ ¦ ¦ ¦ +NOT(SINGLE_PB OR DOUBLE_PB) ¦ ¦ +-----------------------------+

Figure F.12 - Body of program AGV
(continued on following page)

+-------+ ¦ ¦ ¦ +===+===+ (* Perform a single cycle *) ¦ ¦¦READY¦¦ ¦ +===+===+ ¦ ¦ ¦ + CYCLE ¦ ¦ ¦ +---+---+ +-+---------+ ¦ ¦FORWARD+-¦N¦FWD_MOTOR¦ ¦ +---+---+ +-+---------+ ¦ ¦ ¦ + RIGHT_LS ¦ ¦ ¦ +---+---+ +-+---------+ ¦ ¦REVERSE+-¦N¦REV_MOTOR¦ ¦ +---+---+ +-+---------+ ¦ ¦ ¦ + LEFT_LS ¦ ¦ ¦ +--+-+ ¦ ¦DONE¦ ¦ +--+-+ ¦ ¦ ¦ + NOT CYCLE ¦ ¦ +-------+

Figure F.12 - Body of program AGV (continued)

## F.9  Use of enumerated data types

The following example illustrates the use of enumerated data types in ST CASE statements and in Instruction List.  Suppose an enumerated data type has been defined by the following declaration:

TYPE SPEED: (SLOW, MEDIUM, FAST, VERY_FAST); END_TYPE

In addition, suppose an input and output of a function block type is declared by:

VAR_INPUT MOTOR_SPEED: SPEED; END_VAR
VAR_OUTPUT SPEED_OUT: SPEED; END_VAR

Then if the body of the function block type is defined in the ST language, a CASE statement such as the following could be used:

CASE MOTOR_SPEED OF
  SLOW:   (* speed it up *);
  MEDIUM: (* hold the current speed *);
  FAST:   (* slow it down *);
ELSE      (* take special care *);
END_CASE;

If the body of the function block type is defined in the IL language, the following instructions could be used:

LD SPEED#SLOW   (* enumerated value qualified by data type *)
ST SPEED_OUT

## F.10  Function block RTC (Real Time Clock)

The RTC function block shown below sets the output CDT to the input value PDT at the next evaluation of the function block following a transition from 0 to 1 of the IN input.  The CDT output of the RTC function block is undefined when the value of IN is 0.

Function block RTC (Real Time Clock)

| PDT = Preset date and time, loaded on rising edge of IN CDT = Current date and time, valid when IN=1 Q = copy of EN | +-------+ ¦ RTC ¦ BOOL---¦IN Q¦---BOOL DT-----¦PDT CDT¦-----DT +-------+ |
| --- | --- |

## F.11  Function block ALRM_INT

This function block type provides simple high and low level alarming for an input of type INT and illustrates the use of the VAR_OUTPUT declaration with functions. The function output is TRUE if a high or low threshold is exceeded, and separate outputs are provided for the high- or low-level alarm conditions

| +----------+ ¦ ALRM_INT ¦ IN----¦INT ¦---BOOL THI---¦INT HI¦---BOOL THL---¦INT LO¦---BOOL +----------+ +---+ IN---¦ > ¦---+-----------------HI THI--¦ ¦ ¦ +----+ +---+ +--¦ OR ¦---ALRM_INT +---¦ ¦ +---+ ¦ +----+ IN---¦ < ¦--+------------------LO THL--¦ ¦ +---+ | FUNCTION ALRM_INT : BOOL VAR_INPUT IN : INT ; THI : INT ; (* High threshold *) TLO : INT ; (* Low threshold *) END_VAR VAR_OUTPUT HI: BOOL; (* High level alarm *) LO: BOOL; (* Low level alarm *) END_VAR HI := IN > THI ; LO := IN < THL ; ALRM_INT := THI OR THL ; END_FUNCTION |
| --- | --- |

# ANNEX G - Index (informative)

Primary references for delimiters and keywords are given in annex C.

absolute time, 29

access path, 118, 120

communication, 16

keyword, 42

loading/deletion, 16

programming, 18

action, 90, 96, 134

control, 101

qualifiers, 100

action block, 96, 99, 102

active association, 102

active association, 102

activity flow, 147

aggregate, 10

argument, 72, 141, 145

array

declaration, 34, 45

initialization, 35, 45

location assignment, 44

usage, 40, 146

assignment, 93

FOR loop variable values, 146

operator, 35, 93

statement, 144

assignment, 50, 53, 55, 74

based number, 10, 26

bistable function block, 82

bit string

comparison, 64

data types, 31

functions, 64

initial value assignment, 45

variable declaration, 43

body

function, 55, 144

function, 53, 74

function block, 72

program organization unit, 149

Boolean

AND, in ladder diagrams, 153

data type, 31

default initial value, 36

edge detection, 75, 83

expression, 93, 141, 145, 146

functions, 64

functions, 102

input, action control, 102

input, RETURN, 150

literals, 26

negation, 50

operators, 141

OR, LD vs. FBD, 156

output, 150

signal, 150

values, power flow, 152

variable, 91, 92, 96, 97, 101, 124, 145, 150

variable, in ladder diagrams, 153

byte (data element size), 40

BYTE (data type), 31, 36

case (of characters), 24, 25, 30

CASE statement, 145

character code, 25, 67

character set, 24, 91, 147, 154

character string

character positions in, 67

comparison, 67

data type, 31

functions, 67

initialization, 45

literals, 27

variable declaration, 43

cold restart, 41, 44, 45, 46, 47

comment, 25

comparison

bit strings, 64

character strings, 67

functions, 64, 71

compilation, 22

compliance, 21

action declarations, 96

EXIT statement, 144

programs, 23

sequential function chart (SFC), 118

step/action association, 98

syntax, 22

system, 21

concatenation

action blocks, 99, 100

hierarchical addresses, 38

time data, 69

conditional

jump, 150

return, 150

configuration

communication, 16

elements, 118

initialization, 15

programming, 18

starting and stopping, 15

connection, 88

connector, 93, 95, 147, 148

contact, 153

counter, 84

data type

compliance, 22

declaration, 34

elementary, 31

generic, 33, 57

initialization, 35

of an expression, 141

of functions, 54

of internal variables, 54

programming, 18

usage, 37

date and time, 87

data types, 31

default initial values, 36

functions, 69

literals, 30

decimal number (decimal literal), 26, 147

declaration, 18

access paths, 120

actions, 96, 101

configurations, 118

data types, 34

function blocks, 72, 75

functions, 54, 144

programs, 89

resources, 120

tasks, 124

variables, 41

default value, 56

FOR increment, 146

of data types, 35

of variables, 41, 45

task interval, 124

delimiter

comments, 25

LD network, 152

network label, 147

time literals, 29

direct representation, 38, 89, 122

in programs, 90

initial value assignment, 45

variable declaration, 43

double word, 31

size prefix, 40

duration

data type (TIME), 31

literals, 30

of action qualifiers, 100

of step activity, 107

edge detection, 75

function blocks, 83

EN/ENO (enable) variables, 53, 54, 74

errors, 26, 34, 39, 43, 48, 50, 53, 56, 60, 61, 64, 66, 68, 69, 76, 91, 92, 94, 102, 107, 124, 126, 135, 141, 144, 146, 149, 182

documentation, 22

handling, 22, 23

reporting, 22

errors, 92, 107, 147

evaluation

of assignment statements, 144

of expressions, 141

of function blocks, 125

of functions, 63, 141, 144

of language elements, 125

of network elements, 149

of networks, 72, 149, 154, 156

of programs, 125

of transitions, 107

execution

of actions, 90

of EXIT statements, 146

of function blocks, 72, 83, 125

of functions, 53

of iteration statements, 145

of loop elements, 149

of programs, 150

of selection statements, 145

execution control element, 90, 124, 149, 150, 154

extensions, 22, 39

documentation, 22

processing, 22

usage, 23

extensions, 25

falling edge, 75, 78, 83

feedback

path, 149, 150

variable, 149

FOR statement, 145

function, 48

compliance, 22

control statements, 144

extensible, 59

in LD language, 153

overloaded, 57, 58, 61, 64

programming, 18

return value, 144

signal flow, 148

typing, 57

function (procedure), 11

function block, 11, 14, 72

action control, 101

communication, 16, 88

compliance, 22

control statements, 144

in LD language, 153

instance, 124, 125

operation, 84, 87

programming, 18

retentive, 91

SFC structuring, 90

signal flow, 148

type, 72

function block diagram, 11

function block diagram (FBD), 14, 156

action blocks in, 100

execution control, 150

loops in, 149

signal flow in, 148

function block instance, 11

function block type, 11

generic data types, 33, 57

global variable, 118

communication, 16

declaration, 41, 90, 120

function block instance, 72

initial value assignment, 45

initialization, 15

loading/deletion, 16

programming, 18

hierarchical addressing, 39

identifier, 24, 72, 91, 93, 147

identifier, 54

implementation-dependent

feature, 22, 23, 41, 45, 146

side effects, 74

implementation-dependent parameters, 21, 25, 26, 34, 35, 39, 43, 48, 55, 59, 60, 61, 72, 86, 87, 90, 91, 94, 98, 107, 124, 141, 143, 145, 147, 180

initial

state, 91

step, 91, 106

initial value

assignment, 45

default, 35

feedback variable, 149

FOR loop variable, 146

initialization, 41

function blocks, 75

function blocks, 106

programs, 89

programs, 106

SFC networks, 106

steps, 91

initialization, 15

input

declaration, 41, 54, 75

dynamic, 75

extensible, 59

initialization, 41

instance name, 72

location prefix, 40

negated, 50

overloaded, 57

program, 122

string, 61

variable, 72, 153

variable, 89

input/output

variable, 72

instance

function block, 72, 75, 76, 77, 124, 125

name, 72, 76, 77

instantiation

action control, 102

function block, 89

program, 89

instruction, 93, 96

integer

data types, 31, 146

literal, 12, 26, 147

invocation

by tasks, 124

function block, 144

function block, 72, 73, 75

of actions, 90

of functions, 141

of non-PC language elements, 19

recursive, 48

return from, 150

iteration, 144, 145

keyword, 25

Boolean literals, 27, 93

data types, 31

ELSE statement, 145

FOR statement, 146

function block declaration, 75

function declaration, 54

IF statement, 145

program declaration, 89

REPEAT statement, 146

time literals, 29

transition, 93

variable declaration, 41

WHILE statement, 146

label

connector, 147

network, 147, 149, 150

ladder diagram, 152

evaluation, 149

execution control, 150

network, 93

language element, 14

compliance, 21

programming, 18, 89

library, 18, 120

literal, 26, 141, 152

logical location, 38, 39, 41, 43

long real, 31

long word, 31

memory, 153

memory (user data storage)

allocation, 41

direct representation, 38

initial value assignment, 45

initialization, 41

named element, 40, 147

network, 12, 101

direction of flow, 147

evaluation, 72, 149, 154, 156

function block diagram (FBD), 93, 96

label, 150

ladder diagram (LD), 93, 152

sequential function chart (SFC), 91, 105, 106

network, 147

numeric literals, 26

off-delay, 12, 87

on-delay, 12, 87

operand

of an expression, 141

operator

assignment, 35, 93, 144

overloaded, 57

precedence, 141

Structured Text (ST), 141

symbols, 63, 65, 67, 71

output

action control, 101

declaration, 75

function block, 143, 156

location prefix, 40

negated, 50

program, 122

string, 61

typed, 57

values, 72

variable declaration, 41

variables, 72, 89, 153

variables, 72

overloading, 57

of operators, 141

parentheses, 25, 35, 40, 45, 93, 141

power flow, 100, 147, 150, 152, 153

power rails, 152

power rails, 147, 150, 152

pre-emptive scheduling, 124

priority

of tasks, 124

of transitions, 107

program, 14, 18, 89

communication, 16

compliance, 23

declaration, 41, 89, 122

declaration, 72, 81

retentive, 91

scheduling, 124

SFC structuring, 90

program organization unit, 47, 72

compliance, 22

declaration, 41, 49

initial state, 91

jumps in, 150

networks in, 147, 149, 154

scheduling, 124

SFC partitioning of, 90

state, 90

programming, 18, 152

programming, 156

real literal, 26

resource, 14, 89

communication, 18

declaration, 120

global variables in, 121

initialization, 15, 41

programming, 18

starting and stopping, 15, 125

retentive data

declaration, 41, 90

in function blocks, 75

in steps, 91

initial value assignment, 45

initialization, 41

type assignment, 43

return, 143, 144, 145, 150

rising edge, 75, 78, 83, 124, 227

rung, 152

rung, 96

scope

global, 121

of actions, 96

of declarations, 43

of function block instances, 72

of networks, 147

of steps, 91

of transitions, 94

selection

functions, 64, 71

statements, 145

semantics

Structured Text (ST), 141

semigraphic representation, 75, 124, 147, 148

sequential function chart (SFC)

activity flow, 148

compliance, 118

convergence, simultaneous, 107

divergence, selection, 107

divergence, simultaneous, 107

elements, 14, 90

elements, 134, 146, 147

elements, compatibility of, 117

errors, 107

evolution, 106

programming, 18

signal flow, 147, 156

single data element, 37, 38

step, 90

action association, 98

activation, 107

activation, 106

active, 90, 91, 92, 107

active, 102

deactivation, 91, 98, 106

duration, 107

elapsed time, 91

flag, 91

inactive, 90, 91

initial, 91, 106

initialization, 91

retentive, 91

state, 91, 106, 107, 115

structured data type, 72

declaration, 34

initialization, 35

usage, 37

structured variable, 40

assignment, 144

declaration, 45

initialization, 45

step elements, 91

subscripting, 40

array initialization, 45

symbolic representation, 38, 43

synchronization

interprocess, 146

of function blocks, 124

syntax, 14

documentation, 22

step/transition, 107

task, 14, 124

declaration, 120

programming, 18

TIME data type, 31, 91, 92, 100

default initial value, 36

function blocks, 87

functions, 69

time literal, 29

time of day

data types, 31

default initial value, 36

functions, 69

keywords, 30

literals, 29

timer, 87

transition, 90, 91, 93

clearing, 106, 107

clearing time, 107

condition, 90, 93, 96, 106

enabled, 106

evaluation, 107

priority, 107

symbol, 106

type conversion

functions, 58, 60

typed literals, 27

underline character, 24, 26, 57

unsigned integer, 147

data types, 31

variable, 38

declaration, 54, 75

usage, 37

WAIT function, 96, 146

warm restart, 41

white space, 25

wired OR, 156

# ANNEX H - Reference character set (informative)

NOTE 1	The contents of the most recent edition of "Table 1	Row 00: ISO-646 IRV" of ISO/IEC 10646-1 are normative for the purposes of this standard.  The reference character set is reproduced here for information only.

NOTE 2	In variables of type STRING, the individual byte encodings of the characters in this reference character set are as given in Table H.2.  In variables of type WSTRING, the numerical equivalent of individual 16-bit word encodings are  also as given in Table H.2.

Table H.1 - Character representations

|  |  | First hexadecimal digit |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Second hexadecimal digit |  | 2 | 3 | 4 | 5 | 6 | 7 |  |
| 0 |  |  | 0 | @ | P | ` | p |  |
| 1 |  | ! | 1 | A | Q | a | q |  |
| 2 |  | " | 2 | B | R | b | r |  |
| 3 |  | # | 3 | C | S | c | s |  |
| 4 |  | $ | 4 | D | T | d | t |  |
| 5 |  | % | 5 | E | U | e | u |  |
| 6 |  | & | 6 | F | V | f | v |  |
| 7 |  | ' | 7 | G | W | g | w |  |
| 8 |  | ( | 8 | H | X | h | x |  |
| 9 |  | ) | 9 | I | Y | i | y |  |
| A |  | * | : | J | Z | j | z |  |
| B |  | + | ; | K | [ | k | { |  |
| C |  | , | < | L | \ | l | ¦ |  |
| D |  | - | = | M | ] | m | } |  |
| E |  | . | > | N | ^ | n | ~ |  |
| F |  | / | ? | O | _ | o |  |  |
| Table H.2 - Character encodings |  |  |  |  |  |  |  |  |
| dec | hex | Name |  |  |  | dec | hex | Name |
| 032 | 20 | SPACE |  |  |  | 080 | 50 | LATIN CAPITAL LETTER P |
| 033 | 21 | EXCLAMATION MARK |  |  |  | 081 | 51 | LATIN CAPITAL LETTER Q |
| 034 | 22 | QUOTATION MARK |  |  |  | 082 | 52 | LATIN CAPITAL LETTER R |
| 035 | 23 | NUMBER SIGN |  |  |  | 083 | 53 | LATIN CAPITAL LETTER S |
| 036 | 24 | DOLLAR SIGN |  |  |  | 084 | 54 | LATIN CAPITAL LETTER T |
| 037 | 25 | PERCENT SIGN |  |  |  | 085 | 55 | LATIN CAPITAL LETTER U |
| 038 | 26 | AMPERSAND |  |  |  | 086 | 56 | LATIN CAPITAL LETTER V |
| 039 | 27 | APOSTROPHE |  |  |  | 087 | 57 | LATIN CAPITAL LETTER W |
| 040 | 28 | LEFT PARENTHESIS |  |  |  | 088 | 58 | LATIN CAPITAL LETTER X |
| 041 | 29 | RIGHT PARENTHESIS |  |  |  | 089 | 59 | LATIN CAPITAL LETTER Y |
| 042 | 2A | ASTERISK |  |  |  | 090 | 5A | LATIN CAPITAL LETTER Z |
| 043 | 2B | PLUS SIGN |  |  |  | 091 | 5B | LEFT SQUARE BRACKET |
| 044 | 2C | COMMA |  |  |  | 092 | 5C | REVERSE SOLIDUS |
| 045 | 2D | HYPHEN-MINUS |  |  |  | 093 | 5D | RIGHT SQUARE BRACKET |
| 046 | 2E | FULL STOP |  |  |  | 094 | 5E | CIRCUMFLEX ACCENT |
| 047 | 2F | SOLIDUS |  |  |  | 095 | 5F | LOW LINE |
| 048 | 30 | DIGIT ZERO |  |  |  | 096 | 60 | GRAVE ACCENT |
| 049 | 31 | DIGIT ONE |  |  |  | 097 | 61 | LATIN SMALL LETTER A |
| 050 | 32 | DIGIT TWO |  |  |  | 098 | 62 | LATIN SMALL LETTER B |
| 051 | 33 | DIGIT THREE |  |  |  | 099 | 63 | LATIN SMALL LETTER C |
| 052 | 34 | DIGIT FOUR |  |  |  | 100 | 64 | LATIN SMALL LETTER D |
| 053 | 35 | DIGIT FIVE |  |  |  | 101 | 65 | LATIN SMALL LETTER E |
| 054 | 36 | DIGIT SIX |  |  |  | 102 | 66 | LATIN SMALL LETTER F |
| 055 | 37 | DIGIT SEVEN |  |  |  | 103 | 67 | LATIN SMALL LETTER G |
| 056 | 38 | DIGIT EIGHT |  |  |  | 104 | 68 | LATIN SMALL LETTER H |
| 057 | 39 | DIGIT NINE |  |  |  | 105 | 69 | LATIN SMALL LETTER I |
| 058 | 3A | COLON |  |  |  | 106 | 6A | LATIN SMALL LETTER J |
| 059 | 3B | SEMICOLON |  |  |  | 107 | 6B | LATIN SMALL LETTER K |
| 060 | 3C | LESS-THAN SIGN |  |  |  | 108 | 6C | LATIN SMALL LETTER L |
| 061 | 3D | EQUALS SIGN |  |  |  | 109 | 6D | LATIN SMALL LETTER M |
| 062 | 3E | GREATER-THAN SIGN |  |  |  | 110 | 6E | LATIN SMALL LETTER N |
| 063 | 3F | QUESTION MARK |  |  |  | 111 | 6F | LATIN SMALL LETTER O |
| 064 | 40 | COMMERCIAL AT |  |  |  | 112 | 70 | LATIN SMALL LETTER P |
| 065 | 41 | LATIN CAPITAL LETTER A |  |  |  | 113 | 71 | LATIN SMALL LETTER Q |
| 066 | 42 | LATIN CAPITAL LETTER B |  |  |  | 114 | 72 | LATIN SMALL LETTER R |
| 067 | 43 | LATIN CAPITAL LETTER C |  |  |  | 115 | 73 | LATIN SMALL LETTER S |
| 068 | 44 | LATIN CAPITAL LETTER D |  |  |  | 116 | 74 | LATIN SMALL LETTER T |
| 069 | 45 | LATIN CAPITAL LETTER E |  |  |  | 117 | 75 | LATIN SMALL LETTER U |
| 070 | 46 | LATIN CAPITAL LETTER F |  |  |  | 118 | 76 | LATIN SMALL LETTER V |
| 071 | 47 | LATIN CAPITAL LETTER G |  |  |  | 119 | 77 | LATIN SMALL LETTER W |
| 072 | 48 | LATIN CAPITAL LETTER H |  |  |  | 120 | 78 | LATIN SMALL LETTER X |
| 073 | 49 | LATIN CAPITAL LETTER I |  |  |  | 121 | 79 | LATIN SMALL LETTER Y |
| 074 | 4A | LATIN CAPITAL LETTER J |  |  |  | 122 | 7A | LATIN SMALL LETTER Z |
| 075 | 4B | LATIN CAPITAL LETTER K |  |  |  | 123 | 7B | LEFT CURLY BRACKET |
| 076 | 4C | LATIN CAPITAL LETTER L |  |  |  | 124 | 7C | VERTICAL LINE |
| 077 | 4D | LATIN CAPITAL LETTER M |  |  |  | 125 | 7D | RIGHT CURLY BRACKET |
| 078 | 4E | LATIN CAPITAL LETTER N |  |  |  | 126 | 7E | TILDE |
| 079 | 4F | LATIN CAPITAL LETTER O |  |  |  |  |  |  |

-- END OF PART 3 --